3GPP TSG-RAN WG3 Meeting #125	R3-244449

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia, Telecom Italia, Qualcomm

Title:	Continuous Management-based MDT

Document for:	Discussion

# 1	Introduction

In RAN3 #124 we captured the use case description for the continuous MDT data collection as follows (TR 38.743 v1.0.0): a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

We also identified two tasks that need to be solved in order to achieve Data Collection continuity in this scenario (TR 38.743 v1.0.0):

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In this paper, we provide our views on the possible solutions to address continuity in the MDT data collection. We also provide a TP to TR 38.743, with possible standards impacts of continuous Management-based MDT, in [1].

# 2	Discussion

In management-based MDT, MDT continuity is not maintained when a UE switches between RRC_Connected and RRC_Idle states, or when a UE is handed over to another gNB. This is due to the following reasons:

- Immediate MDT configuration is erased by the UE every time it transits to RRC_Idle state.

- When a UE transits from RRC_Idle to RRC_Connected state the network does not have standardized means to select a “specific” UE for continuous MDT. This is because the MDT Configuration does not uniquely characterize a UE.

- In legacy Management-based MDT, the management-based MDT configuration does not propagate during a handover. Therefore, the target node cannot know the configuration that a UE had in the source before the handover.

In order to address the above issues, corresponding to problem A (measurement continuity), the following categories of solutions can be considered:

- Solution A1: Upon transition from RRC_Idle or RRC_Inactive to RRC_Connected, the UE indicates to the gNB that it should be selected for continuous management-based immediate MDT. In case of handover, the source gNB sends this information to the target gNB during handover preparation.

- Solution A2: When UE transits to RRC_Idle, a context related to the MDT configuration is stored by the gNB serving the UE to the Core Network. Upon transition from RRC_Idle to RRC_Connected or after successful handover, the AMF indicates to the second gNB serving the UE that the UE should be selected for management-based immediate MDT. In case of handover, the source gNB sends the context related to the data collection to the target gNB during handover preparation.

- Solution A3: UE logs measurements across RRC states (RRC_Idle, RRC_Inactive, RRC_Connected), and hence also independently of any handover in RRC_connected, by enhancing management-based Logged MDT.

The proposed solutions have impacts to different entities. Solutions A1 and A3 have impacts to the UE. This means that if we adopt those solutions the number of UEs that support the continuous MDT feature will be limited to Rel-19 UEs implementing the feature.

Proposal 1: We propose to consider continuous MDT solutions that do not impact the UE.

Solution A2 is a network-based solution with minor impacts to the Core Network (AMF). This solution may work with simple modifications to the legacy management-based MDT procedures.

In Figure 1, we illustrate an example of maintaining Management-based MDT Data Collection continuity when UE transits to RRC idle state.

In this example it is assumed that management system configures a gNB (gNB 1) with both management-based Logged and Immediate MDT configuration. The gNB 1 chooses the UEs to configure for continuous management-based activation. The gNB1 chooses a UE (UE1) and configures it with both Logged and Immediate MDT. The UE may report the collected measurements related to the Immediate MDT configuration to gNB 1 while in RRC_Connected state. When the UE transits to RRC_Idle the gNB where the UE was connected can store MDT session information about this UE to the Core Network. This also assumes that this MDT session information is forwarded to the target gNB during handover preparation if the target gNB serves cells within the MDT area scope. When the UE later transits to RRC_Connected state in the same or in another gNB, upon connection establishment, the Core Network can provide the stored MDT session information to the gNB to which the UE connects to. In the example in the figure, the UE connects to a different gNB 2. After receiving the information from the Core Network the gNB can configure continuously the UE with Immediate MDT.

Figure 1 An example of continuous management-based MDT based on solution A2 when UE transits to RRC_Idle state.

In Figure 2 we illustrate an example of maintaining management-based MDT Data Collection continuity during a UE handover. Even though propagating the Immediate MDT configuration in handover preparation signalling would be our preference, this scenario would also be an alternative..

In this example it is assumed that both gNBs (gNB1 and gNB2) belong to the same area scope of the MDT session. The management system configures both gNBs (gNB 1 and gNB2) with management-based Immediate MDT configuration. The gNB 1 chooses the UEs to configure for continuous management-based activation. The gNB1 chooses a UE (UE1) and configures it with Immediate MDT. The UE may report the collected measurements related to the Immediate MDT Configuration to gNB 1 while in RRC_Connected state. When gNB1 decides to handover a UE it stores the MDT session information about this UE to the Core Network. After successful Handover, the target gNB (gNB2) receives from the Core Network the stored MDT session information. After receiving the information from the Core Network the target gNB (gNB2) can configure continuously the UE with Immediate MDT.

Figure 2 An example of continuous management-based MDT based on solution A2 when there is a handover.

Proposal 2: Introduce the following solution for measurement continuity in TR 38.743:

- When the UE reconnects from RRC_Idle, the gNB ensures MDT continuity by initiating management-based Immediate MDT according to MDT session information stored in the core network and made available at the gNB receiving the UE.

- When the UE is handed over from a gNB to another, gNB continuity can be ensured either by a) propagating the MDT configuration in handover preparation signalling or b) the gNB, receiving the UE, retrieving the MDT configuration from the core network.

For problem B (trace correlation), several solutions have been discussed:

- Solution B1: Send TR and TRSR of the Logged MDT configuration to the TCE together with the Immediate MDT data records.

- Solution B2: Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the CN and maintained in the CN for the duration of the UE context. Alternatively, if confirmed as a valid option by SA5, use the S-TMSI for this purpose. Provide this identifier to the TCE together with TR/TRSR for the immediate MDT session and for the logged MDT session. This mechanism may rely on the legacy Cell Traffic Trace mechanism.

- Solution B3: Introduce a new Job Type for “Logged and Immediate MDT”

  - Solution B3a: The same TR is used for both Logged and Immediate MDT Configuration

  - Solution B3b: Different TRs may be used for Logged and Immediate MDT Configuration with Continuous Indication in the Logged MDT Configuration

Solution B1 comes with the advantage of absence of impact on standardized interfaces (except towards the TCE), but requires the gNB to retrieve TR/TRSR of the Logged MDT configuration from the UE in order to be able to write MDT data records for immediate MDT to the TCE. However a legacy UE will not be able to provide this information to the network unless it has already logged MDT measurements available, so this solution will in many scenarios not work.

Solution B3a creates a single Job Type for “Logged and Immediate” MDT. Introducing a new Job Type for “Logged and Immediate MDT” is limiting since it forces a gNB to map a single Immediate MDT configuration to a Logged MDT configuration across an area of gNBs. It also requires SA5 changes to introduce a new Job Type. But furthermore, according to TS 32.422 Job Type clause 5.9a, e.g., to support anonymization is described as follows:

***************************************************

## 5.9a	Job Type (M)

The Job Type is a mandatory parameter. The condition is either MDT or RLF or RCEF data collection functionality is supported. It defines if a single trace job, a combined MDT and trace job or RLF report collection job or RCEF report collection job is activated. This parameter also defines the MDT mode.

<<unmodified text skipped>>

***************************************************

According to the above observation creating a new Job Type combining Logged MDT and Immediate MDT goes against the current use of Job Type where it defines the MDT mode. So it is our understanding that Job Type can characterize a single MDT mode only.

Observation 1: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Solution B3b has UE impacts because it requires support of a ‘continuous indication’ in the logged MDT configuration. Both B3 solutions will in practice be restricted to the use of a single MDT configuration within a given area, which is not suitable in case different AI/ML models need to be trained using different MDT measurements and/or targeting different kind of UEs.

Solution B2 will have only limited impact in terms of MDT configuration and handling in the AMF and TCE, while avoiding UE impacts, and this solution seems to be the most suitable.

Proposal 3: Introduce the following solution for Trace Correlation in TR 38.743:

- MDT records are correlated in the TCE based on an identifier provided by the AMF.

Proposal 4: Agree the TP to TR 38.743 in [1].

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: We propose to consider continuous MDT solutions that do not impact the UE.

Proposal 2: Introduce the following solution for measurement continuity in TR 38.743:

- When the UE reconnects from RRC_Idle, the gNB ensures MDT continuity by initiating management-based Immediate MDT according to MDT session information stored in the core network and made available at the gNB receiving the UE.

- When the UE is handed over from a gNB to another, gNB continuity can be ensured either by a) propagating the MDT configuration in handover preparation signalling or b) the gNB, receiving the UE, retrieving the MDT configuration from the core network.

Observation 1: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Proposal 3: Introduce the following solution for Trace Correlation in TR 38.743:

- MDT records are correlated in the TCE based on an identifier provided by the AMF.

Proposal 4: Agree the TP to TR 38.743 in [1].

# References

[1]		R3-244450, (TP to TR 38.743) Towards a Continuous Management-based MDT solution, Nokia, Telecom Italia, Qualcomm