3GPP TSG-RAN WG3 Meeting #125	R3-244450

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia, Telecom Italia, Qualcomm

Title:	(TP to TR 38.743) Towards a Continuous Management-based MDT solution

Document for:	Text Proposal

# 1	Introduction

This paper provides a TP to TR 38.743 based on the discussions in R3-244449.

# Annex TP for TR 38.743

The TP is based on TR 38.743 v1.0.0.

<<< start of changes >>>

## 5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

## 5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

### 5.4.2 Potential Standard Impacts

Standards impacts for problem A (measurement continuity):

NG: Transfer of a context related to the data collection at UE transition from RRC_Connected to RRC_Idle (from the NG-RAN node to the AMF), and at transition from RRC_Idle to RRC_Connected (from the AMF to the NG-RAN node). Transfer of a context related to the data collection during NG handover preparation (from source NG-RAN node to target NG-RAN node, via the AMF).

Core Network: Storage of a context related to the data collection in the AMF.

Xn: Transfer of a context related to the data collection during handover preparation.

Standards impacts for problem B (trace correlation):

Core Network: Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the CN and maintained in the CN for the duration of the UE context. Alternatively, if confirmed as a valid option by SA5, use the S-TMSI for this purpose. Provide this identifier to the TCE together with TR/TRSR for the immediate MDT session and for the logged MDT session. This mechanism may rely on the legacy Cell Traffic Trace mechanism.

NG: None, if the Cell Traffic Trace mechanism is used.

<<< end of changes >>>