3GPP TSG-RAN WG3 Meeting #125	R3-244451

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	TP on AIML support for CCO

Source: 	Ericsson, InterDigital

Agenda item:	11.3

Document Type:	Discussion and decision

# TP to TR 38.743

This TP mirrors the discussions and proposals in R3-244452.

------------------------------------------- Start of changes -------------------------------------------

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2	AI/ML Model Training in the OAM and AI/ML Model Inference in the gNB

In this solution, a gNB takes CCO decisions using AI/ML model trained from OAM.

Figure 4.2.2.2-1. Model Training at OAM, Model Inference at gNB

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to the UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4a: The UE measurements are sent to the OAM as input data for AI/ML model training.

Step 4b-4c: The gNB1 and gNB2 provide input data for training the AI/ML model. For example, measured/predicted radio resource status, measurements collected for UEs served by the gNB1 and reporting coverage of cells served by the gNB2, and measurements collected for UEs served by the gNB2 and reporting coverage of cells served by the gNB1.

Step 5: Model Training at the OAM.

Step 6: The OAM deploys/updates the AI/ML Model to gNB1.

Note: This step is out of RAN3 scope.

Step 7a: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7b: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts a CCO issue, the predicted affected cells and SSB beams served by the gNB1, and a reference prediction time for the predicted CCO issue.

Step 9: The gNB1 generates the future coverage state(s) corresponding to the predicted CCO issue and the predicted affected cells and SSB beams, as well as the time when this will be applied.

Step 10: The gNB1 sends to the gNB2 the new coverage state(s) and the time at which it will be applied to the cells and SSB beams of gNB1.

Step 11: The gNB2 provides feedback to the gNB1.

#### 4.2.2.3	AI/ML Model Training and AI/ML Model Inference in the gNB

In this solution, a gNB takes CCO decisions using AI/ML model trained in the gNB.

Figure 4.2.2.3-1. Model Training and Model Inference at gNB

Step 0: gNB2 is assumed to have an AI/ML model optionally, which can provide gNB1 with input information.

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4: The gNB2 provides input data for training the AI/ML model. For example, measured/predicted radio resource status and measurements collected for UEs served by the gNB2 and reporting coverage of cells served by the gNB1.

Step 5: Model Training at the gNB1 trains AI/ML model for AI/ML based CCO.

Step 6: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts a CCO issue, the predicted affected cells and SSB beams served by the gNB1, and a reference prediction time for the predicted CCO issue.

Step 9: The gNB1 generates the future coverage state(s) corresponding to the predicted CCO issue and the predicted affected cells and SSB beams, as well as the time when this will be applied.

Step 10: The gNB1 sends to the gNB2 the new coverage state(s) and the time at which it will be applied to the cells and SSB beams of gNB1.

Step 11: The gNB2 provides feedback to the gNB1.

#### 4.2.2.4	AI/ML Model Training in the OAM and AI/ML Model Inference in the gNB-CU

In this solution, a gNB-CU takes CCO decisions using AI/ML model trained in the OAM.

Figure 4.2.2.4-1. Model Training at OAM and Model Inference at gNB-CU

Step 1: The gNB-CU of gNB1 configures the measurement information on the UE side and sends configuration message to the UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB-CU of gNB1.

Step 4a: The gNB1 further sends the UE measurements to the OAM as input data for AI/ML model training.

Step 4b-4c: The gNB1 and gNB2 provide input data for training the AI/ML model. For example, measured/predicted radio resource status, measurements collected for UEs served by the gNB1 and reporting coverage of cells served by the gNB2, and measurements collected for UEs served by the gNB2 and reporting coverage of cells served by the gNB1.

Step 5: Model Training at the OAM.

Step 6: The OAM deploys/updates the AI/ML Model to the gNB-CU of gNB1.

Note: This step is out of RAN3 scope.

Step 7a: The UE sends the UE measurement report(s) to the gNB-CU of gNB1 as input for AI/ML model inference.

Step 7b: The gNB-DU of gNB1 sends to the gNB-CU input data for AI/ML model inference.

Step 7c: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB-CU of gNB1 predicts a CCO issue, the predicted affected cells and SSB beams served by the gNB1, and a reference prediction time for the predicted CCO issue.

Step 9: The gNB-CU of gNB1 sends to the gNB-DU the predicted CCO issue, the predicted affected cells and SSB beams, and the reference prediction time.

Step 10: The gNB-DU of gNB1 generates the future coverage state(s) corresponding to the predicted CCO issue for the predicted affected cells and SSB beams.

Step 11: The gNB-DU sends to the gNB-CU of gNB1 the new coverage state(s) applied to the cells and SSB beams of gNB1, as well as the time when this will be applied.

Step 12: The gNB1 sends to the gNB2 the new coverage state(s) applied to the cells and SSB beams of gNB1, as well as the time when this will be applied.

Step 13: The gNB2 provides feedback to the gNB1.

#### 4.2.2.5	AI/ML Model Training and AI/ML Model Inference in the gNB-CU

In this solution, a gNB-CU takes CCO decisions using AI/ML model trained in the gNB-CU.

Figure 4.2.2.5-1. Model Training and Model Inference at gNB-CU

Step 0: gNB2 is assumed to have an AI/ML model optionally, which can provide gNB1 with input information.

Step 1: The gNB-CU of gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB-CU of gNB1 for AI/ML model training.

Step 4a: The gNB-DU of gNB1 provides input data to the gNB-CU of gNB1 for training the AI/ML model. For example, measured radio resource status.

Step 4b: The gNB2 provides input data for training the AI/ML model. For example, measured/predicted radio resource status and measurements collected for UEs served by the gNB2 and reporting coverage of cells served by the gNB1.

Step 5: Model Training at the gNB-CU of gNB1.

Step 6a: The UE sends the UE measurement report(s) to the gNB-CU of gNB1 as input for AI/ML model inference.

Step 6b: The gNB-DU of gNB1 sends to the gNB-CU input data for AI/ML model inference.

Step 6c: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 7: The gNB-CU of gNB1 predicts a CCO issue, the predicted affected cells and SSB beams served by the gNB1, and a reference prediction time for the predicted CCO issue.

Step 8: The gNB-CU of gNB1 sends to the gNB-DU the predicted CCO issue, the predicted affected cells and SSB beams, and the reference prediction time.

Step 9: The gNB-DU of gNB1 generates the future coverage state(s) corresponding to the predicted CCO issue for the predicted affected cells and SSB beams.

Step 10: The gNB-DU sends to the gNB-CU of gNB1 the new coverage state(s) applied to the cells and SSB beams of gNB1, as well as the time when this will be applied.

Step 11: The gNB1 sends to the gNB2 the new coverage state(s) applied to the cells and SSB beams of gNB1, as well as the time when this will be applied.

Step 12: The gNB2 provides feedback to the gNB1.

#### 4.2.2.6		Input of AI/ML based CCO

To support AI/ML based CCO in predicting a future CCO issue, a gNB1 may need the following information as input:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO State

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	SON Reports (e.g., RLF, CEF, RA)

From neighbouring gNBs:

-	Measured/Predicted radio resource status

-    UE measurements of UEs served by the neighbouring gNB and reporting coverage of cells of gNB1

#### 4.2.2.7  Output of AI/ML based CCO

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue

-    Prediction time for the predicted CCO issue

#### 4.2.2.8	Feedback of AI/ML based CCO:

To optimize the performance of AI/ML-based CCO, following feedback can be considered to be collected from gNBs:

- Measured radio resource status

- Legacy UE performance feedback for those UEs handed over from the source gNB

- SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.9	Standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Future coverage state(s) for cells and beams of the gNB affected by a predicted CCO issue

- Time in the future when future coverage state(s) will be applied

- UE measurements for UEs served by neighbouring gNBs and reporting coverage of cells of gNB1

- UE measurements for UEs served by gNB1 and reporting coverage of cells of neighbouring gNBs

F1 interface:

- Predicted CCO issue

- Predicted cells and beams affected by the predicted CCO issue

- Time in the future when the predicted CCO issue will occur

# 6	Conclusion

The AI/ML functionality and the following use cases are recommended to be specified in Rel-19 normative phase:

-	AI/ML-based Coverage and Capacity Optimization

Recommendations for each use case take the section of “Solutions and standard impacts” for each use case as basis.

------------------------------------------- End of changes -------------------------------------------