3GPP TSG-RAN WG3 Meeting #125	R3-244452

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	Further discussion on AIML support for CCO

Source: 	Ericsson, InterDigital

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

RAN3#125 is the last meeting for the SI on AI/ML in RAN in rel-19. We summarize below the agreements and open points for the AI/ML based CCO use case, as captured in the previous meetings.

At RAN3#123-bis the following agreements were reached:

For AI/ML based CCO,

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Solution for non-split architecture:

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

Step 2: gNB sends the future coverage status to neighbour gNBs.

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether 
    only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

At RAN3#124, the following agreements were captured:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

The following points have been captured as to be continued:

How to support the future CCO status and/or predicted CCO issue transmission over Xn?

- Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.

- Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time 
information to NG-RAN node 2.

Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status?

Time information to be further discussed in next meeting.

In this contribution, we provide further discussion related to AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## Future CCO state

Considering the agreements captured at RAN3#124:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

we note that there is a misalignment between the two which requires to be addressed.

The agreements above create a disparity for the functionalities supported by a gNB depending on whether a gNB relies on split or non-split architecture. It should be noted that this has never been done before, namely there is functional and feature parity for gNBs in split and non split architecture. Such disparity creates problems, which we explain below.

Observation 1: Allowing a non-split gNB to derive future CCO states via AI/ML while not allowing a split architecture gNB to derive future CCO states via AI/ML creates a feature disparity between split and non-split architecture gNB. This is incorrect because a gNB should always support the same features independently of wheather it is a split gNB or a non split gNB

As stated in the agreement for split architecture, “the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19”. This has some important implications which impact the first agreement. Even if an implementation would opt to derive a future CCO state as inferred by an AI/ML model at the gNB-DU, the standard will not support collection at the gNB-DU of data to be used as input or feedback to support such AI/ML model. Also, as we have studied and specified in rel-18 in relation to predictions, a prediction has an associated timing information (reference prediction time). However, according to the agreements taken for split architectures, RAN3 will not specify a prediction time for the future CCO state, as AI/ML based future CCO states are out of scope.

From the agreements taken, it is logical to derive that a split architecture gNB should not be required to support any functionality involving AI/ML based inference of the future CCO state.

However, when a future CCO state is signalled to a peer gNB , the receiver does not know if the sender is a split gNB or a non-split gNB. So, if different solutions are used at the sender to “produce” and to encode in XnAP IEs the future CCO state (in one case using AI/ML, and in another case using legacy means), what functionalities should the receiver support? We concluded above that a split gNB should not be required to support AI/ML based future CCO states, but what if a split gNB receives a future CCO state from a non-split gNB? In this case would the split gNB need to support AI/ML derived future CCO states, despite we agreed that it does not need to support this functionality?

Observation 2: Allowing a non-split gNB to derive future CCO states via AI/ML while not allowing a split gNB to derive future CCO states via AI/ML implies that even a split gNB needs to support AI/ML based future CCO state, while a split gNB should not be mandated to support this.

Therefore, a consistent solution between split and non-split architecture, which can be supported in the signalling to be specified in rel-19, is that, for non-split architecture, the future CCO state is derived via legacy mechanism. An implementation where – irrespective of split architecture or non-split architecture - AI/ML techniques would be used to derive such future CCO state will not be supported by the standard in rel-19, although it is not forbidden at implementation level.

Therefore, we propose to align the agreement related to the non-split architecture to make it consistent with the one for split architecture.

Proposal 1: Modify the agreement for non-split architecture as follows:

“For non-split architecture, the standardization of AI/ML based future CCO state inference in the gNB is not supported in R19, how to derive future CCO state is up to implementation in the gNB, e.g., legacy or AI/ML.”

At RAN3#124 the following was captured as to be continued:

How to support the future CCO status and/or predicted CCO issue transmission over Xn?

- Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.

- Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time 
information to NG-RAN node 2.

In relation to that, we observe that the only alternative consistent with legacy CCO is alternative 2. As specified in 38.300, in section 15.5.5.3:

An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations.

From the text above, it can be appreciated what is in scope of CCO, that is: based on a detected CCO issue, first an NG-RAN node adjusts its configuration and then it notifies to a neighbor NG-RAN node a list of cells (and optionally SSBs) with the respective coverage state indicators. As a consequence, the receiving NG-RAN node may adopt matching coverage configurations.

According to alternative 1: “NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.”. In this case, the way the CCO functionality works is changed, since the NG-RAN node 1, which has not adjusted yet its configuration, notifies the future CCO state to the NG-RAN node 2, but it may further change its configuration on the basis of the feedback received from NG-RAN node 2. In legacy CCO, the coverage modification performed at the node where the CCO issue is detected does not depend on the feedback of the neighbor node. And another major problem is that nothing seems to prevent endless iterations. According to this alternative, adjustments in coverage at NG-RAN node 2 leads to adjustments in coverage at NG-RAN node 1, which will cause the NG-RAN node 1 to send a new coverage state to NG-RAN node2, which will then try to further adjust its own coverage states, and it is not clear how this process can end.

Observation 3: Alternative 1 is not consistent with the legacy CCO and can result in an endless loop where one node changes the coverages of its cells and beams to adapt to the changes performed by the neighbour node.

In alternative 2, “”NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time information to NG-RAN node 2.” the CCO functionality way of working is respected. The related time information provided to NG-RAN node 2 will help NG-RAN node 2 understand when the coverage change will occur at NG-RAN node 1 and when it is the right time to “adopt coverage configurations matching with neighbouring cells coverage configurations”.

According to the discussion, we think alternative 2 is to be preferred, as captured in the following proposal.

Proposal 2: For AI/ML based CCO, agree to Alt2 as way forward, namely:

Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time 
information to NG-RAN node 2..

## Predicted CCO issues and affected cells and beams

For legacy CCO, a detected CCO issue is associated with a list of concerned cells and beams, which the gNB-CU communicates to the gNB-DU. It is natural to extend this to the AI/ML assisted CCO use case, i.e., the gNB-CU will send to the gNB-DU a predicted CCO issue and an associated list of cells and beams which are predicted to be affected. Accordingly, it’s good to clarify this in step 1 of the solution for split-architecture.

Proposal 3: The gNB-CU sends to the gNB-DU a predicted CCO issue and an associated list of predicted affected cells and beams.

## Signalling CCO issues and future coverage state

On whether to reuse or not existing signaling for CCO, the following was left for further discussion:

Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status?

Regarding the signalling of a CCO issue from gNB-CU to gNB-DU and between two gNBs, it should be clear in the signalling whether a CCO issue signalled by the gNB-CU is a detected CCO issue (now), or a predicted CCO issue (in the future). To achieve this in a simple and clear way, we think that the same F1AP and XnAP messages used for legacy CCO can be reused. To support AI/ML based CCO, we foresee that new IEs or extensions of existing IEs are needed to the current signalling mechanisms:

- In F1AP:

  - new IEs would be needed to indicate that the affected cells and beams are related to a prediction

  - a reference prediction time would be needed corresponding to a predicted CCO issue

- In XnAP:

  - New IEs would be needed to indicate to neighbouring gNB the predicted CCO issue and associated reference prediction time

At last meeting, it has been proposed to send predicted CCO related information using DATA COLLECTION REQUEST / RESPONSE / UPDATE XnAP messages. We think that use of these messages is not adequate to support this use case. From a functional point of view, the signalling design for legacy CCO to exchange detected issues and coverage state related information is already specified in NG-RAN Configuration Update in Xn and gNB-DU/gNB-CU Configuration Update in F1. Using a totally different procedure to support AI/ML based CCO will make the overall CCO functionality more complex (not only in standard, but also in implementation). For example, a gNB1 could receive a detected CCO issue from gNB2 in a Class 1 procedure, namely the NG-RAN Configuration Update and a predicted CCO issue from gNB3 in a Class 2 procedure, namely Data Collection Update, which brings to an unnecessarily more complex handling in gNB1 compared to receiving CCO related information (for detected and predicted CCO issues) in a common signalling procedure. Moreover, CCO functionalities imply a specific behaviour at the node receiving notification of a CCO coverage modification (which is why this information is signalled in a Class 1 procedure and acknowledged by the receiving node). It is inappropriate to signal a predicted CCO coverage modification via a Class 2 procedure, as the Data Collection Update procedure, because it would not even be known if the receiver received the information and if it could trigger the appropriate counter modifications.

Proposal 4: To support AI/ML based CCO, reuse the same F1AP and XnAP messages as in legacy CCO, and introduce new information elements to indicate a predicted CCO issue, the corresponding predicted affected cells and beams, and a reference prediction time of the predicted CCO issue.

Regarding the signalling of a future coverage state(s):

- for non-split architecture, it is important to signal when the coverage state will be used, i.e., the neighbour NG-RAN node needs to know when the future coverage state will be applied. In this case the Cell Deployment Status indicator could be reused, and the neighbour NG-RAN node needs to receive the information of when in the future the new configuration will be applied. This is already captured in Proposal 2.

- for split architecture, the legacy solution can be reused. As discussed in Proposal 3, the gNB-DU receives a “reference prediction time” from the gNB-CU for a given predicted CCO issue. Hence, the gNB-DU knows when the predicted CCO issue is expected. Then the gNB-DU can select a configuration, as in legacy CCO, within the alternative coverage configurations provided by OAM and it informs the gNB-CU of when the new configuration will be applied.

## Data collection for AI/ML based CCO

In this section we analyze further information that can be used to support AI/ML based CCO.  Some elements of the Rel-18 solution can be reused, and others may be added. The gNB-CU, as per current state of art, collects UE measurements of various kinds, that are used for CCO detection. To support predictions of CCO issues, based for instance on AI/ML inference, the same measurements can also be used. For example, subsequent measurements taken by UEs at cell edge can indicate a trend where the inference at the edge of such cells increases over time. This information can be used to infer that a certain CCO issue due to capacity is likely to occur for specific cells and beams. Predicting a certain CCO issue due to coverage can be tricky, and additional measurements may be needed. For example, some UEs are located at the border between a cell of gNB1 and a cell of gNB2, and if they are served by gNB2, they can send coverage measurements to gNB2 (the vice versa is also possible for UEs at the border between gNB1 and gNB2 but served by gNB1). As long as these UEs are connected to gNB2, the gNB1 is not aware of how these UEs perceive the coverage from gNB1 (and vice versa). If these UEs experience coverage problem, this might be avoidable by slightly adjusting the coverage of a cell or beam of gNB2 or a cell or beam of gNB1. To support this and to potentially avoid the UE entering a coverage hole, gNB2 could signal to gNB1 some of the measurements received from the UEs at its border with the cells of gNB1. This allows gNB1 to gain an understanding of coverage at cell edge, as seen from the UEs of gNB2. Such understanding may not be available and deductible by gNB1 because the UEs served by gNB1 may not have the same channel conditions towards gNB2 and vice versa (namely there is no reciprocity between channels of gNB1 UEs and gNB2 UEs). Knowledge at gNB1 of measurements from cell edge UEs served by gNB2 will help gNB1 to predict CCO issue due to coverage.

Figure 1: Example of coverage hole that can be resolved with cell shaping and CCO configuration changes

Figure 2: Example of how gNB1 may deduce from its UE measurements that a coverage hole is between gNB1 and gNB2, while gNB2 does not perceives such conditions

In the Figures above it is shown how, if a gNB relies only on measurements at cell edge provided by its own UEs, there might be misleading situations where coverage holes might be assumed and CCO measures taken, while there is no issue.

In Figure 1 gNB1 receives measurements form its UEs revealing a real coverage hole. By checking measurements from the UEs at cell edge of gNB2, gNB1 can confirm that there is a coverage hole between gNB1 and gNB2. Based on that, gNB1 can take a CCO action to correct the coverage issue.

In Figure 2, gNB1 checks measurements from its served UEs and detects a possible coverage hole between Cell A and Cell B (served by gNB2). However, when gNB1 checks measurements taken from UEs at the edge of Cell B and served by gNB2, gNB1 realizes that there is no coverage hole and a corrective CCO action is not needed.

The above proves that it is beneficial to receive cell edge measurements taken by UEs served by neighbour nodes.

Proposal 5: RAN3 to discuss additional measurements to be used for predicting CCO issue due to coverage, e.g. cell edge measurements taken by UEs served by neighbour nodes.

Finally, we would like RAN3 to discuss how to collect feedback after an action (e.g., a coverage modification) has been executed as a consequence of a predicted CCO issue. Since the issue did not materialize (at least not in full, due to the CCO action taken by the gNB-DU), how to understand whether an action taken was appropriate? A possibility can be to use a pragmatic approach, where some metrics and performance are compared before and after the action was taken. For example, if the predicted CCO issue was for capacity, due to an increasing number of demanding users at cell edge, we can assume that when the CCO issue was detected, the gNB-CU had already collected some measurements showing such trend and leading to that prediction (without AI/ML assistance, the CCO issue would have been detected later). The same type of measurements can be collected after the action, revealing if the CCO issue manifested or not. If the CCO issue did not manifest, then this implies that the potential CCO issue was successfully avoided.

Proposal 6: RAN3 to discuss how to collect feedback after an CCO related action is executed.

Based on the discussion provided, we propose to agree on the TP to TR 38.743 provided in R3-244451.

Proposal 7: Agree on the TP to TR 38.743 provided in R3-244451.

## Overall solution

In the flow charts below, we illustrate the overall solution for CCO for the non-split architecture and the split architecture, based on the proposal formulated above.

Figure 3 – AI/ML based CCO – Solution in non-split architecture

Figure 4 – AI/ML based CCO – Solution in split architecture

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Proposal 1: Modify the agreement for non-split architecture as follows:

“For non-split architecture, the standardization of AI/ML based future CCO state inference in the gNB is not supported in R19, how to derive future CCO state is up to implementation in the gNB, e.g., legacy or AI/ML..”

Observation 1: Alternative 1 is not consistent with the legacy CCO and can result in an endless loop where one node changes the coverages of its cells and beams to adapt to the changes performed by the neighbour node.

Proposal 2: For AI/ML based CCO, an NG-RAN node can send to neighbour NG-RAN nodes a future coverage state and indicate when the future coverage state will be applied”.

Proposal 3: The gNB-CU sends to the gNB-DU a predicted CCO issue and an associated list of predicted affected cells and beams.

Proposal 4: To support AI/ML based CCO, reuse the same F1AP and XnAP messages as in legacy CCO, and introduce new signalling elements to indicate a predicted CCO issue, the corresponding predicted affected cells and beams, and a reference prediction time of the predicted CCO issue.

Proposal 5: RAN3 to discuss additional measurements to be used for predicting CCO issue due to coverage, e.g. cell edge measurements taken by UEs served by neighbour nodes.

Proposal 6: RAN3 to discuss how to collect feedback after an CCO related action is executed.

Proposal 7: Agree on the TP to TR 38.743 provided in R3-244451.