3GPP TSG-RAN WG3 Meeting #125	R3-244454

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	TP on Mobility Optimization for NR-DC

Source: 	Ericsson, InterDigital, AT&T

Agenda item:	11.4

Document Type:	Discussion and decision

# TP to TR 38.743

This TP mirrors the discussions and proposals in R3-244453.

-------------------------------------------------------- Start of changes --------------------------------------------------------

# Rel-18 Leftovers and solutions

## 5.1	Mobility optimization for NR-DC

### 5.1.1 Use case description

When NR Dual Connectivity (NR-DC) is used, a UE is configured to utilise resources provided by two different nodes connected via non-ideal backhaul, both providing NR access. One node acts as the MN and the other as the SN. 
While, on one hand, use of NR-DC operation is meant to provide a boost in UE performance, it comes at the expense of quite complex operations, with many possible procedures (i.e. interactions) between the nodes involved, and various options to handle UE mobility (e.g., either the MN or the SN can initiate modification of the current setup).

Mobility optimization for NR-DC can take advantage of AI/ML support to optimal NR-DC actions and configuration.

In Rel-19, AI/ML-based Mobility optimization for NR-DC assumes inference only at the MN and Conditional Dual Connectivity procedures are out of scope.

### 5.1.2	Solutions and standard impacts

#### 5.1.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based Mobility optimization for NR-DC:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the MN.

- AI/ML Model Training and AI/ML Model Inference are both located in the MN.

#### 5.1.2.2	AI/ML Model Training in the OAM and AI/ML Model Inference in the MN

In this solution, the MN takes mobility optimization decisions for NR-DC using AI/ML model trained from OAM.

Figure 5.1.2.2-1. AI/ML Model Training in OAM and AI/ML Model Inference in the MN

In the flowchart, the MN is represented by gNB1, and the SN is represented by gNB2.

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4a: The UE measurements are sent to the OAM as input data for AI/ML model training.

Step 4b-4c: The gNB1 and gNB2 provide input data for training the AI/ML model. For example, measured/predicted radio resource status.

Step 5: Model Training at the OAM.

Step 6: The OAM deploys/updates the AI/ML Model to gNB1.

Note: This step is out of RAN3 scope.

Step 7a: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7b: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts an action for Mobility optimization for NR-DC.

Step 9: According to the prediction, recommended actions or configuration, the MN, the SN, and the UE perform the Mobility optimization for NR-DC (e.g., SN Addition).

Step 11: The gNB2 provides feedback to the gNB1.

#### 5.1.2.3	AI/ML Model Training and AI/ML Model Inference in the MN

In this solution, the MN takes mobility optimization decisions for NR-DC using AI/ML model trained in the MN.

Figure 5.1.2.3-1. AI/ML Model Training and AI/ML Model Inference in the MN

In the flowchart, the MN is represented by gNB1, and the SN is represented by gNB2.

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4: The gNB2 provides input data for training the AI/ML model. For example, measured/predicted radio resource status.

Step 5: Model Training at the gNB1 trains AI/ML model for AI/ML based Mobility Optimization for NR-DC.

Step 6: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts an action for Mobility Optimization for NR-DC.

Step 9: According to the prediction, recommended actions or configuration, the MN, the SN, and the UE perform the Mobility Optimization for NR-DC (e.g., SN Addition).

Step 10: The gNB2 provides feedback to the gNB1.

#### 5.1.2.4		Input of AI/ML-based Mobility Optimization for NR-DC

The following data is required as input data for mobility optimization for NR-DC.

From local node:

-	Measured/Predicted radio resource status

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	Mobility History Information.

From neighbouring gNBs:

-	Measured/Predicted radio resource status

- Measured/Predicted F1-U propagation delays

#### 5.1.2.5  Output of AI/ML-based Mobility Optimization for NR-DC

AI/ML-based mobility optimization for NR-DC can generate following output:

-	MN triggered dual connectivity procedures (e.g. SN Addition, MN-Initiated SN Change)

-	Predicted SN to add and assumptions for Dual Connectivity configuration (e.g., assumed PSCell, assumed QoS flows served via SN terminated SCG bearers)

#### 5.1.2.6	Feedback of AI/ML-based Mobility Optimization for NR-DC

The following data is required as feedback data for mobility optimization for NR-DC.

-	UE performance per PSCell while the UE is served at the SN, in terms of QoS parameters such as throughput, packet delay of the handed-over UE, etc.

-	Resource status information updates from SN.

#### 5.1.2.7	Standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work.

Xn interface:

- Enhancement for Data Collection Reporting Initiation and Data Collection Reporting procedures

- Enhancement in the reporting of UE associated measurements (e.g., UE performance per cell)

- Reporting of non-UE associated performance (e.g., measured F1 propagation delay)

- Enhancements of MN-initiated dual connectivity procedures (e.g., SN Addition, MN-Initiated SN Change)

-------------------------------------------------------- Next change --------------------------------------------------------

# 6	Conclusion

The AI/ML functionality and the following use cases are recommended to be specified in Rel-19 normative phase:

-	AI/ML-based Mobility Optimization for NR-DC

Recommendations for each use case take the section of “Solutions and standard impacts” for each use case as basis.

-------------------------------------------End of changes-------------------------------------------