3GPP TSG-RAN WG3 Meeting #125	R3-244460

Maastricht, NL, 19th - 23th August 2024

Agenda Item:	10.3.1

Source:	ZTE Corporation

Title:	(TP to BL CR for TS 38.300) Further discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1  Introduction

In last two meetings, some initial agreements on SON/MDT enhancements for NTN were reached as below.

Logged MDT enhancement based on scenarios to be worked first, work on immediate MDT if time allows. MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later. Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility. MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility. FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN. Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM. Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

In this contribution, we provide our further views on this topic, including the MRO for intra-NTN failure scenarios and the logged MDT enhancement based on scenarios.

2  Discussion

2.1  Involvement of IoT NTN

In the previous meetings, there was some discussion on whether to consider IoT NTN in the SON/MDT enhancements for intra-NTN mobility. While, there is no consensus on this issue. In previous releases, the agreed mobility features of NR NTN can be reused in IoT NTN, e.g., mobility restriction and conditional handover. In addition, the IoT NTN applies for both NB-IoT UEs and LTE-M UEs, for the LTE-M UEs, the mobility feature is also supported. In this case, the MRO enhancement for NR NTN can be also reused for IoT NTN. Furthermore, as given in the WID[1], the LTE specifications have already been captured, and the IoT NTN should not be excluded. Therefore, the IoT NTN should be also considered in the SON/MDT enhancements for intra-NTN mobility.

In this release, the SON/MDT enhancements for NR NTN could be discussed with high priority, and the corresponding progress of NR NTN can be reused in IoT NTN afterwards, just following the working manner of NR NTN and IoT NTN WIs in the previous releases.

Proposal 1: In Rel-19, IoT NTN should be also considered in the SON/MDT enhancements for intra-NTN mobility, and the progress of NR NTN can be reused in IoT NTN.

2.2  MRO for intra-NTN mobility failure

In last meeting, it has been agreed that both conditional and non-conditional intra-NTN handover should be considered in the MRO for intra-NTN mobility. Furthermore, there could be multiple trigger conditions for the conditional intra-NTN handover.

MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

2.2.1  MRO for conditional intra-NTN handover

Considering the conditional intra-NTN handover with single trigger condition, for the CHO with time-based trigger condition, when the RLF/HOF happens, since the time threshold and duration have been configured by the NW, it seems that there is no need for the UE to send the corresponding information to the NW. While, the time when the failure happens (e.g. the absolute time) could be useful for the NW to analyse the failure and adjust the configuration of time-based trigger condition for the UE.

Similarly, for the CHO with location-based trigger condition, the distance threshold and the reference location may be not needed, and the location where the RLF/HOF happens could also be useful for the NW to analyse the failure and adjust the configuration of location-based trigger condition for the UE. In addition, for the NTN, possibly, the UE can only provide its coarse UE location information to the NW.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

In addition, an indication showing the failure due to measurement-based CHO/time-based CHO/location-based CHO should be introduced to distinguish the three different types of CHO. With this indication, the NW could be aware of which type of failure has occurred.

Proposal 2: Regarding the MRO for the intra-NTN CHO with time-based trigger condition, the time information when the failure happens (e.g. absolute time) should be included in the RLF report.

Proposal 3: Regarding the MRO for intra-NTN CHO with location-based trigger condition, the location information where the failure happens should be included in the RLF report.

Proposal 4: An indication to distinguish the different types of CHO (measurement/time/location-based) should be included in the RLF report.

Considering the conditional intra-NTN handover with multiple trigger conditions, according to the TS 38.300, whether the UE is configured with single time-based/location trigger condition or multiple trigger conditions depends on whether the service discontinuity gap time length is assumed to be zero or negligible, which can corresponds to the Satellite switch with re-sync (PCI-unchanged use case) defined by RAN2.

------------------------------------------------------------TS 38.300----------------------------------------------------------------------

16.14.3.2.2	Conditional Handover

The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified.

NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	The RRM measurement-based event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

Time-based or location-based trigger conditions may be configured independently from the measurement condition for CHO in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible. However, if the service discontinuity gap time length is not zero or negligible, then a time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

------------------------------------------------------------TS 38.300----------------------------------------------------------------------

Therefore, comparing with the CHO with single trigger condition, there is no extra need to enhance the RLF report for the CHO with multiple trigger conditions.

Proposal 5: There is no extra need to enhance the RLF report for the CHO with multiple trigger conditions.

2.2.2  MRO for non-conditional intra-NTN handover

For non-conditional intra-NTN, since the NTN communication is tolerant of time delay, the timers related to the RLF report could be redefined or extended.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

Taking the T311 as an example, in current RRC specification, the maximum value of this timer is 30000ms, i.e., 30 seconds. For NTN scenario, the time delay may exceed 30 seconds, and this timer may be needed to extended to support the delay-tolerant services. In addition, the new cause value for the radio link failure could be introduced to match the NTN related scenarios.

Proposal 6: Regarding the MRO for non-conditional intra-NTN handover, the existing timer (e.g., T311) could be extended to support the delay-tolerant services.

In addition, considering the potential stage 3 impact, since the NG based handover between the NTN base stations should be applied in case of no Xn interface between the NTN base stations, some existing MRO mechanisms only supported over Xn should be also applied over NG, e.g., the missing part over NG, the transfer of assistance information for RRC Re-establishment reporting without RLF report. And the details could be discussed later.

Proposal 7: RAN3 should apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

2.2.3  MRO for RACH-less handover

For intra-NTN mobility, the RACH-less handover could be fallback to RACH based handover, while, after that, there still could be RLF/HOF after the fallback procedure. Therefore, to help the NW to analyse the cause of the failure and adjust the handover strategy, an indication on whether the UE has performed the RACH-less HO fallback could be introduced in the RLF report.

Proposal 8: Regarding the MRO for RACH-less handover, an indication on whether the UE has performed the RACH-less HO fallback before the intra-NTN mobility failure should be included in the RLF report.

2.2.4  Stage 2 update

In last meeting, it has been agreed that “Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.”. Therefore, the corresponding stage 2 description could be update to capture the connection failure scenarios for intra-NTN mobility.

Proposal 9: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

2.3  Logged MDT enhancement

The NTN cell provides much larger coverage area than the TN cell and it may cover the area of multiple countries. For Logged MDT, the Area Scope/Configuration should be enhanced. As given in TR 38.821, the cell diameter of a NTN cell varies from 50km to 1000km.

------------------------------------------------------------TR 38.821---------------------------------------------------------------------

Table 7.3.2.1.4-1: Time to HO for min/max cell diameter and varying UE speed

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56 (NOTE 1) | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

------------------------------------------------------------TR 38.821---------------------------------------------------------------------

Considering a NTN cell with 500km diameter which covers two different countries (e.g. country A and country B). The Operator in country A may only want to know the NTN specific coverage status in country A instead of the whole NTN cell. In addition, it could be not legal for the Operator in country A to obtain the NTN specific coverage status in country B. Therefore, the existing Area Scope over network interfaces or Area Configuration over Uu is not sufficient, and the new granularity of Area Scope or Area Configuration should be introduced for logged MDT for NTN.

Proposal 10: New granularity of Area Scope/Configuration should be introduced for logged MDT enhancement for NTN.

Since whether to introduce the above enhancement of RLF report and logged MDT for NTN also depends on RAN2 and the corresponding TU in RAN2 is limited, it could be beneficial to inform RAN2 of RAN3’s understanding on the SON/MDT enhancements for NTN.

Proposal 11: An LS on the enhancement of RLF report and logged MDT for NTN should be sent to RAN2.

3  Conclusion

Involvement of IoT NTN

Proposal 1: In Rel-19, IoT NTN should be also considered in the SON/MDT enhancements for intra-NTN mobility, and the progress of NR NTN can be reused for IoT NTN.

MRO for intra-NTN mobility failure

Proposal 2: Regarding the MRO for intra-NTN CHO with time-based trigger condition, the time information when the failure happens (e.g. absolute time) should be included in the RLF report.

Proposal 3: Regarding the MRO for intra-NTN CHO with location-based trigger condition, the location information where the failure happens should be included in the RLF report.

Proposal 4: An indication to distinguish the different types of CHO (measurement/time/location-based) should be included in the RLF report.

Proposal 5: There is no extra need to enhance the RLF report for the CHO with multiple trigger conditions.

Proposal 6: Regarding the MRO for non-conditional intra-NTN handover, the existing timer (e.g., T311) could be extended to support the delay-tolerant services.

Proposal 7: RAN3 should apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

Proposal 8: Regarding the MRO for RACH-less handover, an indication on whether the UE has performed the RACH-less HO fallback before the intra-NTN mobility failure should be included in the RLF report.

Proposal 9: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

Logged MDT enhancement

Proposal 10: New granularity of Area Scope/Configuration should be introduced for logged MDT enhancement for NTN.

Proposal 11: An LS on the enhancement of RLF report and logged MDT for NTN should be sent to RAN2.

The corresponding LS is given in the Annex.

References

- RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, CMCC

- TS 38.331 v18.2.0

- TS 38.300 v18.2.0

- TR 38.821 v16.1.0

Annex

LS to RAN2 on RAN3’s progress on SON/MDT enhancements for NTN

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, NL, 19th – 23th Aug 2024

Title:	[Draft] LS on SON/MDT enhancements for intra-NTN mobility

Response to:	N/A

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	ZTE (to be RAN3)

To:	RAN2

Cc:	N/A

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 has discussed the scope of the work related to the support of SON/MDT enhancements for intra-NTN mobility and identified the following issues RAN2 may need to support:

- - The enhancement of RLF report used for intra-NTN mobility, including the both the non-conditional and conditional handover.

    - - For the conditional handover with time-based trigger condition, the time information when the failure happens should be introduced in the RLF report.

    - - For the conditional handover with location-based trigger condition, the location information where the failure happens should be introduced in the RLF report.

    - - An indication to distinguish the measurement-based/time-based/location based intra-NTN conditional handover should be introduced in the RLF report.

    - - An indication on whether the UE performs the RACH-less HO fallback for the intra-NTN mobility failure

- - The enhancement of logged MDT for NTN, including the new Area Configuration over Uu.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #125bis     14th – 18th, Oct. 2024 		 Hefei, China

TP for TS 38.300 on connection failure scenarios for intra-NTN mobility

--------------------------------------------------------TS 38.300-------------------------------------------------------------------------

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of intra-NTN mobility, the Too Late Handover, Too Early Handover, Handover to Wrong Cell, Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell are all applied.

--------------------------------------------------------TS 38.300--------------------------------------------------------------------------