3GPP TSG-RAN WG3 Meeting #125	R3-244462

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	14.3

Source:	ZTE Corporation

Title:	Further discussion on support of regenerative payload

Document for:	Discussion

1  Introduction

In last two meetings, the discussion on support of regenerative payload was triggered, and the NG interface management for regenerative payload was discussed. However, there was no consensus on this issue among companies.

In this contribution, we provide our further views on the support of regenerative payload for NTN in Rel-19.

2  Discussion

2.1  NG connection management

For the management of NG connection, there are three options on the table.

Option 1: NG Setup/Removal mechanism

Option 2: NG Suspend/Resume mechanism

Option 3: Graceful removal of NG/S1 using legacy SCTP SHUTDOWN

For Option 1, when the gNB decides to leave the service area of AMF based on the ephemeris information, it shall trigger a new NG Removal procedure to the AMF to release or handover its served UEs. After the NG Removal procedure, the UE context should be released and the NG interface is completely removed. When the gNB re-enters the service area of AMF, it triggers the existing NG Setup procedure to the AMF.

For Option 2, when the gNB decides to leave the service area of AMF based on the ephemeris information, it shall trigger the existing NGAP procedure (e.g. RAN Configuration Update procedure) to the AMF to suspend the NG interface instead of removing it. In this case, the gNB and the AMF shall store the UE context and the configuration information related to the NG interface. When the gNB re-enters the service area of AMF, the stored UE context and configuration information could be reused, and the NG interface shall be resumed.

Considering the NGAP signalling, two NGAP procedures need to be handled for Option 1, i.e., the NG Setup procedure and NG Removal procedure. Furthermore, the NG Removal procedure is a totally new NGAP procedure. While, for Option 2, only one NGAP procedure need to be handled, e.g, the existing RAN Configuration Update procedure. Therefore, from the prospective of standard impact on NG interface, the Option 2 is more appropriate.

Considering the energy consumption, for Option 2, since the gNB and AMF shall store the UE context and configuration information, it shall consume some energy for storage. While, for Option 1, although there is no storage energy consumption for the gNB, it will still consume some extra energy for sending the additional NGAP signalling. As the satellite is far away from the NTN GTW, the energy consumption for NGAP signalling transmission cannot be overlooked. Therefore, from the prospective of energy consumption, it is hard to decide which option is better, which may need to be evaluated by satellite vendors.

For Option 3, since the working procedure shall apply the legacy SCTP SHUTDOWN without any explicit NGAP signalling exchange, this option should be regarded as implementation way, which may lead to Inter-operation issue.

Proposal 1: The NG Suspend/Resume mechanism should be applied for the management of NG connection.

2.1  Misalignment between SA2 and RAN3

In TR 23.700-29 [1] from SA2, SA2 has considered the reply LS from RAN3 on support of Regenerative-based satellite access in [2], and the final conclusion on support of Regenerative-based satellite access is given as below.

The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

While, for the second bullet, it seems that the SA2’s understanding is not aligned with RAN3.

According the reply LS from RAN3 on the IP address changes due to soft feeder link switch issue, RAN3 only confirms that ‘gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures’. And based on the online discussion, the common understanding is that the eNB IP address changes due to soft feeder link switch cannot be supported by using existing S1 procedures, since the Multiple SCTP associations are not support in E-UTRAN.

Therefore, RAN3 should send a LS to SA2 to confirm RAN3’s understanding on the eNB IP address changes due to soft feeder link switch issue.

Proposal 2: RAN3 should send a LS to SA2 to confirm RAN3’s understanding on the issue of eNB IP address changes due to soft feeder link switch.

3  Conclusion

Observation : RAN3 shall focus on the management of NG connection in this meeting.

Proposal 1: The NG Suspend/Resume mechanism should be applied for the management of NG connection.

Proposal 2: RAN3 should send a LS to SA2 to confirm RAN3’s understanding on the issue of eNB IP address changes due to soft feeder link switch.

The draft LS to SA2 in given in the Annex.

References

- TR 23.700-29 v19.0.0

- R3-243954, Reply LS on Support of Regenerative-based Satellite Access, RAN3

Annex

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, NL, 19th – 23th May 2024

Title:	[Draft] LS on RAN3’s view on eNB IP address changes due to soft feeder link switch

Response to:	N/A

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	ZTE (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

In TR 23.700-29, SA2 has taken RAN3’s reply LS on support of Regenerative-based satellite access (i.e. R3-243954) into account and made the final conclusion.

According to the R3-243954, for the IP address changes due to soft feeder link, RAN3 only confirmed that ‘gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures’. And it was RAN3’s common understanding that the eNB IP address changes due to soft feeder link switch cannot be supported by using existing S1 procedures, since the Multiple SCTP associations are not support in E-UTRAN.

However, in the latest TR 23.700-29, SA2’s conclusion is not aligned with RAN3’s understanding.

- gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG

Therefore, the TR 23.700-29 needs to be corrected to avoid the misalignment.

2. Actions:

To SA2 group.

ACTION: 	RAN3 kindly asks SA2 to take the above into consideration and correct the TR 23.700-29, if needed.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #125bis     14th – 18th, Oct. 2024 		 Hefei, China