3GPP TSG-RAN WG3 Meeting #125	R3-244464

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	15.2

Source:	ZTE Corporation

Title:	Initial considerations on support of Store and Forward satellite operation for IoT NTN

Document for:	Discussion

1  Introduction

The WID of Rel-19 NR NTN has been finalized in RAN#104 [1], and the RAN3 related objectives of this work item are shown as below.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In Rel-19, RAN3 shall focus on the support of Store&Forward satellite operation with full eNB as regenerative payload. And the TR 23.700-29 discussed in the SI phase of SA2 could be a valuable reference for RAN3’s WI phase.

In this contribution, we provide our initial considerations on the Store and Forward (S&F) mechanism, including the potential architecture and impact on RAN3.

2  Discussion

2.1 Potential architecture to support S&F mechanism

During the SI phase in SA2 [2], there are three options of architecture to support S&F satellite operation with full eNB as regenerative payload, i.e., Only eNB on board architecture, Split MME architecture and Full CN on board architecture.

2.1.1 Only eNB on board

The Only eNB on board architecture is the easiest solution to support the S&F mechanism among the three potential options. As given below, in step A, the signalling/data exchange between the UE and the eNB take place without the simultaneous connection between the eNB and the ground CN. In step B, the eNB is able to connect with the ground CN, and the communication between the eNB and the ground CN shall take place.

Figure 1. S&F satellite operation mode

2.1.2 Split MME

For the Split MME architecture, the corresponding description from SA2 is given as below.

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles: 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP. 3) The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS. 4) For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

In this architecture, the MME functionality is split into MME-onboard and MME-ground. And the MO data is send to MME-onboard and stored in MME-onboard in step A, then, the MO data is transferred to the MME-ground in step B. Meanwhile, the MT data is located in the MME-ground or the S-GW in step A and transferred to the MME-onboard in step B, then the MT data is stored in MME-onboard. Finally, the MT data is transferred to UE in step A.

The general working procedure of this architecture is similar with the Only eNB on board architecture, while the difference is where the data/RRC signalling is stored. For the Only eNB on board architecture, the data/RRC signalling is stored in the eNB, while for the Split MME architecture, the data/RRC signalling is stored in the MME-on board.

2.1.2 Full CN on board

For the Full CN on board architecture, the corresponding description from SA2 is given as below.

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. - The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list. NOTE 5: MT traffic is delivered to the UE after it performs an ATTACH. NOTE 6: After mobility from S&F operation, MT traffic could be stuck in the ground proxy and will only be retrieved once the UE goes back to S&F operation. For MT traffic, the UE attaches to a satellite and to allow delivery of MO traffic from the user or applications on the UE. A UE, based on implementation, could first wait for an indication from the user or from an application on the UE of pending MT traffic or could wait based on knowledge of when MT traffic may arrive, before performing the attach. - Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite. - Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network. NOTE 7: The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5].

Since the eNB and full CN are all located on board, the working procedure for this architecture is similar with the legacy IoT NTN, the MT and MO data are both stored in the MME.

As none of the three options is precluded by SA2, our RAN3 shall study all the three kinds of architecture at the beginning of the Rel-19, and discuss whether all the three kinds of architecture should be kept in Rel-19.

Proposal 1: For the support the Store and Forward mechanism in Rel-19 IoT NTN, RAN3 shall study all the three kinds of architecture from SA2 and decide the priority among the three options.

2.2 Single-satellite and multi-satellite scenario

All the mentioned three kinds of architecture can support both single-satellite and multi-satellite scenarios. For the single-satellite scenario, the satellite can only connect with the certain UE within one cycle of operation, which means it can only support the delay-tolerant IoT services. While, for the multi-satellite scenario, the satellites can relay the service data or the RRC signalling via ISL and the service delay could be much smaller than the single-satellite scenario.

However, the single-satellite scenario is much easier than the multi-satellite scenario, and it could be taken as the starting point.

Proposal 2: RAN3 shall consider both the single-satellite and multi-satellite scenarios, and the single-satellite scenario can be taken as the starting point.

2.3 Potential impact

Based on the conclusion in SA2’s TR, considering the potential impact on the S1 interface, some related information, such as the S&F indicator and wait timer, should be transferred from the MME to the UE. While, since these IEs are dedicated to the UE, whether they can be included in the NAS-PDU container directly or they should be explicitly included in the S1AP DOWNLINK NAS TRANSPORT message needs to be further checked. And the details could be discussed after RAN3 decides the scope of the architecture and scenarios in 2.1 and 2.2.

Meanwhile, RAN2 has already started this WI, and some agreements are also reached in the previous meetings.

S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

According to the above agreement, at least, for the eNB and MME co-located architecture where both the eNB and MME are aware of the ephemeris info, there is no need to transfer the S&F indication from MME to eNB, and it can be simply broadcast from eNB to the UE via SIBx. Therefore, when RAN3 starts the Rel-19 work, the agreements in RAN2 should also be checked.

Proposal 3: RAN3 shall discuss the detailed impact after deciding of the scope of the architecture and scenarios and take RAN2’s agreement into consideration.

3  Conclusion

Proposal 1: For the support the Store and Forward mechanism in Rel-19 IoT NTN, RAN3 shall study all the three kinds of architecture from SA2 and decide the priority among the three options.

Proposal 2: RAN3 shall consider both the single-satellite and multi-satellite scenarios, and the single-satellite scenario can be taken as the starting point.

Proposal 3: RAN3 shall discuss the detailed impact after deciding of the scope of the architecture and scenarios and take RAN2’s agreement into consideration.

References

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

- TR 23.700-29 v19.0.0