3GPP TSG-RAN WG3 Meeting #125						                      R3-244465

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	17.2

Source:	ZTE Corporation

Title:		Further discussion on on-demand SSB Scell operation

Document for:	Discussions & Approval

1	Introduction

This paper discusses the RAN3 aspect of the on-demand SSB Scell operation for energy saving.

2	Discussion

The following agreement in RAN1#116 meeting is noticeable from our RAN3 perspective.

Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE

Observation: The on-demand SSB Scell operation triggered by gNB is supported.

Although the timing to trigger the on-demand SSB Scell operation is still under discussion in RAN1, with the above agreement, the potential impact on network interfaces could be discussed in RAN3.

In our view, before triggering on-demand SSB Scell operation for UEs, for a Scell configured with on-demand SSB, beams(CD-SSB or NCD-SSB or both) of the this cell are turned off(case1: no always-on SSB) or periodically transmitted (case2: with always-on sparse SSB).When it comes to the CU-DU split architecture, after Scell(s) establishment for UEs via F1 UE context setup/modification procedure, DU decides to activate or deactivate SCell(s) for UEs, and indicates Scell (de)activation decision to UE via MAC CE. Therefore, it is also reasonable for DU to decide when to start/stop on-demand SSB transmission, e.g., to start transmission before the UE receives SCell activation command and to stop transmission after the UE receives SCell deactivation. In other words, DU determines the on-demand SSB Scell operation for UEs. On the other hand, we believe CU can decide whether to use the on-demand SSB function for Scells for the purpose of energy saving. Additionally,Some on-demand SSB configuration should also be determined by CU,e.g, no always-on mode, sparse always-on mode, periodicity etc. Other configurations, like SSB positions within a burst, should be determined by the DU.

Proposal 1: DU determines the on-demand SSB Scell operation for UEs, e.g.,to start/stop on-demand SSB transmission.

Proposal 2: CU determines whether to use the on-demand SSB function for Scells for the purpose of energy saving. Some on-demand SSB configuration should also be determined by CU,e.g, no always-on mode, sparse always-on mode, periodicity etc. Other configurations, like SSB positions within a burst, should be determined by the DU. (details up to RAN1).

According to the above proposals, RAN3 could discuss the potential F1AP impact and signaling. For example, the CU should obtain the full OD-SSB configuration and then provide the UE with this configuration via an RRC configuration. The DU also needs to obtain the full configuration for SSB transmission.

3	Conclusion

In this contribution, we have the following observations and proposals:

Observation: The on-demand SSB Scell operation triggered by gNB is supported.

Proposal 1: DU determines the on-demand SSB Scell operation for UEs, e.g.,to start/stop on-demand SSB transmission.

Proposal 2: CU determines whether to use the on-demand SSB function for Scells for the purpose of energy saving. Some on-demand SSB configuration should also be determined by CU,e.g, no always-on mode, sparse always-on mode, periodicity etc. Other configurations, like SSB positions within a burst, should be determined by the DU. (details up to RAN1).