# 3GPP TSG-RAN WG3 Meeting #125	R3-244469

# Maastricht, Netherlands, 19 - 23 Aug 2024

# Agenda Item:	21.1

# Source:	ZTE Corporation, China Telecom

# Title:	Discussion on SA2 LS with draft reply LS

# Document for:	Discussions & Approval

- Introduction

RAN3 received an LS(R3-244020) from SA2 before the start of this meeting. In this LS, following questions shall be responded by RAN3 and other WGs.

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

This paper is used to explain our views on RAN3 related questions and provide draft reply LS in annex.

# 2	Discussion

Several numbers of LS on XR have been received by RAN3 before the start of this meeting. In this paper, we prefer to further discuss questions contained in SA2 LS(R3-244020). Here are the RAN3 related questions in this LS:

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

In the current specification, the periodicity is provided by QoS flow, e.g. in TSCAI, and the UL periodicity can also be provided by UAI, which are semi-static, and mainly used for semi-static resource scheduling and/or configuration, e.g. for SPS configuration, CG configuration and/or C-DRX configuration. Till now, there is no use case for dynamic periodicity seen, we think the periodicity provided by QoS flow in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

Proposal 1: From RAN3 perspective,  the periodicity provided by QoS flow in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

RAN3 has already discussed the similar issue and sent our understanding to SA2 in reply LS(R3-243958) at the end of previous meeting. More specifically, RAN3 thinks that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. Companies in RAN3 have different views on how to provided the available data rate to CN(e.g., via CP or UP). In addition, Some companies in RAN3 would like to clarify the usage of available data rate for GBR QoS flows.

Proposal 2: RAN3 have different views on available data rate exposure for GBR QoS flow and prefer to clarify the usage.

A draft reply LS is prepared in annex and may be considered as baseline.

Proposal 3: It is proposed fro RAN3 to consider the draft reply LS in annex as baseline.

# 3	Conclusion

Based on above analysis, we provide the following proposals.

Proposal 1: From RAN3 perspective,  the periodicity provided by QoS flow in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

Proposal 2: RAN3 have different views on available data rate exposure for GBR QoS flow and prefer to clarify the usage.

Proposal 3: It is proposed fro RAN3 to consider the draft reply LS in annex as baseline.

# Annex:  draft reply LS to SA2

# 3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

# Maastricht, Netherlands, 19 - 23 Aug 2024

Title:	Response LS to SA2 on FS_XRM Ph2

Response to:	R3-244020/S2-2407351

Release:	Rel-19

Work Item:	FS_XRM Ph2

Source:	ZTE(shall be RAN3)

To:	SA2

Cc:	RAN2, SA4

Contact person:	Yansheng Liu

yansheng.liu@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks SA2 for the LS on FS_XRM Ph2(R3-244020/S2-2407351) and kindly ask SA2 to consider the following response into account.

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

[RAN3 answer to Question1]: From RAN3 perspective, the periodicity provided by QoS flow in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

[RAN3 answer to Question3]:

RAN3 has discussed similar issue in previous received SA2 LS and replied to SA2 in R3-243958.

2	Actions

To SA2

ACTION: 	RAN3 kindly asks SA2 to take above answers into account.

3	Dates of next RAN3 meetings

RAN3#125bis		14th – 18th October 2024			Hefei, China

RAN3#126			18th - 22rd November 2024 		Orlando, US