# 3GPP TSG-RAN WG3 Meeting #125	R3-244470

# Maastricht, Netherlands, 19 - 23 Aug 2024

# Agenda Item:	21.2

# Source:	ZTE Corporation, China Telecom, China Unicom

# Title:	Further discussion on XR supporting

# Document for:	Discussions & Approval

- Introduction

We made the following progress at the RAN3#124 meeting.

For XR in NR-DC, non-homogenous deployment is possible similar like other features in DC. WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer. For SN-terminated bearer, the MN provide the ECN Marking or Congestion Information Reporting Request per QoS flow to SN via the PDU Session Resource Setup Info – SN terminated IE, and PDU Session Resource Modification Info – SN terminated IE. Add list of QoS flow related to SN-terminated SCG bearers, and for each QoS flow, reporting the ECN Marking or Congestion Information Reporting status.

This paper further discusses how to support XR in DC. Another contribution on TP to BL CRs which is used to capture the proposals in this paper can be found in R3-244471.

# 2	Discussion

2.1	PDU set based handling

In Rel-18, non-homogenous support was considered for the Xn handover and UE context retrieval scenarios, i.e. the source/old NG-RAN node can know whether the target/new NG-RAN node supports PDU set based handling via the PDU Set based Handling Indicator IE so that the source/old NG-RAN nodes can decide whether to include the PDU Set Information Container in the data to be forwarded.

Observation 1: In Rel-18, the source/old NG-RAN node can know whether the target/new NG-RAN node supports PDU set based handling via the PDU Set based Handling Indicator IE for the Xn handover and UE context retrieval scenarios.

In DC case, non-homogenous support should also be considered between MN and SN, i.e. the MN needs to know whether the SN supports PDU set based handling to decide whether to include the PDU Set Information Container in the data to be forwarded to the SN. If the SN addition or SN modification request message includes the PDU Set QoS Parameters IE, the SN needs to send the PDU Set based Handling Indicator IE in the SN addition or SN modification request acknowledge message to indicate whether the SN supports PDU Set based handling.

Observation 2: The MN needs to know whether the SN supports PDU set based handling to decide whether to include the PDU Set Information Container in the data to be forwarded to the SN.

One WA on the non-homogeneous support for DC case has been agreed in previous meeting. In previous meeting, it is proposed for RAN3 to turn this WA to agreement.

Proposal 1: Turn the following WA to agreement:

- WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

2.2	PSI Discard coordination

An LS(R3-243957) on this issue has been sent to RAN2 at the end of previous meeting. RAN3 may postpone the discussion until RAN2 replies.

Proposal 2: Further discussion on whether/how to support (de)activation of the UL PSI based discard for the split bearer in DC shall be re-started with the conclusion from RAN2.

2.3	Burst Arrival Time reporting

RAN2 has defined the UL BAT indicated as referenceTime or referenceSFN-AndSlot. When the BAT is indicated as referenceSFN-AndSlot, it is a relative time to PCell. If the PCell and PSCell are not synchronized, there may be issue if the SN uses the BAT indicated as referenceSFN-AndSlot.

burstArrivalTime-r18 CHOICE { referenceTime ReferenceTime-r16, referenceSFN-AndSlot ReferenceSFN-AndSlot-r18 }

UL-TrafficInfo field descriptions

burstArrivalTime Indicates the expected arrival time of the first packet of the Data Burst for the concerned QoS flow. If the UE provides both burstArrivalTime and jitterRange, burstArrivalTime is used as a reference time for the indicated jitter range. If burstArrivalTime is indicated as referenceTime, the indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If burstArrivalTime is indicated as referenceSFN-AndSlot, it refers to the UL timing of the closest SFN and slot of the PCell with the indicated number.

Observation 3: When the UL BAT is indicated as referenceSFN-AndSlot, it is a relative time to PCell.

Observation 4: If the PCell and PSCell are not synchronized, there may be issue if the SN uses the UL BAT indicated as referenceSFN-AndSlot.

More specifically,as shown in highlighted content, the referenceSFN-AndSlot is a kind of relative timing which refers to the closet SFN and slot in specific period(e.g. DRX). As shown in the figure below, a wrap around issue may be happened if  referenceSFN-AndSlot are forwarded to other network node at different SFN cycle from the SFN cycle when it it is received.

Figure 1: Mismatch between XR Burst arrival and the indicated SFN, if the referenceSFN-AndSlot is sent to other network node in next SFN cycle but the value is not updated

2 different ways may be selected to solve this potential issue:

- MN guarantees the forwarded UL BAT info can be directly used by SN.( MN may modify the received UL BAT from UE to avoid the wrap around issue happens at SN side.)

- SN may re-calculate the received UL BAT info from MN to avoid the wrap around issue happens.

Considering this is the last meeting of the Rel-19 XR WI, we prefer to solve this issue with the minimum spec impact at current stage. Moreover,a similar issue exists for Rel-18 XR is that for the handover case, the target gNB can not use the BAT received from the source gNB directly if the source and the target gNBs are not synchronized. Therefore, we endorsed R3-241784 to clarify that the target gNB needs to recalculate the burst arrival time based on the SFN offset between the source and target gNBs when they are not synchronized.

Observation 5: For Rel-18 XR, we endorsed R3-241784 to clarify that the target gNB may need to recalculate the burst arrival time based on the SFN offset between the source and target gNBs when they are not synchronized.

We think a similar implementation based solution should be considered for the DC case to keep align with the handover case, i.e. the SN uses the SFN offset between the MN and SN to get the correct BAT.

Proposal 3: The SN uses the SFN offset between the MN and SN to get the correct BAT when the MN and SN are not synchronized.

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Observation 1: In Rel-18, the source/old NG-RAN node can know whether the target/new NG-RAN node supports PDU set based handling via the PDU Set based Handling Indicator IE for the Xn handover and UE context retrieval scenarios.

Observation 2: The MN needs to know whether the SN supports PDU set based handling to decide whether to include the PDU Set Information Container in the data to be forwarded to the SN.

Proposal 1: Turn the following WA to agreement:

- WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

Proposal 2: Further discussion on whether/how to support (de)activation of the UL PSI based discard for the split bearer in DC shall be re-started with the conclusion from RAN2.

Observation 3: When the UL BAT is indicated as referenceSFN-AndSlot, it is a relative time to PCell.

Observation 4: If the PCell and PSCell are not synchronized, there may be issue if the SN uses the UL BAT indicated as referenceSFN-AndSlot.

Observation 5: For Rel-18 XR, we endorsed R3-241784 to clarify that the target gNB may need to recalculate the burst arrival time based on the SFN offset between the source and target gNBs when they are not synchronized.

Proposal 3: The SN uses the SFN offset between the MN and SN to get the correct BAT when the MN and SN are not synchronized.