3GPP TSG-RAN WG3 Meeting #125	R3-244476

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Further discussion on Inter-CU LTM signaling design

Document for:	Discussion and Decision

# Introduction

For Rel-19 Inter-CU LTM, RAN3 has progressed as follows:

RAN3-123bis (Changsha)

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3-124 (Fukuoka)

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

In this contribution, we continue providing our views for Inter-CU LTM.

# Discussion

## General consideration

The core features of LTM compared to other mobility solutions are early DL/UL synchronizations and cell switch based on L1 measurement (triggered by MAC CE) [1]. Such core features have to be continued on Rel-19 extension for Inter-CU LTM, and thus RAN3 has agreed to extend the TA Information Transfer (for early UL synchronization) and Cell Switch Notification specified over F1AP in Rel-18 toward across CUs:

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Introduce a new procedure on Xn to transfer the TA information.

Regarding TA handling, additional things have been worked out in Rel-18 that CU can share with a DU the following: (a) a known TA value (i.e. TA value = 0) between the current serving cell and a candidate cell; and (b) whether UE-based TA measurement can be applied (and configured to the UE) in-between the serving and candidate cells. The former was an optimization to cater a special deployment (co-located cells), while the latter was one of early UL synchronization mechanism specified that is utilized only when DU knows so.

While those TA handling is perfectly fine within a single CU (as specified in Rel-18), the question is whether they can be applicable between cells lying under different CUs. It is hard to imagine that either co-located cells are managed under different CUs, or the 260ns TAE (timing alignment error) requirement could be satisfied between cells under different CUs [2]. We think it is better for RAN3 to specify those TA handling across CUs once their usage across CUs is validated.

Proposal 1: RAN3 works on additional TA handling specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), once their usage across CUs is validated.

Moreover, the WID has been revised in RAN-104 (Shanghai) and the second priority works of inter-CU LTM when DC is configured has been further clarified [3]:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

RAN2-126 (Fukuoka) also agreed as follows:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

In Rel-18, when DC is configured, independent “intra-CU” LTM operations from either MN or SN were allowed for their respective cells. The RRC signalling specified in Rel-18 allowed NW to configure LTM over MCG and over SCG, as long as it does not incur simultaneous PCell and PSCell changes to the UE. Given that Rel-18 LTM was limited to “Intra-CU” mobility, the serving MN-CU or the serving SN-CU was able to independently decide LTM over their respective PCells or PSCells for a UE, without having to coordinate with each other.

In Rel-19, on the other hand, such independent operations become limited when Inter-CU LTM is involved in MN or SN. As clarified above in [3], Inter-CU LTM shall not be configured in both MN and SN at the same time for a UE. As a result, some coordination between MN and SN may be needed to prevent simultaneous LTM configuration in both MCG and SCG in Rel-19 Inter-CU LTM scenarios. For example, when Inter-MN LTM (or Inter-SN LTM) is being prepared and configured to the UE, the other peer, i.e. SN (or MN) needs to know such so that it does not initiate the Inter-SN LTM (or Inter-MN LTM) for the same UE. However, those coordination efforts are related to the second priority and we think they should be studied and worked out only after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

Proposal 2: Any necessary coordination between MN and SN to prevent simultaneous Inter-CU LTM configuration in both MCG and SCG in Rel-19 should be studied and worked out, only after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

The same applies to the modification of LTM configuration that we have currently FFS:

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Given that the initial Inter-CU LTM preparation details are still nascent, we believe that it is premature to discuss their modification or release or cancellation. They can be worked out after the preparation details are stabilized.

Proposal 3: Prioritize discussions on Inter-CU LTM preparation details.

## Inter-CU LTM preparation details

Regarding the preparation, RAN3 has agreed as follows:

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

RAN2-126 (Fukuoka) also agreed the following:

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

But overall, still high-level and far from completion. Re-using the existing HO procedure initiated by S-CU was expected, but there are many important aspects that should be agreed before we progress further.

In Rel-18, the initial LTM candidate cells were completely decided by the source CU (i.e. S-CU) based on L3 measurement report, and the final cells were selected based on admission results with the involved DU(s).

Now in Rel-19, the scenario considers one or more CUs (i.e. candidate CU, a.k.a. C-CU) in addition to S-CU. One may think that C-CU may also decide “initial” candidate cells for LTM (like SN in DC case), however, the HO principle has been that the source decides the cell for HO where the target performs admission control. HO is different to DC, and Inter-CU LTM should follow the same way where S-CU shall decide initial LTM candidate cells based on L3 measurement report, and the final cells are selected based on admission results with the involved CU/DUs.

Proposal 4: For Inter-CU LTM,

- S-CU decides initial LTM candidate cells based on L3 measurement report, and the final cells are selected/configured based on admission results with all the involved CU/DUs (following the existing HO principle).

Once LTM is decided for a UE and the initial LTM candidate cells are chosen by S-CU, following what had been specified in Rel-18, there should be at least two round-trip of interactions between S-CU and each C-CU. When multiple entities are involved, such two round-trip is inevitable because admission control and generating final configuration to the UE cannot be done simultaneously. The first round-trip shall be for initial context setup and retrieval of necessary configurations so that S-CU can collect the configurations and information of admitted candidate cells to generate the common lower-layer configuration for candidate cells. The second round-trip shall be to exchange the collected information and finalizing configuration to the UE from each C-CU. This observation is exactly aligned with RAN2 agreement in (3-6) above.

The procedure for the first round-trip shall be definitely the very HO procedure for Inter-CU LTM, triggered per each candidate cell. RAN2 already agreed that the maximum number of LTM candidate cells is 8, regardless of intra-CU or inter-CU. This number 8 is the same to that of CHO (maxnoofCellsinCHO = 8), that of (S-)CPAC (maxnoofPSCellCandidates = 8), and that of the combination of both (i.e. CHO+CPAC). From Rel-16, the parallel CHO preparations per cell basis have been working well. There is no reason to deviate differently for Inter-CU LTM.

On the other hand, the second round-trip doesn’t have to stick to the existing HO procedure. It is not about “modification” of already prepared. For Inter-CU LTM, the second round-trip is rather a post-LTM-preparation to finalize LTM configuration to the UE with each C-CU based on the common collected information from all the admitted candidate cells. As a result, it doesn’t have to be triggered per cell basis. A UE-associated signalling looks enough (like what we did in Rel-18 over F1AP), and for that, we think it is better to define a new class-1 UE-associated signalling for such post-LTM-preparation procedure.

Proposal 5: Once LTM is decided for a UE and the initial LTM candidate cells are chosen by S-CU,

- Inter-CU LTM requires at least two round-trip interactions between S-CU and each C-CU (similar to what was done in Rel-18 between CU and DU, and as RAN2 already agreed):

- The first round-trip shall be the existing HO procedure, triggered by S-CU (already agreed) per each candidate cell basis, given that RAN2 already agreed the maximum number of LTM candidate cells is 8 (regardless of intra-CU or inter-CU) and that number 8 is the same to that of Rel-16 CHO candidate cells already based upon the per-cell parallel CHO preparations.

- The second round-trip is rather a post-LTM-preparation with each C-CU to finalize LTM configuration to the UE based on the common collected information from all the admitted candidate cells, so it doesn’t have to stick to the existing HO procedure or be triggered per cell basis. A UE-associated signalling is enough (like what we did in Rel-18 over F1AP), and it is better to define a new class-1 UE-associated signalling for such post-LTM-preparation procedure triggered by S-CU.

With that, mirroring what we have done in Rel-18 Intra-CU LTM and S-CPAC (for which we can borrow some inter-node subsequent mobility operations related to data forwarding), we can deduce the following.

Proposal 6: When initiating Inter-CU LTM preparation to a C-CU, the S-CU triggers the existing HO procedure per each candidate cell, and the below information can be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - LTM initiation indication

  - LTM reference configuration, or its request.

  - SSB / CSI resource configuration of candidate cells (if available)

  - Early Sync Information Request (already agreed), which can contain a list of DU ID(s) associated with S-CU (including S-DU) for LTM.

- (per-cell) HO REQ ACK message to include:

  - LTM reference configuration if requested

  - SSB / CSI resource configuration allocated from the admitted cell

  - Early Sync Information (already agreed), which contains DL/UL early sync info allocated from the admitted candidate cell

  - Data Forwarding proposal to be used for subsequent LTM

  - LTM candidate configuration (i.e. RRCReconfiguration) for the admitted cell (which is via HO CMD) + its complete configuration indicator (if LTM reference configuration is provided)

Proposal 7: After the first round of initial preparation with each C-CU, the S-CU triggers the post-LTM-preparation procedure with each C-CU:

- Define a new class-1 UE-associated signalling for the post-LTM-preparation procedure.

- Post-LTM-preparation REQ message to include:

  - LTM reference configuration

  - SSB / CSI resource configuration of all the candidate cells admitted by other CU(s)

  - Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell.

  - LTM Configuration ID Mapping List (each admitted cell is with a unique LTM candidate ID)

  - Data Forwarding TNLs assigned by other involved CU(s)

- Post-LTM-preparation RESP message to include the following information:

  - L1 measurement reporting configuration from each admitted candidate cell

  - (Updated) LTM candidate configuration (i.e. RRCReconfiguration) for each admitted candidate cell + their complete configuration indicator (if LTM reference configuration is provided)

Other information could be further discussed, but we believe that the above information exchange over the first/second round trips are required and essential for the Inter-CU LTM configuration preparation across S-CU and C-CU(s).

## E1AP impacts

E1AP is finally included as an impacted spec in the updated WID [3], based on the following observations [4]:

E1-AP potential impacts

1.       Add the abbreviations of LTM/Conditional LTM.

2.       Clarify that some procedures can be reused for LTM (e.g. Early Forwarding SN Transfer).

3.       The enhancement on the BEARER CONTEXT SETUP procedure to indicate the request concerns a conditional LTM cell switch.

4.       The security handling for inter-CU LTM (Maybe, depends on RAN2 solution).

5.       Some enhancement on the UP handling to support early data forwarding for LTM (Maybe, depends on RAN3 solution).

While (4) and (5) are further to discuss, (1)-(3) are quite straightforward, given that

- LTM shares the similar design philosophy with conditional mobility, where serving cell switch does not happen immediately upon RRC configured.

- RAN3 already agreed early data forwarding for Inter-CU LTM:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

- Conditional LTM is yet to come, but from CU-UP standpoint, no difference is expected for (1)-(3).

As a result, we just need to extend their usage in E1AP also to cater for LTM (which was neglected in Rel-18 as explained in [1]), and their impact is small. As an E1AP BLCR rapporteur, we have provided the corresponding E1AP BL CR in [5].

Proposal 8: Endorse the E1AP BL CR proposed in [5].

## Stage-3 for XnAP class-2: TA Information Transfer, LTM Cell Switch Notification, and HO SUCCESS

For Inter-CU LTM, RAN3 reached fast consensus on the necessary XnAP extensions to support early TA acquisition and PCell switch execution across CUs, all of which shall be by class-2:

Introduce a new procedure on Xn to transfer the TA information.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

Especially, the usage of HO SUCCESS was agreed because cell switch notification doesn’t guarantee the successful serving PCell change. As explained in [6], the purpose of cell switch notification is rather more on delivering the right beam to be used by the new serving DU for faster mobility. Moreover, in Rel-18, RAN2 allowed “failure/single-reattempt” for LTM (controlled by NW). That is, in case the UE fails to execute LTM, it can be pre-configured to try LTM once more if the suitable cell found by cell re-selection is one of the candidate cells configured to the UE. In such scenario, the UE reverts back to the original configuration and re-starts from there. The selected suitable candidate cell may belong to a different CU than the CU who received cell switch notification from the previous serving CU. Due to these reasons, HO SUCCESS for Inter-CU LTM was agreed to make sure to inform the previous serving CU which PCell the UE actually (and successfully) accessed after executed.

While the above agreements are written from S-CU point of view, in fact, early TA and PCell switch execution can be triggered from any CU serving the UE. And if triggered between C-CUs (i.e. from one C-CU to another C-CU), each procedure has to be routed via S-CU because there is no direct UE-associated XnAP connection established in-between C-CUs:

- (Class-2, non-UE associated) TA Information Transfer is triggered from a CU who receives a preamble and calculates the TA value toward the current serving CU who triggers that early TA acquisition procedure. If any of both CUs are not S-CU, then should be routed via S-CU.

- (Class-2, UE-associated) LTM Cell Switch Notification is triggered from the current serving CU to the target CU. If any of both CUs are not S-CU, then it should be routed via S-CU.

- (Class-2, UE-associated) HO SUCCESS is triggered from the new serving CU to the previous serving CU when Inter-CU serving PCell change is successful. If any of both CUs are not S-CU, then it should be routed via S-CU.

The following Figure 1 depicts an example signalling flow for Inter-CU LTM cell switch notification, TA information transfer, and HO SUCCESS:

Figure 1: An example signalling flow for Inter-CU LTM cell switch notification, TA information transfer, and HO SUCCESS across CUs

Normally a class-2 procedure is described from one node to another node. However, for Inter-CU LTM, as explained above, the fact that these procedures can be triggered from any involved CU which may have to be routed via S-CU requires a delicate touch when capturing their stage-3 details into XnAP.

Moreover, when routing vis S-CU is required for these class-2 procedures, we observe that no additional specification efforts are required to support that, because:

- (Class-2, non-UE associated) TA Information Transfer: Given that it is always triggered toward the current serving CU, the S-CU (who is always aware that the UE is currently served by which CU) can determine whether it is destined to itself or needs to be forwarded to the (other) C-CU currently serving the UE.

- (Class-2, UE-associated) LTM Cell Switch Notification: Given that the target PCell ID is included in the message, the S-CU can determine whether it is addressed to itself or needs to be forwarded to the (other) C-CU serving that target PCell.

- (Class-2, UE-associated) HO SUCCESS: Given that it is triggered every time successful Inter-CU serving PCell change happens, the S-CU (who is always aware that the UE was previously served by which CU) can determine whether it is destined to itself or needs to be forwarded to the (other) C-CU previously served the UE.

Considering these aspects, we have provided an example on how to properly capture their stage-3 details in our proposed TP [7].

Proposal 9: Capture the stage-3 details of XnAP class-2 procedures (TA Information Transfer, LTM Cell Switch Notification, and HO SUCCESS) considering that these procedures can be triggered from any involved CU which may have to be routed via S-CU.

Proposal 10: Agree the XnAP TP proposed in [7].

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

General consideration

Proposal 1: RAN3 works on additional TA handling specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), once their usage across CUs is validated.

Proposal 2: Any necessary coordination between MN and SN to prevent simultaneous Inter-CU LTM configuration in both MCG and SCG in Rel-19 should be studied and worked out, only after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

Proposal 3: Prioritize discussions on Inter-CU LTM preparation details.

Inter-CU LTM preparation details

Proposal 4: For Inter-CU LTM,

- S-CU decides initial LTM candidate cells based on L3 measurement report, and the final cells are selected/configured based on admission results with all the involved CU/DUs (following the existing HO principle).

Proposal 5: Once LTM is decided for a UE and the initial LTM candidate cells are chosen by S-CU,

- Inter-CU LTM requires at least two round-trip interactions between S-CU and each C-CU (similar to what was done in Rel-18 between CU and DU, and as RAN2 already agreed):

- The first round-trip shall be the existing HO procedure, triggered by S-CU (already agreed) per each candidate cell basis, given that RAN2 already agreed the maximum number of LTM candidate cells is 8 (regardless of intra-CU or inter-CU) and that number 8 is the same to that of Rel-16 CHO candidate cells already based upon the per-cell parallel CHO preparations.

- The second round-trip is rather a post-LTM-preparation with each C-CU to finalize LTM configuration to the UE based on the common collected information from all the admitted candidate cells, so it doesn’t have to stick to the existing HO procedure or be triggered per cell basis. A UE-associated signalling is enough (like what we did in Rel-18 over F1AP), and it is better to define a new class-1 UE-associated signalling for such post-LTM-preparation procedure triggered by S-CU.

Proposal 6: When initiating Inter-CU LTM preparation to a C-CU, the S-CU triggers the existing HO procedure per each candidate cell, and the below information can be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - LTM initiation indication

  - LTM reference configuration, or its request.

  - SSB / CSI resource configuration of candidate cells (if available)

  - Early Sync Information Request (already agreed), which can contain a list of DU ID(s) associated with S-CU (including S-DU) for LTM.

- (per-cell) HO REQ ACK message to include:

  - LTM reference configuration if requested

  - SSB / CSI resource configuration allocated from the admitted cell

  - Early Sync Information (already agreed), which contains DL/UL early sync info allocated from the admitted candidate cell

  - Data Forwarding proposal to be used for subsequent LTM

  - LTM candidate configuration (i.e. RRCReconfiguration) for the admitted cell (which is via HO CMD) + its complete configuration indicator (if LTM reference configuration is provided)

Proposal 7: After the first round of initial preparation with each C-CU, the S-CU triggers the post-LTM-preparation procedure with each C-CU:

- Define a new class-1 UE-associated signalling for the post-LTM-preparation procedure.

- Post-LTM-preparation REQ message to include:

  - LTM reference configuration

  - SSB / CSI resource configuration of all the candidate cells admitted by other CU(s)

  - Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell.

  - LTM Configuration ID Mapping List (each admitted cell is with a unique LTM candidate ID)

  - Data Forwarding TNLs assigned by other involved CU(s)

- Post-LTM-preparation RESP message to include the following information:

  - L1 measurement reporting configuration from each admitted candidate cell

  - (Updated) LTM candidate configuration (i.e. RRCReconfiguration) for each admitted candidate cell + their complete configuration indicator (if LTM reference configuration is provided)

E1AP impacts

Proposal 8: Endorse the E1AP BL CR proposed in [5].

Stage-3 for XnAP class-2: TA Information Transfer, LTM Cell Switch Notification, and HO SUCCESS

Proposal 9: Capture the stage-3 details of XnAP class-2 procedures (TA Information Transfer, LTM Cell Switch Notification, and HO SUCCESS) considering that these procedures can be triggered from any involved CU which may have to be routed via S-CU.

Proposal 10: Agree the XnAP TP proposed in [7].

# Reference

[1] R3-242096, “High-level principles and discussions for Inter-CU LTM signaling design”, LG Electronics

[2] R3-235019/R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

[3] RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[4] RP-241516, “Discussion on rel-19 mobility enhancements WID revision”, Apple Inc, China Telecom

[5] R3-244477, “Baseline CR for introducing Rel-19 Mobility enhancement in E1AP”, LG Electronics Inc.

[6] R3-243528, “Further discussions on Inter-CU LTM signaling design”, LG Electronics Inc.

[7] R3-244478, “(TP for NR_Mob_Ph4 TS 38.423)”, LG Electronics Inc.