3GPP TSG-RAN WG3 Meeting #125	R3-244479

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	14.3

Source: 	LG Electronics Inc.

Title: 	Discussion on N2/S1 connection management

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN3 received questions from SA2 about several issues for the support of Rel-19 Regenerative-based satellite access [1]. The issue of N2/S1 connection management was not answered back to SA2 [2] due to lack of consensus in RAN3:

With respect to NG connection management: RAN3 has just begun to discuss the options, and there is no conclusion yet. RAN3 will continue discussing this issue in Rel-19 and inform SA2 on the conclusion. With respect to IP address changes due to soft feeder link switch: RAN3 confirms that the gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures. With respect to Mapped Cell ID handling: RAN3 also thinks that the AMF/MME can treat the Mapped Cell ID as in previous releases.

In the meantime, the following is captured as the conclusions from SA2 study [3]:

## 8.1 Conclusion for KI#1: Support of Regenerative-based satellite access The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

For this issue, we provide our analysis from NG point of view (assuming its progress will be mirrored to S1 through IoT-NTN WI) and propose the corresponding TP [4] for NGAP [5].

# Discussion

According to TS 38.413 [5], the main purpose of NG connection management procedures is to exchange the application-level data between gNB and AMF. Unless mistaken, the application-level data is basically TACs (earth-fixed), and their associated PLMNs and slices supported.

NG Setup has been designed to be initiated from a gNB. Before NG connection is established, a gNB must be pre-configured with the IP address of the AMF that it should initiate the initial NG connection setup to, and this part is out of 3GPP scope. When it comes to NTN deployments, we also think no exception for satellite on-board gNBs – though each satellite follows the pre-determined orbits and visits a specific geographical area for specific time duration as pre-planned, IP addresses of AMFs (or NTN gateways) on the grounds along its pre-determined orbits (could be a few tens or hundreds) and the point of time when to initiate NG connection setup to which one should be pre-configured. Of course, once NG interface becomes operational, more than one NG interface instances (or SCTP associations) could be added or removed in the middle until it leaves the AMF’s service area. But from the perspective of a satellite on-board gNB, it is hard to imagine that the information necessary for establishing initial NG connection with which AMF at which time would be dynamically changed.

We think this is the same to the basic configuration information at gNB. In NTN, TAC is earth-fixed. For each TAC, which PLMNs or which slices to be supported can be pre-configured. Given it is the gNB who initiates NG connection setup, those basic configuration data anyway have to be stored in a satellite on-board gNB for all the geographical areas that it visits along its orbit (also considering earth rotation) and which data gets to be used for initial NG interfacing at specific time durations.

Moreover, over NG, tracking area information is uni-directional from gNB to AMF, i.e. earth-fixed TACs supported by a gNB is only reported/updated to AMF. Unless mistaken, this seems to indicate that NGAP has not been designed in a way that the TACs supported by a gNB gets negotiated with AMF during initial NG setup or gets updated by AMF in the middle. Conversely, this also seems to indicate that the service area (in terms of tracking areas) of a specific AMF is not exposed to a gNB over NGAP. It is not entirely clear how a gNB is made aware of the exact service areas of AMFs or selects a right serving AMF for UEs beyond PLMN level (could be OAM magic). However, at least from NGAP point of view under RAN3 turf, we think that two options on the table (NG Removal, NG Suspend/Resume) don’t seem to make sense, for which the most companies seem to believe to be initiated by a regenerative payload. What looks clear from the current NGAP signalling design is that AMF can be made aware of whether its NG-connected gNB is leaving its service area or not. So, if we want a graceful removal of NG, then AMF should be the one who triggers that.

Given that a satellite beam may cover multiple (earth-fixed) tracking areas, after NG interface becomes operational with a specific AMF, the supported TACs from a satellite on-board gNB would keep changing as it moves, and will be kept updated to AMF via the NGAP RAN Configuration Update procedure. The AMF can predict when this gNB will leave its service area (or could be pre-configured by NTN control function based on the gNB’s ephemeris data). By the time that NG interface needs to be tore down with this gNB, the AMF can trigger the NGAP AMF Configuration Update procedure and tear down all the SCTP associations with this gNB using the AMF TNL Association to Remove List IE. Once acknowledged by the gNB at satellite, all the SCTP associations between two can be removed, leading to NG interface tear-down. Based on these reasonings, there seems no need to introduce new procedure or update the existing procedure to support when a regenerative payload leaves the service area of a specific AMF for which NG interface has been established with.

Once the tear-down is acknowledged, AMF can remove all the application-level data associated with the tore-down NG interface from its database. We think there is no point for AMF to store the application-level data of this regenerative payload, because anyway the regenerative payload would contact the AMF again when it comes back and the stored data (supported TACs, etc.) would have to be updated (see the example of San Franscisco and New York in [6]). The form of the resume procedure initiated by a regenerative payload won’t be that much different to that of the existing NG Setup procedure anyway. Moreover, AMF is on the ground and it is a gNB at satellite who is moving. Even though the regenerative payloads would re-enter at some point of time in future, we observe no compelling motivation to make an AMF maintain NG interfaces (deactivated due gNBs getting out of its service area), for which the number of payloads is expected to be huge (e.g. tens of thousands) in NTN industries [6].

In the same way, we also think it is inefficient to make a regenerative payload store the application-level data of the deactivated NG interfaces with the AMFs that it has gone through. It can simply remove those data and receive/store again for only a few NG connections active over the course of time. Of course. it doesn’t need to erase everything. The basic configuration information of the supported TACs, PLMNs, slices, etc., required for NG interfacing along its orbit will remain.

Proposal 1: AMF initiates the removal of NG interface using existing NGAP AMF Configuration Update procedure when a NG-connected satellite on-board gNB leaves its service area. No need to introduce new procedures (e.g. NG removal, NG suspend/resume).

However, the existing AMF Configuration Update procedure doesn’t seem to be re-used as it is because the removal of SCTP association from AMF was not originally designed to tear down the entire associations and NG interface. This is why, no clear relationship was captured between the removal of TNLA and responding back to AMF [5]:

### 8.7.3 AMF Configuration Update #### 8.7.3.1 General The purpose of the AMF Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling. #### 8.7.3.2 Successful Operation Figure 8.7.3.2-1: AMF configuration update: successful operation The AMF initiates the procedure by sending an AMF CONFIGURATION UPDATE message including the appropriate updated configuration data to the NG-RAN node. The NG-RAN node responds with an AMF CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. Unless stated otherwise, if an information element is not included in the AMF CONFIGURATION UPDATE message, the NG-RAN node shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data. //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////// If the AMF CONFIGURATION UPDATE message includes the AMF TNL Association to Remove List IE, the NG-RAN node shall, if supported, initiate removal of the TNL association(s) indicated by AMF TNL endpoint(s) and NG-RAN node TNL endpoint(s) if the TNL Association Transport Layer Address NG-RAN IE is present, or the TNL association(s) indicated by AMF TNL endpoint(s) if the TNL Association Transport Layer Address NG-RAN IE is absent: - if the received AMF TNL Association Address IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s). - if the received TNL Association Transport Layer Address NG-RAN IE includes the Port Number IE, the NG-RAN node TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN node TNL endpoints correspond to all NG-RAN node TNL endpoints identified by the Endpoint IP Address IE and any port number(s). //////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////

At least one TNLA used for NG-C non-UE associated signalling shall be kept alive in order to respond back to AMF by sending the AMF CONFIGURATION UPDATE ACKNOLWEDGE message. Moreover, given that the NG interface is tore down, the satellite on-board gNB needs to initiate the release of allocated resources related to this NG interface (e.g. releasing the served UEs into IDLE state, or initiating HO for CONNECTED UEs). Therefore, the procedural description needs an update to make sure the above aspects.

Proposal 2: Update the description of the NGAP AMF Configuration Update procedure so that a satellite on-board gNB knowing the NG interface termination can initiate the removal of TNLA(s) after responding back to AMF, and initiate the release of allocated resources related to this NG interface (e.g. releasing the served UEs into IDLE state, or initiating HO for CONNECTED UEs)

The corresponding TP is proposed in [4]. The draft LS to SA2 to inform the RAN3 decision is also provided in Annex.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: AMF initiates the removal of NG interface using existing NGAP AMF Configuration Update procedure when a NG-connected satellite on-board gNB leaves its service area. No need to introduce new procedures (e.g. NG removal, NG suspend/resume).

Proposal 2: Update the description of the NGAP AMF Configuration Update procedure so that a satellite on-board gNB knowing the NG interface termination can initiate the removal of TNLA(s) after responding back to AMF, and initiate the release of allocated resources related to this NG interface (e.g. releasing the served UEs into IDLE state, or initiating HO for CONNECTED UEs)

The corresponding TP is proposed in [4].

The draft LS to SA2 to inform the RAN3 decision is also provided in Annex.

# Reference

[1] R3-243017/S2-2405600 LS on Support of Regenerative-based Satellite Access, SA2

[2] R3-243954, Reply LS on Support of Regenerative-based Satellite Access, RAN3

[3] TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

[4] R3-244480, “(TP for NR_NTN_Ph3 TS 38.413)”, LG Electronics Inc.

[5] TS 38.413, NG-RAN; NG Application Protocol (NGAP), v18.2.0

[6] R3-243355, “(TP for TS 38.300 and TS 38.413) Discussion on the support of Regenerative payload”, Nokia, Nokia Shanghai Bell

# Annex: [DRAFT] LS to SA2

Title:	[Draft] RAN3 feedbacks on the support of NG connection management of the regenerative-based satellite access

Response to:	R3-243017/S2-2405600 LS on Support of Regenerative-based Satellite Access

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	LG Electronics Inc. [To be RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name: 	Jaemin Han

E-mail Address: 	jaeminh.han@lge.com

Send any reply LS to:           3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

Regarding the question from SA2 LS (R3-243017/S2-2405600) on the NG/S1 connection management:

- A procedure to handle the N2 and S1 connections when the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) should be supported. Options e.g. disconnecting/suspending/performing configuration update of the N2/S1 connections are considered in SA2. It is up to RAN3 to determine the final option about whether to reuse existing or new mechanisms/procedures.

RAN3 discussed the matter and concluded that the AMF can initiate the removal of the NG interface using existing NGAP AMF Configuration Update procedure when the NG-connected satellite on-board gNB leaves its service area, and thus there is no need to introduce a new procedure.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take the above information into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN3 Meeting #125bis 	October 14th – 18th, 2024 			Hefei, China

TSG RAN3 Meeting #126		November 18th – 22nd, 2024 			Orlando, Florida, USA