3GPP TSG-RAN WG3 Meeting #125	R3-244481

Maastricht, NL, 19 -23 Aug, 2024

Title:	SON for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei

Document for:	discussion

1 Introduction

The following progress were made at RAN3 #124 meeting for SON for intra-NTN mobility.

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

In this contribution, we continue on this topic and present our further proposals.

2 Discussion

2.1 MRO for intra-NTN mobility

As defined in TS 38.300, NTN supports the following three additional trigger conditions upon which UE may execute CHO to a candidate cell:

- The RRM measurement-based event A4;

- A time-based trigger condition;

- A location-based trigger condition.

The RRC measurement trigger conditions are defined as:

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell; CondEvent A4: Conditional reconfiguration candidate becomes better than absolute threshold where condEventA4 can also be used for current PSCell (i.e., in case it is configured as candidate PSCell for CondEvent A4 evaluation) for CHO with candidate SCG(s) case; CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

For location-based CHO, the handover will be triggered according to the following rules:

CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2; CondEvent D2: Distance between UE and a moving reference location determined based on referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

In short, if the distance between UE and the reference point of the serving cell is larger than distanceThreshFromReference1, and such distance of a candidate cell is shorter than distanceThreshFromReference2, the CHO will be triggered.

Similarly, for time-based condition, CHO will be triggered if the time measured at UE falls into an interval, i.e.,

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

Based on the agreement that take the failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point, we further demonstrate the failure cases, the root causes, and the useful information to tackle the failure.

Time-based trigger condition:

Figure 1 shows the possible occasions that RLF/HOF may occur after the time-based trigger condition is configured to the UE.

Figure 1: Failure scenarios in time-based condition

- Case 1: RLF happens in the source cell before T1 is triggered. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after T1 is triggered and before the duration expires. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too late handover. Case 2 and case 3 could be either a too early handover or a to wrong cell handover depending on the reestablishment cell. In order for the network to identify the failure types and perform root cause analysis, the following additional information needs to be reported in the UE RLF report:

- The measured UTC time when CHO is  executed.

- The measured UTC time when RLF is occurred.

For case 1, it may be regarded as a kind of coverage hole of the NTN coverage. While for case 2 and case 3, with above information, the network may optimize the T1 threshold and duration. Especially, for moving cell case, the UE’s t1-threshould and the duration may be finely tuned by considering the UE location.

Proposal 1: The measured UTC time when CHO is executed and when RLF is occurred shall be reported in the UE RLF report.

Location based trigger condition:

Figure 2 shows the possible RLF/HOF happening places after the location-based trigger condition is configured to the UE.

Figure 2: Failure scenarios in location-based condition

- Case 1: RLF happens in the source cell before D1/D2 is triggered. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after D1/D2 is triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO. Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the reestablishment cell. In order for the network to identify the failure types and perform root cause analysis, the following additional information related to distance measurement needs to be reported in the UE RLF report:

- The measured distanceFromReference1 and distanceFromReference2 when CHO condition D1/D2 is satisfied.

- The indicators that indicate whether distanceThreshFromReference1 and distanceThreshFromReference2 are satisfied, respectively.

- The time duration between the two thresholds fulfilled. This will imply the extent how the thresholds are set improperly.

With above information, the network is able to determine the failure causes of the location-based CHO and further to tune either one or both distance thresholds in D1/D2.

Proposal 2: The measured distanceFromReference1 and distanceFromReference2 when CHO condition D1/D2 is satisfied, and the indicators as well as the duration of two distance conditions are reached shall be reported in the UE RLF report.

RRM-based trigger condition:

The analysis of RRM-based CHO can be carried out in a similar way as above.

- Case 1: RLF happens in the source cell before A3/A4/A5 is triggered. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after A3/A4/A5 is triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO. Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the reestablishment cell. In order for the network to identify the failure types and perform root cause analysis, the following RRM-related information needs to be reported in the UE RLF report:

- RRM measurement results when A3/A4/A5 is satisfied (already contained in the RLF reports).

- Specifically, for A5, the indicators that indicate whether threshold1 and threshold2 are satisfied, respectively.

- Proposal 3: The indicators that indicate whether threshold1 and threshold2 for A5 shall be included in RLF report for optimization of RRM-based CHO.

Joint trigger condition:

Figure 3: Failure scenarios in joint condition CHO

As illustrate in Figure 3, the failure scenarios for joint trigger condition could be:

- Case 1: RLF happens in the source cell when only 1 condition is satisfied, or neither is satisfied. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after both conditions are triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO since at least one condition is set too hard to be reached. Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the reestablishment cell, which are the result of that some conditions are not out-of-time or incongruous, leading to the exclusion of the optimal cell. In order for the network to identify the failure types and perform root cause analysis, the following conditional information needs to be reported in the UE RLF report:

Both

- The time duration between the two conditions fulfilled (for case 2 and case3). This will imply the extent how the thresholds are set improperly.

- Time+RRM

- The measured UTC time when RRM based conditions are fulfilled.

- The RRM measurement results when the time condition is fulfilled.

- Location+RRM

- The RRM measurement results when the location conditions are fulfilled.

- The measured distance (with serving cell and neighbour cell) when RRM based conditions are fulfilled.

- Proposal 4: For joint-condition CHO, at least the following information shall be reported in RLF report.

  - the time duration between the serving cell meeting the conditions and the CHO candidate cell meeting the conditions;

  - measured distance in D1/D2 when CHO conditions are satisfied;

  - measured time in T1 when CHO conditions are satisfied;

  - RRM measurement results A3/A4/A5 when CHO conditions are satisfied.

2.2 MDT enhancements for intra-NTN mobility

Logged MDT is mainly applicable for UEs in RRC_IDLE/INACTIVE, whose mobility management in NTN is also worthy to be addressed. The UE measurement in idle/inactive mode in NTN follows the network configurations, such as SMTCs, and assistance information provided in SIB19. Specifically, in NTN scenarios, The UE can perform time-based and location-based measurements on neighbour cells. For example, the time-based measurement proceeds independently with the location/quality conditions, i.e.,

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

Hence, there is no need to optimize the configuration of the time-base measurement. However, the location-based measurement conducts the following rules:

If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements; - else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-movin g system and has obtained its location information: - if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - else, the UE shall perform intra-frequency measurements; - Else, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements.

It can be shortly summarized as Fig.3: If the signal power/quality of the serving cell falls lower than the thresholds, the measurement will be directly triggered; Else if the distance is larger than the threshold, the measurement will be triggered.

Fig.3 A diagram of location-based measurement for UE at RRC_IDLE/RRC_INACTIVE

From function point of view, it is beneficial for the network to configure the UE to perform logging measurements in certain location trigger events.

Furthermore, to assist the network to optimize the measurement conditions through UE’s logged MDT, RAN3 should also discuss the contents of the UE’s logged MDT records. For example, the UE may at least additionally record the trigger conditions (power/quality, or distance), and the measured distance.

- Proposal 5: RAN3 to discuss the enhancement on location based event trigger for logged MDT, and the following additional record contents:

- the current trigger conditions (power/quality & distance)

- the measured distance

3 Conclusion

This document proposes:

Proposal 1: The measured UTC time when CHO is executed and when RLF is occurred shall be reported in the UE RLF report.

Proposal 2: The measured distanceFromReference1 and distanceFromReference2 when CHO condition D1/D2 is satisfied, and the indicators as well as the duration of two distance conditions are reached shall be reported in the UE RLF report.

- Proposal 3: The indicators that indicate whether threshold1 and threshold2 for A5 shall be included in RLF report for optimization of RRM-based CHO.

- Proposal 4: For joint-condition CHO, at least the following information shall be reported in RLF report.

- the time duration between the serving cell meeting the conditions and the CHO candidate cell meeting the conditions;

- measured distance in D1/D2 when CHO conditions are satisfied;

- measured time in T1 when CHO conditions are satisfied;

- RRM measurement results A3/A4/A5 when CHO conditions are satisfied.

- Pros pal 5: RAN3 to discuss the enhancement on location based event trigger for logged MDT, and the following additional record contents:

- the current trigger conditions (power/quality & distance)

- the measured distance