3GPP TSG-RAN WG3 Meeting #125	R3-244482

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Discussion on the PDCP anchor based solution

Agenda Item:	13.2

Source:	Huawei, Ericsson, CATT, China Telecom, CMCC，Samsung

Document for:	Discussion

1	Introduction

In the discussion on inter-CU LTM at last meeting, one solution in [1] which is characterized by keeping the PDCP layer at the anchor gNB during inter-gNB LTM was proposed to be a complementary approach of the Xn handover signalling based solution.

This contribution provides a detailed analysis on the anchor gNB based inter-CU LTM on the aspects of feasibility, standard impact, performance, and the backwards capability to help the group make a decision.

2	Discussion

2.1	Reusing intra-CU LTM

In Rel-18, necessary function enhancements have been introduced to F1AP to support the intra-CU LTM. The Rel-19 WID [2] includes a note indicating that intra-CU LTM procedures will be considered when designing the inter-CU LTM signaling.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

According to the illustration in [1], the PDCP anchor based solution behaves as a completely new method for inter-CU LTM, which has nothing related to the Rel-18 intra-CU LTM design over F1AP.

Observation 1: The PDCP anchor based solution is outside of the scope of R19 WID.

2.2	Architecture impact

In section 6.1.1 Overall Architecture of NG-RAN in TS 38.401, it states that:

A gNB may consist of a gNB-CU and one or more gNB-DU(s). A gNB-CU and a gNB-DU is connected via F1 interface. One gNB-DU is connected to only one gNB-CU.

In section 6.1.2 Overall architecture for separation of gNB-CU-CP and gNB-CU-UP in TS 38.401, it shows that:

6.1.2 Overall architecture for separation of gNB-CU-CP and gNB-CU-UP The overall architecture for separation of gNB-CU-CP and gNB-CU-UP is depicted in Figure 6.1.2-1. NOTE 0: NG-RAN could also consist of a set of ng-eNBs, an ng-eNB may consist of an ng-eNB-CU-CP, one or more ng-eNB-CU-UP(s), and one or more ng-eNB-DU(s). An ng-eNB-CU-CP and an ng-eNB-CU-UP is connected via the E1 interface. An ng-eNB-DU is connected to an ng-eNB-CU-CP via the W1-C interface, and to an ng-eNB-CU-UP via the W1-U interface. The general principle described in this clause also applies to ng-eNB and its corresponding E1 and W1 interfaces, if not explicitly specified otherwise. Figure 6.1.2-1. Overall architecture for separation of gNB-CU-CP and gNB-CU-UP - A gNB may consist of a gNB-CU-CP, multiple gNB-CU-UPs and multiple gNB-DUs; - The gNB-CU-CP is connected to the gNB-DU through the F1-C interface; - The gNB-CU-UP is connected to the gNB-DU through the F1-U interface; - The gNB-CU-UP is connected to the gNB-CU-CP through the E1 interface; - One gNB-DU is connected to only one gNB-CU-CP; - One gNB-CU-UP is connected to only one gNB-CU-CP; NOTE 1: For resiliency, a gNB-DU and/or a gNB-CU-UP may be connected to multiple gNB-CU-CPs by appropriate implementation. - One gNB-DU can be connected to multiple gNB-CU-UPs under the control of the same gNB-CU-CP; - One gNB-CU-UP can be connected to multiple DUs under the control of the same gNB-CU-CP;

With the combination of the statements in section  6.1.1 and section 6.1.2, it can be concluded that one gNB-DU is connected to only one gNB-CU. And the connections between gNB-DUs and the gNB-CU-CPs are under the control of the same gNB-CU-CP. Such basic architecture principles are supported by the interface management related procedures in F1AP and E1AP, including F1 Setup, F1 Removal, E1 Setup, and E1 Release, etc.

And especially on F1 interface, the gNB-DU shall report its served cells list with essential served cell information of each cell to the gNB-CU that it is connecting to. Which is the essential and mandatory step to enable the gNB-CU to manage and control the radio resources of the cells served by the gNB-DU. On E1 interface, the gNB-CU-UP reports the NR CGI Support List to the gNB-CU-CP on a per PLMN level.

The PDCP anchor based solution will result in the situation that a gNB-CU manage the radio resources of the gNB-DU under the control of another gNB-CU for a UE, which could be potentially indefinite. LTM allows subsequent mobility, and during LTM, a UE may remain in one cell for an extended period. All the UE associated F1AP and/or NGAP procedures impacting the gNB-DU may have to be supported between the gNB-DU of the current serving cell and the another gNB-CU hosting RRC (anchor) and mirrored onto XnAP, as there is no direct F1-C connection between them. Apart from specification redundancies, the byproduct could be as if having a direct UE-associated F1-C interface connection between CU and DU of different gNBs, i.e. allowing a gNB-DU connected to two gNB-CUs for a UE lying in different gNBs.

We believe that such abovementioned facts are a violation to the current NG-RAN architecture. This is illustrated in Figure 1 below.

Figure 1: Architecture impact from the PDCP anchor solution

It should be noted that neither the dual connectivity function nor the SDT violates the current NG-RAN architecture. As for dual connectivity, both the mast node and the secondary node will host their respective RRC function which means that the secondary node has full control on all the bearers going through it by SN initiated SN change/release procedures. Due to their respective RRC control, there was no need to mirror the UE-assocaited F1AP handling across MN-CU and SN-DU (or vice versa).

For SDT, the data transmission on the gNB other than the UE’s serving gNB is only momentary. There are no any long term connection for the UE between the two network nodes.

Observation 2: The PDCP anchor based solution violates the basic NG-RAN architecture.

2.3	Standard impact

The PDCP anchor based solution described in [1] may have huge standard impacts to support it.

Firstly, It cannot reuse the current Xn handover signallings for LTM preparation as the key distinction of it is not transferring the UE Context Information to the target node which is mandatory in current Xn Handover Request message. It cannot reuse the current SN addition signallings as for example, it does not derive the security keys for the SN node which is mandatory in the S-Node Addition Request message.

Secondly, anchor relocation cannot be avoided and the anchor relocation function which is completely new is anyway essential for the PDCP anchor based solution to enable the serving node to relocate the UE context to a new node due to many reasons, like overlong user plane latency, transport layer restrictions, etc.

Thirdly, new signallings/procedures are foreseen to support the interaction between the UE’s anchor gNB-CU and the candidate gNB-DU in another node to support the LTM configuration modification, LTM candidate cell cancellation functions, etc.

Fourthly, in order for the anchor gNB-CU to communicate with the target gNB-DU after the UE’s cell switch, all the UE associated F1AP procedures and all the UE associated NGAP procedures impacting on gNB-DU have to be replicated into XnAP. This may lead to not only functional redundant and huge standard workload.

Fifthly, new Xn procedures have to be introduced to forward the NAS messages and NRPPa messages between the anchor gNB-CU and the target gNB-DU for the UE. This would involve another set of new functions and likely another round of lengthy discussions.

Observation 3: Supporting the PDCP anchor based solution would have a significant impact on the standard and require a substantial workload.

2.4	Performance impact

The PDCP anchor based solution will remain the connected mode UE in a strange connecting state, which increases the transmission delay on both control plan and user plane.

The extra delay on control plane comes from the RRC message forwarding back and forth between the anchor gNB-CU and the controller gNB-CU of the target gNB-DU. For example, for the gNB-DU initiated UE Context Modification procedure, the CellGroupConfig is send by the target gNB-DU to its controller gNB-CU firstly, and then, forwarded to the anchor gNB-CU-CP to generate the final RRCReconfiguration message. After that, the RRCReconfiguration message will be sent back to the controller gNB-CU, to the target gNB-DU, then to the UE finally.

The extra delay on user plane comes from the transport layer topology between the anchor gNB-CU-UP and the UE’s latest serving gNB-DU located inside another gNB.  Normally, if there is no direct route configured in the TNL router and switch between the two nodes, the extra user plane delay equals to the Xn-U latency times the number of routers that the user data is going through.

Anyhow, the extra delay on control plane and user plan introduced by the PDCP anchor based solution will lower down the user experience and performance. And it may also impact on the support of many latency sensitive URLLC services, like 5QI 86, v2x messages which having a 5ms UE-UPF packet delay budget as specified in section 5.7.4 in TS 23.501.

Observation 4: The PDCP anchor based solution will introduce extra delay on both control plan and user plane on transmissions from/to the UE after cell switch. Which may lower down the user experience and performance and impact on the support of latency sensitive URLLC services.

2.5	Backwards compatibility after cell switch

Impact on high layer RRM functions:

The PDCP anchor based solution will impact negatively on the high layer RRM functions which are placed at the gNB-CU, like MRO, MLB, overload control, energy saving , etc.

The main reason is that the RRC protocol and the corresponding high layer RRM functions for the UE are placed at the anchor gNB-CU, while the target gNB-DU hosting the real radio resources of the target cell serving the UE is controlled and managed by its controller gNB-CU. But the controller gNB-CU cannot control anything allocated to the UE in the target gNB-DU that is migrated into the target cell by the PDCP anchor based LTM solution.

For example, in section 8.2.7 in TS 38.473, it specifies that the F1-C interface support gNB-DU overload control function by the gNB-DU Status Indication function. Once received the overload indication from the gNB-DU, the gNB-CU may react to reduce the load in the gNB-DU, by avoiding handover more UEs into the overloaded gNB-DU, handover out some UEs to its neighbours, etc. But it cannot do anything to the UEs which is controlled by the other gNB-CU.

Due to such distinction, it may also disable the gNB-CU to perform its load balancing strategies, cell switch off strategies, etc.

Particularly, in MRO,  normally the RLF report will be delivered to the UE’s last serving node indicated by the NR CGI in the report for root analysis. In the scenario here, the RLF report will be forwarded to the controller gNB-CU rather than the anchor gNB-CU. This may mislead the MRO algorithm in the controller gNB-CU, because it is the anchor gNB-CU who made the handover decision for the UE.

Observation 5: The PDCP anchor based solution will disable the controller gNB-CU to perform high layer RRM functions towards a UE out of control , like MRO, MLB, overload control, energy saving , etc.

Impact on OAM NRM:

In TS 28.541, SA5 defines the gNB NRM (network resource model) for network resource management purpose. And SA5 also defines the IOC (Information Object Class) for each manageable object.

Section 4.2.1.1 TS 28.541 The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below. - Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions. - 2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction. - 3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction. […] Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios […] 4.3.32 NRCellRelation 4.3.32.1 Definition This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance. The source cell can be a NRCellCU instance. This is the case for an Intra-NR neighbour cell relation. The source cell can be a EUtranGenericCell instance. This is the case for Inter-LTE-NR neighbour cell relation, from E-UTRAN to NR. See 3GPP TS 28.658 [19]. Neighbour cell relations are unidirectional.

According to above quotation, we may interpret that a gNB contains a gNB-CU-CP represented by the IOC of the GNBCUCPFunction, which further contains a list of IOCs of NRCellCU representing the served cells controlled by the gNB-CU-CP. Furthermore, one IOC of NRCellCU representing a logical NR cell contains a list of IOCs of NRCellRelation. The IOC of NRCellReleation represents a neighbour cell relation from a source cell to a target cell. And the source cell can be a NRCellCU instance.

Based on above interpretation, we may conclude that one gNB-CU-CP can only manage the neighbour cell relations of which the source cell is under the control of it. While the PDCP anchor based solution may need one gNB-CU-CP to manage and maintain the cell relationship for cells that are not controlled by the gNB-CU-CP which seems contradicting to the current NRM in SA5 specification.

Observation 6:  SA5 needs to be involved to evaluate the impact on the current NRM of the gNB for the PDCP anchor based solution, for example, managing the Cell Relations of a cell by a gNB-CU not controlling the cell.

Impact on functions in core network:

Currently, the UE’s User Location Information is reported to the AMF in many NGAP messages for connected mode UEs to enable UE location and area of interest based functions, services, like paging, registration area management, charging, Lawful Interception, positioning, etc…

If there is an anchor gNB-CU for the UE, and the serving cell is not controlled by the anchor gNB-CU, theoretically the anchor gNB-CU will report the NR CGI and TAI of the real serving cell in the ULI to the AMF. Basically, this behaviour leads to a fact that one gNB indicates a NR cell which is not controlled by itself and not reported in NG setup to the AMF. Evaluation should be done in SA2, CT1 and CT4 if any impact on core network functions when the AMF receives a NR CGI which does not belong to the reporting gNB.

One should note that the impact analysis on legacy functions in this section are preliminary and not exhaustive. More impacts may be discovered along with the evaluation to the PDCP anchor based inter-CU LTM solution.

Observation 7:  Evaluation should be done in SA2, CT1 and CT4 to prove if any impact on core network functions when the AMF receives a NR CGI in the ULI which does not belong to the reporting gNB.

2.6	Summary

In this document, we make a deep and wide analysis on the architecture impact, standard impact, latency performance, and backwards capability coming from the PDCP anchor based inter-CU LTM. Based on the above observations, there seems no any motivation and benefit to adopt the PDCP anchor based inter-CU LTM as a complementary solution in Rel-19.

Therefore, we propose:

Proposal: Given the extensive workload across multiple working groups and the significant changes to the existing architecture and mechanisms, RAN3 should not proceed with the PDCP anchor based inter-CU LTM solution.

3	Conclusion

In this contribution, we provide a deep and wide analysis on the PDCP anchor based inter-CU LTM solution, and have the following observations and proposal:

Observation 1: The PDCP anchor based solution is outside of the scope of R19 WID.

Observation 2: The PDCP anchor based solution violates the basic NG-RAN architecture.

Observation 3: Supporting the PDCP anchor based solution would have a significant impact on the standard and require a substantial workload.

Observation 4: The PDCP anchor based solution will introduce extra delay on both control plan and user plane on transmissions from/to the UE after cell switch. Which may lower down the user experience and performance and impact on the support of latency sensitive URLLC services.

Observation 5: The PDCP anchor based solution will disable the controller gNB-CU to perform high layer RRM functions towards a UE out of control , like MRO, MLB, overload control, energy saving , etc.

Observation 6:  SA5 needs to be involved to evaluate the impact on the current NRM of the gNB for the PDCP anchor based solution, for example, managing the Cell Relations of a cell by a gNB-CU not controlling the cell.

Observation 7:  Evaluation should be done in SA2, CT1 and CT4 to prove if any impact on core network functions when the AMF receives a NR CGI in the ULI which does not belong to the reporting gNB.

Proposal: Given the extensive workload across multiple working groups and the significant changes to the existing architecture and mechanisms, RAN3 should not proceed with the PDCP anchor based inter-CU LTM solution.

4	Reference

[1] R3-243522 Support for inter-CU LTM

[2] RP-241515 Revised Work Item: NR mobility enhancements Phase 4