3GPP TSG-RAN WG3 Meeting #125	R3-244483

Maastricht, NL, 19 - 23 Aug, 2024

Agenda Item:	13.2

Source:	Huawei

Title:	Discussion on Inter-CU LTM procedure

Document for:	Discussion

Introduction

In RAN3 #123bis and RAN #124 meetings, RAN3 discussed Rel-19 inter-CU LTM and made some agreements. In this contribution, we continue to discuss the basic procedures of inter-CU LTM.

Discussion

In previous RAN3 meetings, RAN3 has made the following agreements:

RAN3 #123bis: Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution RAN3 #124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

Some of them may have stage 2 or stage 3 specifications impacts. In order to capture all these agreements, it is proposed to  agree on the TPs for TS 38.000, and TS 38.423.

Proposal 1: RAN3 to agree on the related TPs to capture the earlier agreements.

  - LTM Preparation

General

In RAN3 #123bis and RAN3 #124 meetings, RAN3 agreed to re-use existing Xn Handover Request and Handover Request ACK for inter-CU LTM preparation, and it is the source gNB-CU that initiates the inter-CU LTM preparation procedure.

RAN3 #123bis Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. RAN3 #124 Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

The existing Xn Handover Request procedure is initiated per candidate cell, instead of per UE. In case of inter-gNB-CU LTM, the source gNB-CU sends a Handover Request message per LTM candidate cell to the candidate gNB-CU (candidate gNB), which triggers a F1 UE Context Setup Request message to the candidate gNB-DU at the target side. After receiving the UE Context Setup Response message from the candidate gNB-DU, the candidate gNB-CU responds with a Handover Request Acknowledge message with the LTM configuration of the candidate cell to the source gNB-CU.

Proposal 2: RAN3 confirms that the handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

L1 measurement and report configuration

In Rel-18 intra-CU LTM, L1 measurement and report is based on SSB. The CU utilizes the SSB configuration obtained in F1 SETUP to produce the CSI resource configuration and send it to the source DU to generate the CSI report configuration.

In Rel-19 inter-CU LTM, to support SSB based L1 measurement and reporting, similarly, the source gNB is able to obtain the SSB configuration of candidate cells from neighbour gNBs via Xn setup. It can then utilize the obtained SSB configuration to produce the CSI resource configuration and send it to the source gNB-DU to generate the CSI report configuration. The processing on F1 can re-use the UE Context Modification Request procedure.

Proposal 3: The source gNB-CU utilizes the candidate cells’ SSB configuration obtained from neighbour gNBs during Xn setup to produce the CSI resource configuration.

In case of subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs, so the other candidate gNB-CUs can request the corresponding candidate gNB-DU to generate the CSI report configurations.

Proposal 4: To support subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs.

The SSB configuration exchanged in Xn Setup does not include the SSB transmit power information, which is needed in the UE LTM configuration. Therefore, in Rel-18, the SSB Time/Frequency Configuration is included in the UE Context Setup Response message from the candidate DU to the CU.

In Rel-19, similarly, it’s better to let the candidate gNB-CU to respond the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 5: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

  - Early Synchronization

In RAN2 #125bis meeting, RAN2 agreed that early DL and UL sync are supported for inter-CU LTM, and PDCCH ordered early RACH without RAR method is the baseline.

Agreements on early sync phase: - Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. - PDCCH ordered early RACH is supported for inter-CU LTM. - For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

In PDCCH ordered early RACH without RAR method, to avoid RACH resource conflict, the early RACH resources of one candidate cell should be different for different source gNB-DUs. This is the reason why RAN3 introduced the source gNB-DUs List in Early UL Sync Configuration in Rel-18. The gNB-DUs List ensures that the candidate gNB-DUs can allocate non-overlapping RACH resources for different source gNB-DUs.

In Rel-19 inter-CU LTM, two gNB-DUs of difference gNB-CUs may have the same gNB-DU ID. Therefore, to avoid early RACH resource conflict, the pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then the candidate gNB-DUs during handover preparation.

Proposal 6: The pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then to the candidate gNB-DUs during handover preparation, in order to avoid early RACH resource conflict.

After the candidate gNB-DU receives a preamble for early TA acquisition, it transfers the TA value to the source gNB-DU. In Rel-18, on F1 interface, new messages (e.g. DU-CU/CU-DU TA Information Transfer) are introduced to transfer the TA information from candidate gNB-DU to source gNB-DU.

In Rel-19 inter-CU LTM, in order to transfer the TA information between the candidate gNB and the source gNB, RAN3 agreed to introduce a new procedure on Xn.

RAN3 #124 Introduce a new procedure on Xn to transfer the TA information.

Since when the candidate gNB-DU does not know the UE identity when receiving the preamble, the TA information transfer procedure should be a new non-UE associated class 2 procedure, namely TA Information Transfer message. In this message, source gNB-DU ID is needed to let the source gNB-CU know where to forward this TA information.

Proposal 7: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message to transfer the TA information from the candidate gNB-CU to the source gNB-CU including the Source gNB-DU ID.

Proposal 8: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, to let the candidate gNB-CU know where to forward the TA information, a source gNB ID is need in the corresponding DU-CU TA Information Transfer message.

Proposal 9: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

  - LTM Completion

After receiving the ACCESS SUCCESS message from the target gNB-DU, the target gNB-CU initiates the path switch procedure to CN via the existing path switch message and CN can start to forward the DL data to the target gNB-CU accordingly.

Proposal 10: After receiving the ACCESS SUCCESS message from the target gNB-DU, the target gNB-CU initiates the path switch procedure to CN.

  - LTM Cancel

In RAN3 #124 meeting, RAN3 agreed that the source gNB can send Handover Cancel message to candidate gNB to release LTM candidate cells. In addition, the candidate gNB can initiate cancellation of configured LTM candidate cells, but the details are FFS.

RAN3 #124 The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS

In the LTM candidate cell cancel initiated by candidate gNB case, the candidate gNB needs to notify the serving gNB and other involving candidate gNBs. However, in subsequent inter-CU LTM, the serving gNB of the UE will change. How the candidate gNB knows which candidate gNB is the current serving gNB of the UE needs further study.

The straightforward thinking is that the candidate gNB always contacts the initial source gNB, then the initial source gNB forwards the information of candidate cell to be cancelled to the current serving gNB of the UE. This makes the initial source gNB behave like an anchor gNB of the UE and requires the initial source gNB should be always informed by the new serving gNB after each LTM cell switch.

Another option could be that after each LTM cell switch, the target gNB (the new serving gNB) notifies all candidate gNBs about the serving gNB change. E.g., the target gNB sending a Xn Handover Success message to all candidate gNBs. In this procedure, the UE associated XnAP connections between this new serving gNB and other candidate gNBs are established, so when a candidate gNB decides to cancel candidate cells, it can directly contact with this new serving gNB. Then the new serving gNB can forward the information to other involving candidate gNBs.

Proposal 11: RAN3 to discuss how to support candidate gNB initiated LTM cancel with the two options:

Opt1: the candidate gNB always contacts the initial source gNB first which then forwards the candidate cells to be cancelled to the current serving gNB and other involving candidate gNBs.

Opt2: the candidate gNB contacts the current serving gNB which then forwards the candidate cells to be cancelled to other involving candidate gNBs.

  - LTM configuration sync up among candidate gNBs

In addition, to support subsequent inter-CU LTM, some LTM configurations, e.g., early sync configuration, configuration ID, and data forwarding addresses, etc may need to sync up among candidate gNBs.

Taking the TCI states as an example, after the source gNB-CU collects the TCI state configuration of candidate cells in Handover Request Ack message, it should send the TCI state configuration to other candidate gNB-CUs.

In general, there are three options to sync up the TCI state configuration among candidate gNB-CUs. Option 1 is to sync up just after the initial handover preparation. Option 2 is that the source gNB sends the configuration to the target gNB once the LTM is initiated. Option 3 is when receiving the successful access notification from the target gNB. The three options are illustrated in Figure 1.

Figure 1: TCI state configuration transfer for subsequent inter-CU LTM

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Basically, the three options above can be mapped to the three cases of NOTE 2. However, the three options may have different Xn signalling requirements. For example, option 1 may need to introduce a new class 2 procedure between gNB like Handover Modification proposed by some companies at previous meetings. Option 2 may also need new Xn signalling or reuse the Xn LTM Cell Notification message. Option 3 may have similar signalling requirements as option 1 and also require the target gNB to inform the source about the successful access to the target cell.

Proposal 12: RAN3 to discuss how the LTM configurations are synced up among candidate gNBs in case of subsequent LTM.

  - UE associated Xn signalling connection setup between candidate gNBs

During LTM, the LTM Cell Switch Notification from the source gNB-CU to the target gNB-CU can be successfully transmitted for the first time cell switch, because the source gNB-CU and the target gNB-CU already have the NG-RAN node UE XnAP ID of each other.

However, in subsequent inter-CU LTM, it is unclear how the new source gNB sends the LTM Cell Switch Notification to the target gNB and how the target gNB identify the correct UE, as there is no any Xn signalling connection setup procedure between these two gNBs beforehand.

One possible solution is that each candidate gNB allocates UE XnAP ID and send to other candidate gNBs in the preparation step.

Another solution is that after each cell switch, the new serving gNB allocates the source node UE XnAP ID and send it to other candidate gNB-CUs, and then the candidate gNB-CUs reply with the target node UE XnAP ID.

Proposal 13: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

  - LTM modification

In Rel-18, the intra-CU inter-DU LTM supports to update/modify the prepared LTM configuration in the candidate gNB-DUs (step 7 in the figure below) e.g., to update the CSI report configuration due to candidate cell preparation failure in step 3 and 4,.

Figure 2: Intra-CU inter-DU LTM procedure in TS 38.401

In Rel-19, the same scenario exits for inter-CU case. Hence, new Xn messages (e.g., HANDOVER REQUEST MODIFICATION and HANDOVER REQUEST MODIFICATION RESPONSE) need to be introduced. Reusing existing Handover Request procedure may be also feasible if the mandatory UE Context Information IE is marked to be ignored.

Proposal 14: RAN3 to discuss the support of source gNB-CU initiated LTM configuration modification.

Conclusion

The following proposal are made based on the above discussion:

Proposal 1: RAN3 to agree on the TPs for TS 38.000 and TS 38.423 in [2]and [3] to capture the earlier agreements.

LTM Preparation:

Proposal 2: RAN3 confirms that the handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

Proposal 3: The source gNB-CU utilizes the candidate cells’ SSB configuration obtained from neighbour gNBs during Xn setup to produce the CSI resource configuration.

Proposal 4: To support subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs.

Proposal 5: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Early Synchronization:

Proposal 6: The pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then to the candidate gNB-DUs during handover preparation, in order to avoid early RACH resource conflict.

Proposal 7: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message to transfer the TA information from the candidate gNB-CU to the source gNB-CU including the Source gNB-DU ID.

Proposal 8: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 9: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

LTM Completion:

Proposal 10: After receiving the ACCESS SUCCESS message from the target gNB-DU, the target gNB-CU initiates the path switch procedure to CN.

LTM Cancel:

Proposal 11: RAN3 to discuss how to support candidate gNB initiated LTM cancel with the two options:

Opt1: the candidate gNB always contacts the initial source gNB first which then forwards the candidate cells to be cancelled to the current serving gNB and other involving candidate gNBs.

Opt2: the candidate gNB contacts the current serving gNB which then forwards the candidate cells to be cancelled to other involving candidate gNBs.

LTM configuration sync up：

Proposal 12: RAN3 to discuss how the LTM configurations are synced up among candidate gNBs in case of subsequent LTM.

UE associated Xn signalling connection setup:

Proposal 13: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

LTM modification:

Proposal 14: RAN3 to discuss the support of source gNB-CU initiated LTM configuration modification.

The TPs are provided in [2] and [3].

Reference

[1] R3-243107	Discussion on Additional Considerations for Inter-gNB-CU LTM, Nokia.

[2] R3-244484	(TP for LTM BLCR for TS38.300): Inter-CU LTM basic procedure

[3] R3-244485	(TP for LTM BLCR for TS 38.473): F1 impact for Inter-CU LTM