3GPP TSG-RAN WG3 Meeting #125	R3-244489

Maastricht, NL, 19 - 23 Aug, 2024

Agenda Item:	9.2

Source:	Huawei, China Unicom, CMCC

Title:	Further clarification on MIMO with 2TA in LTM

Document for:	Discussion

Introduction

Technically, one cell may support the MIMO with 2TA feature, i.e., the cell may maintain two TA values for a UE, each TA value is associated with a TAG. In order to support the MIMO with 2TA in LTM, in RAN3 #124 meeting, RAN3 agreed a CR [1] for early TA transfer from a candidate DU to the source DU via the CU by adding  the Tag ID Pointer IE in the DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages. The reason for change is shown below:

The LTM can coexist with the feature of MIMO with 2TA. In particular, one cell can have two TA values, each of which is associated with one TAG. In current F1AP signaling, the TA value of the candidate cell is provided from the candidate gNB-DU to the source gNB-DU via gNB-CU. However, the TAG information of TA value is not provided. As a result, the source gNB-DU cannot determine TAG ID of the received TA value, so that it cannot include the correct TA in the LTM cell switch command MAC CE.

After further analysis, we find that this change may be problematic and need further clarification. Because in some cases, the candidate DU may be not able to set a proper value for the Tag ID Point IE.

Discussion

Issue to be clarified:

Basically, the CR [1] agreed at last meeting assumes that in MIMO with 2TA, there is a fixed mapping relationship between the TRPs and the tag IDs. However, there is no such statement on how the RAN node will assign the tag ID to a UE in any specifications. If in an implementation, the Tag ID does not bind to the TRP, it implies that different tag ID pointers for a same TRP may be allocated to different UEs which seems supported by the per UE ServingCellConfig in RRC protocol. In this case, the candidate gNB-DU will not be able to set the Tag ID Pointer IE properly.

Figure 1 illustrates the use case in detail. In figure 1, Cell1 is the source cell for UE1 and UE2. Cell2 is a LTM candidate cell for UE1 and UE2, and Cell2 has two TRPs. Both UE1 and UE2 support MIMO with two TA, but the TAG ID may be different for those two UEs for a TRP. For example, for TRP1 in Cell2, TAG ID 0 is configured to  UE1, and TAG ID 1 is configured to UE2. While the assignment is opposite for TRP2.

Figure 1: Case2 for LTM candidate cell with two TA feature

When the DU2 receives a preamble for early TA acquisition, it does not know how to set the TAG ID Pointer IE, because it cannot identify the UE by the preamble detected.

Proposed solution:

Given that two TRPs usually use different TCI States, the source gNB-DU may deduce the UE’s tag ID from the TCI state configuration of the UE when a TA value is received from the candidate gNB-DU. This is feasible because that the source gNB-DU sends the preamble index and the SSB index in the PDCCH order to a UE to trigger early TA acquisition.

The quoted RRC protocol shows the link between the SSB index and the tag id pointer.

CandidateTCI-State-r18 ::= SEQUENCE { tci-StateId-r18 TCI-StateId, qcl-Type1-r18 LTM-QCL-Info-r18, qcl-Type2-r18 LTM-QCL-Info-r18 OPTIONAL, -- Need R pathlossReferenceRS-Id-r18 PathlossReferenceRS-Id-r17 OPTIONAL, -- Need R Tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL, -- Cond 2TA ul-powerControl-r18 Uplink-powerControlId-r17 OPTIONAL, -- Need R ... } LTM-QCL-Info-r18 ::= SEQUENCE { referenceSignal-r18 CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId }, qcl-Type-r18 ENUMERATED {typeA, typeB, typeC, typeD}, ... }

According to the above analysis,  it concludes that the source gNB-DU may determine the Tag ID pointer without any assistance information from the candidate gNB-DU.

Proposal 1: To remove the TAG ID Pointer IE from the DU-CU/CU-DU TA Information Transfer messages.

Proposal 2: To capture in stage 2 somewhere about how the source DU determines the TAG ID Pointer by the TCI States Configuration.

If above proposals are not agreeable, it is proposed to ask RAN2 to clarify the following basic questions:

- Q1: Is the MIMO with 2 TA supported in the early UL synchronization in Rel-18 LTM?

- Q2: If the answer of Q1 is yes, how the source gNB-DU determine the tag ID associated to the TA value received from the candidate gNB-DU?

Proposal 3: If Proposal 1 and 2 are not agreed, send a LS to RAN2  to ask for clarification on the support of MIMO with 2TA in LTM.

Conclusion

The following proposals are made based on the above discussion:

Proposal 1: To remove the TAG ID Pointer IE from the DU-CU/CU-DU TA Information Transfer messages.

Proposal 2: To capture in stage 2 somewhere about how the source DU determines the TAG ID Pointer by the TCI States Configuration.

Proposal 3: If Proposal 1 and 2 are not agreed, send a LS to RAN2  to ask for clarification on the support of MIMO with 2TA in LTM.

The corresponding CRs to F1AP and TS38.300, and the draft LS are provided in [2] ~ [4].

Reference

[1] R3-243825, Correction on TAG information transfer for LTM.

[2] R3-244490	Correction on MIMO with 2TA in LTM

[3] R3-244491	Correction on MIMO with 2TA in LTM

[4] R3-244492	[DRAFT] LS on MIMO with 2 TA in LTM