3GPP TSG-RAN WG3 Meeting #125	R3-244496

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Emergency call support in barred cells

Agenda Item:	8.3

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In this contribution, we continue to discuss the emergency call support if the cell is considered as barred, based on RAN2 latest progress.

# 2	Discussion

## 2.1	Background

RAN2-125bis meeting agreed a LS on emergency call support for (e)RedCap in barred cells in [1], containing the agreed CRs, copied as follows.

1. Overall Description: In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the above information into account and update RAN3 specifications if needed.

From the attached R18 CRs for RedCap UE and eRedCap UE agreed separately in RAN2 (R2-2402902, R2-2402903) , as discussed in [1], when the cell supports the RedCap UE access, yet bars those UEs due to the 1Rx branch or 2Rx branches or both barrring in the cell, it would be beneficial to introduce an exception for those UEs to have access to the cell to make an emergency call or receive emergency information broadcast when possible.

At RAN2-126 meeting, RAN2 re-discussed it, and agreed a common solution as follows.

Agreements

1	NES (i.e. ingoring MIB barring) will not be considered in our common solution discussion.  FFS if anything specific for NES will need to be done.  If anything needs to be done, it would not be part of the common solution.

2	For Rel-18, we introduce 1 bit that enables EM call for RedCap, eRedCap, and 2Rx XR. One RRC Rel-18 with magic sentence that it is early implementable in Rel-17.  A CR for 38.304 doesn’t need to have the magic sentence.

3	This replaces the previous agreement and we will notify RAN3 verbally via delegates

The 38.331 CR agreed in [2] was copied as follows.

SIB1-v1800-IEs ::= SEQUENCE { <Skip the irrelevance > barringExemptEmergencyCall-r18 ENUMERATED {true} OPTIONAL, -- Cond EM-Barring EM-Barring; The field is optionally present, Need R, in a cell that supports (e)RedCap or XR UEs, otherwise it is absent.. barringExemptEmergencyCall Indicates whether the cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [20]..

It can be observed this exempt bit is applicable to the (e)RedCap UEs, and XR UEs.

Observation 1: RAN2 agreed CRs commonly palpable to e)RedCap UEs, and XR UEs to have emergency call when the cell is considered barred.

## 2.2	Impacts on RAN3 specs

As observed above, the cell can enable such (e)RedCap/2 RX XR UEs to have emergency calls even if barred by the cell. When considering the connected mode handover for target node selection, two options are provided below:

- Option 1: introduce the barring exempt bit over network interfaces

In this option, we can introduce this barring exempt bit over the Xn and F1 interfaces under the served cell information IE, then, the source gNB can select the proper target cell, based on the barring exempt bit exchanged by neighbouring gNBs, and the ARP value of the QoS flow corresponding to the emergency PDU session.

- Option 2: Always allow emergency handover to the target cell no matter whether target cell supports the barring exempt bit.

In this option, the source gNB can always trigger the outgoing handover during the emergency call, without the knowledge of the barring exempt of the target cell. Then the target gNB can always admits the handover, since the emergency call is the critical service to be supported.

Compared with both options, option 1 is more aligned with RAN2 agreements, and is logical in the same way as we handle the RedCap/eRedCap barring bit over network interfaces. Hence option 1 is preferred.

And it should be noted that there is no need to specify the UE types (i.e., (e)RedCap or 2Rx UEs) in RAN3 specs to avoid any further changes when RAN2 reuse it for other type of UEs.

- RAN3 to agree the F1AP and XnAP CRs to introduce the barringExempt bit under the served cell information IE.  And there is no need to specify the UE types (i.e., (e)RedCap or 2Rx UEs) in RAN3 specs to avoid any further changes when RAN2 reuse it for other type of UEs.

For TS 38.470, it is specified that the F1 setup and gNB-DU Configuration Update functions allow to provide information on either RedCap access configuration, or eRedCap access configuration, or both at the gNB-DU. Then there is also need to capture the barring exempt bit.

- RAN3 to agree the stage2 F1AP to capture the barring exempt bit.

Last, it is suggested to provide reply LS to RAN2 for courtesy. And RAN3 can also highlight that the agreed CRs did not specify the UE types (i.e., (e)RedCap or 2Rx UEs) to avoid any further changes for future reason.

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: RAN2 agreed CRs commonly palpable to e)RedCap UEs, and XR UEs to have emergency call when the cell is considered barred.

- RAN3 to agree the F1AP and XnAP CRs to introduce the barringExempt bit under the served cell information IE.  And there is no need to specify the UE types (i.e., (e)RedCap or 2Rx UEs) in RAN3 specs to avoid any further changes when RAN2 reuse it for other type of UEs.

- RAN3 to agree the stage2 F1AP to capture the barring exempt bit.

The CRs can be found in [3-5]. The response LS can be found in [6].

# 4	References

- R2-2403982, LS on emergency call support for (e)RedCap in barred cells, RAN2

- R2-2405957 Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call], Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia, Huawei

- R3-244497	Introduction of Emergency call support in barred cells [RedCap_EM_Call],	Huawei

- R3-244498	Introduction of Emergency call support in barred cells [RedCap_EM_Call],	Huawei

- R3-244499 	Introduction of Emergency call support in barred cells [RedCap_EM_Call],	Huawei

- R3-244500	[draft] reply LS on emergency call support for (e)RedCap in barred cells,	Huawei