3GPP TSG-RAN WG3 Meeting #125	R3-244501

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Discussion on the Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion

# 1	Introduction

The IETF TEAS WG sent a LS to RAN on the Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies in [1], as excerpted as follows.

1. Description As a follow up to the liaison statement [1] that was posted on 2023-10-09, the IETF TEAS WG [2] would like to inform 3GPP about the progress of “A Realization of IETF Network Slices for 5G Networks Using Current IP/MPLS Technologies” [3], an informational document that has completed Working Group Last Call. This document describes a Network Slice realization model for IP/MPLS networks with a focus on the Transport Network fulfilling 5G slicing connectivity service objectives. The realization model reuses many building blocks currently commonly used in service provider networks. In addition to stating that the reader should refer to "TS 23.501: System architecture for the 5G System (5GS)" [4] for a definitive description of 3GPP network architectures, the document also includes an Appendix [5] that provides a brief 5G overview for the reader’s convenience. 2. Actions The TEAS WG would like to request 3GPP to review the document and provide confirmation that the brief 5G overview included in the Appendix is accurate. The TEAS WG would like to request that feedback on this is provided in an LS by the deadline. The TEAS WG also encourages the use of the TEAS WG mailing list [6] for individuals to send comments, raise concerns and seek clarification on the document.

In this contribution, we provide our replies to the incoming LS.

# 2	Discussion

## 2.1	General views on the main sections

As indicated above, the incoming LS asks to review the document, in which it describes a Network Slice realization model for IP/MPLS networks with a focus on the Transport Network fulfilling 5G slicing connectivity service objectives.

In [2], it can be observed at least the following several agenda are related to RAN.

- 3.5. Mapping 5G Network Slices to Transport Network Slices

In this section, it provides multiple options for mapping 5G Network Slices to TN slices, including 1 to N, M to 1, and M to N. it also provides the two examples how the mapping happens between the CU-CP/UP, and AMF/UPF.

- 4.2. IP Hand-off

It provides multiple ways for the mapping between source/destination IP addresses and slice's specific S-NSSAI, including the S-NSSAI to a S-NSSAI to a dedicated IP address for each NF, S-NSSAI to a pool of IP addresses for global TN deployment, S-NSSAI to a subset of bits of an IP address, S-NSSAI to a DSCP value etc.

- 5. QoS Mapping Realization Models

The QoS mapping model to support 5G slicing connectivity uses two layers of QoS: 5G QoS layer, and TN QoS layer. And it describes that the 5G slicing connectivity realization defines two high-level realization models for slicing in the TN domain: a 5QI-unaware model and a 5QI- aware model.  For each model, it describes how the network slice is finally mapped to the TN QoS class.

Basically, the transport layer implementation details are beyond of RAN3 scope. Taking the NG interface as an example, the NG data transport and the NG signalling transport are provided in TS 38.414 and TS 38.412, where the protocol stack is provided, and reference to other specs (e.g., the IETF) are provided without implementation details.

Hence the realization of Network Slices for 5G Networks via IP/MPLS is out of RAN3 scope. Also, RAN3 has no expertise on such transport layers implementation details, and on the implementation via IP/MPLS to realize the network slices.

- Feedback that the transport layer implementation details, and the realization of Network Slices for 5G Networks via IP/MPLS are out of RAN3 scope. RAN3 has no expertise on such transport layers implementation details

## 2.2	The RAN architecture in the Appendix

In Section B.3, it is described that the RAN is composed of three components: BBU, RU, and the antenna, with the following descriptions.

- The Baseband Unit (BBU) provides the interface between the Core Network and the Radio Network. It connects to the Radio Unit and is responsible for the baseband signal processing to packet. - The Radio Unit (RU) is located close to the Antenna and controlled by the BBU. It converts the Baseband signal received from the BBU to a Radio frequency signal. - The Antenna converts the electric signal received from the RU to radio waves.

First there are no such definitions in RAN specs. These RRU/RU/Antenna components are RAN implementation details. Also Figure 36 copied below provides the RAN disaggregation, where the F2 connects between the RU and the DU. However, there is no such logical interface and terminology described in RAN specification.

It can be understood that the components described and F2 interface are just used for the transport layer descriptions and implementations. But it is better to notify IETF TEAS to align with RAN specifications, and it is suggested for IETF to refer to TS 38.300 as RAN architecture, and TS 38.401 as disaggregated RAN architecture. This is also beneificial to avoid IETF TEAS to make further updates to align after RAN makes changes.

- Feedback that for the RRU/RU/Antenna components, there are no such definitions in RAN specs. Also, for the F2 interface connecting between the RU and the DU in Figure 36 there is no such logical interface and terminology described in RAN specification.

- Feedback that it is suggested for IETF to refer to TS 38.300 as RAN architecture, and TS 38.401 as disaggregated RAN architecture, if needed. This is also beneficial to avoid further update after RAN potential changes.

# 3	Conclusion

This document proposes the following observation and proposals:

- Feedback that the transport layer implementation details, and the realization of Network Slices for 5G Networks via IP/MPLS are out of RAN3 scope. RAN3 has no expertise on such transport layers implementation details

- Feedback that for the RRU/RU/Antenna components, there are no such definitions in RAN specs. Also, for the F2 interface connecting between the RU and the DU in Figure 36 there is no such logical interface and terminology described in RAN specification.

- Feedback that it is suggested for IETF to refer to TS 38.300 as RAN architecture, and TS 38.401 as disaggregated RAN architecture, if needed. This is also beneficial to avoid further update after RAN potential changes.

The reply LS can be found in [3].

# 4	References

- R3-244027 LS on "A Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies", IETF Traffic Engineering Architecture and Signaling Working Group (teas)

- https://datatracker.ietf.org/doc/html/draft-ietf-teas-5g-ns-ip-mpls#appendix-B

- R3-244502 [DRAFT] Reply LS on Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies, Huawei