3GPP TSG-RAN WG3 Meeting #125	R3-244504

# Maastricht, NL, 19th – 23th Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1473 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Cause IE in MULTICAST CONTEXT NOTIFICATION REFUSE message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, ZTE, Ericsson,Huawei,Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For an unsuccessful class 1 procedure, a cause should be provided in the response message.However,the Cause IE is missing in the MULTICAST CONTEXT NOTIFICATION REFUSE message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Cause IE in the MULTICAST CONTEXT NOTIFICATION REFUSE message. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the MBS function. This CR is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The node trigging the Multicast Context Notification procedure would not be able to understand why the procedure failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.14.17,9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.2.14.17	MULTICAST CONTEXT NOTIFICATION REFUSE

This message is sent by the gNB-CU to notify the gNB-DU that the execution of the requested functions was not successful.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- MULTICAST CONTEXT NOTIFICATION REFUSE

--

-- **************************************************************

MulticastContextNotificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{MulticastContextNotificationRefuseIEs}},

...

}

MulticastContextNotificationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>