3GPP RAN WG3 Meeting #125	R3-244505

# Maastricht, Netherlands, 19th – 23rd August 2024

Source:	CATT

Title:	Handling GTP-U exceptions for tunnels delivering PDCP PDU

Agenda Item:	9.2

Document for:	Discussion and decision

- Introduction

This document discusses the justification of specifying how to handle GTP-U exceptions (i.e. errors and path failures) for tunnels delivering PDCP PDU, and proposes a method to handle it.

- Discussion

  - Justification

RAN3 had kept GTP-U exceptions untouched for a long time, spanning many releases, until recent meetings:

- At first, RAN3 noticed that one SA2 specification stated that one NGAP procedure can be used when N3mb (i.e. tunnel between RAN and CN to deliver MBS user packets) fails in addition to a more normal case, in the scenario of RAN sharing: the RAN node may switch the UP path to another PLMN if the N3mb with one PLMN fails.

- Then, RAN3 received one (and later more) LS from CT4, as CT4 starts specifying how to handle various exceptions in 5G system, including NG-U GTP-U exceptions.

During recent meetings, multiple CRs were agreed on these topics, including introducing “User Plane Failure Indication” into TS 38.413 for PDU session NG-U GTP-U error:

#### 9.3.1.271	User Plane Failure Indication

This IE is used to notify the NG-RAN node that a user plane failure occurred over NG-U interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| User Plane Failure Type | M |  | ENUMERATED (GTP-U Error Indication Received, UP Path Failure, …) | Indicates the type of NG-U failure. |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the UPF for which the failure occurred. |
| DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node for which the failure occurred. |

But the most noticeable one was R3-243909 agreed last meeting, which added the following section into TS 38.401:

### 7.7.4 Recovery of shared F1-U Failure Recovery of shared F1-U failure is supported for broadcast MBS sessions with the gNB-DU detecting the F1-U failure and triggering recovery handling. If the gNB-DU detects failure of a shared F1-U bearer of a broadcast MBS session it may send the F1AP Broadcast Transport Resource Request message to the gNB-CU CP indicating the F1-U failure and optionally including a new DL F1-U transport address. The gNB-CU CP should understand that any shared F1-U resources previously established for the broadcast MBS session have been implicitly released by the gNB-DU. The gNB-CU CP should then: - trigger towards gNB-CU UP the release of the current gNB-CU UP UL transport resources of the shared F1-U bearer and establishment of new ones and the provision of the new DL F1-U transport addresses if received; - trigger the F1AP Broadcast Context Modification procedure to deliver the corresponding new gNB-CU UP address to the gNB-DU to establish the new F1-U resources.

Observation 1: For F1-U tunnels used to deliver packets for Broadcast Service, RAN3 has already specified how to handle GTP-U exceptions by using a “releasing the old one and adding a new one” method. Its scope is beyond the original statement in SA2 considering only N3mb in RAN sharing scenario, and the solution is different as well.

Therefore, we consider it justified to specify the handling of GTP-U exceptions for Xn-U and F1-U tunnels delivering PDCP PDUs.

Proposal 1: It is proposed for RAN3 to discuss whether to support handling of GTP-U exceptions for Xn-U and F1-U tunnels delivering PDCP PDUs and to make a conclusion. From our side, we think it is reasonable to extend the feature to Xn-U and F1-U for unicast.

We provide solutions in the following paragraphs in case there is consensus to discuss GTP-U exceptions for Xn-U and F1-U.

  - GTP-U Error Indication

The LS concerning split PDU session received from CT4 stated that:

CT4 agreed the attached CR specifying that, for a split PDU session, the SMF may report those errors to the NG-RAN using a PDU session resource modification procedure, so that the NG-RAN may allocate a new N3 DL tunnel or move the QoS flows conveyed by the failed GTP-U tunnel to the other healthy GTP-U tunnel.

That is to say, CT4 considered it alright to use modification for such indication, and potential solution includes releasing the old tunnel and establishing a new one. RAN3 accepted such principle. This can be followed for Xn-U and F1-U tunnels delivering PDCP PDUs.

GTP-U Error typically occurs at only one side of a GTP-U tunnel (due to e.g. context misalignment), therefore it should be supported for both ends (both GTP-U peers) to report such error.

Proposal 2: To use modification (required) procedures, rather than release procedures, to indicate the reception of GTP-U Error Indications for Xn-U and F1-U tunnels delivering PDCP PDUs. Both ends of a tunnel may trigger such procedure.

  - Path Failure

Beside “Loss of GTP-U contexts”, there is another type of GTP-U exception: User Plane failure. This type of exception, as specified in Section 5.2.2 of TS 23.527, is detected by missing of Echo Response message. A User Plane failure, unlike a GTP-U error, can be detected by both GTP-U nodes, thus it works that only one end reports failure.

For NG-U paths (i.e. each encompassing every tunnels using the same UDP/IP address, regardless of GTP-U TEID), the management nodes are SMFs, whereas the GTP-U nodes are UPFs and RAN nodes. It was specified that UPFs, rather than RAN nodes, take the responsibility to report GTP-U failures of NG-U paths.

For F1-U paths / Xn-U paths delivering PDCP PDUs, the management nodes are gNB-CU-CPs, whereas the GTP-U nodes are the gNB-CU-UPs and gNB-DUs. Currently there is no means on either E1AP or F1AP/XnAP to report a user plane failure of F1-U paths / Xn-U paths delivering PDCP PDUs, thus we propose adopting a similar solution for NG-U paths, i.e. to enhance E1AP so that gNB-CU-UPs can report GTP-U failures on F1-U paths / Xn-U paths delivering PDCP PDUs.

Likewise, TS 23.527 allows keeping the Control-Plane context temporarily when GTP-U failure occurs. Therefore we propose using modification (required) procedures for such indication.

Proposal 3: To use modification (required) procedures, rather than release procedures, to indicate either the failure or the recovery (from failure) of the GTP paths used for Xn-U or F1-U tunnels delivering PDCP PDUs. Only the end hosting the PDCP layer of a tunnel may trigger such procedure.

  - Signalling design

For exceptions identified by the PDCP hosting node, it is natural to follow a similar approach in TS 38.413, i.e. the PDCP hosting node reports such exception within step-by-step CP message(s) by an IE similar to Section 9.3.1.271 in TS 38.413, and the corresponding node reports its handling within an IE similar to “User Plane Failure Indication Report” in TS 38.413 except the code point to move flows—nevertheless, it is also possible that any control-plane node within this procedure intervenes by issuing legacy signalling, e.g. to release everything totally or to re-align the context.

Proposal 4: For exceptions identified by the PDCP hosting node, an IE similar to Section 9.3.1.271 in TS 38.413 should be introduced into step-by-step AP messages sent from the PDCP hosting node to the corresponding node, and an IE similar to “User Plane Failure Indication Report” excluding the code point to move flows should be introduced in the AP messages of the opposite direction.

For exceptions identified by the corresponding node, the signalling can be similar except the direction: the “UP Path Failure” is not applicable for this case, but it may be fine to reuse the same IE structure and notes that this code point is not used here.

Proposal 5: For exceptions identified by the corresponding node, the abovementioned IE should be introduced in messages in the opposite direction, and a note should be added stating that the “UP Path Failure” code point is not used for this case.

- Conclusion

Observation 1: For F1-U tunnels used to deliver packets for Broadcast Service, RAN3 has already specified how to handle GTP-U exceptions by using a “releasing the old one and adding a new one” method. Its scope is beyond the original statement in SA2 considering only N3mb in RAN sharing scenario, and the solution is different as well.

Proposal 1: It is proposed for RAN3 to discuss whether to support handling of GTP-U exceptions for Xn-U and F1-U tunnels delivering PDCP PDUs and to make a conclusion. From our side, we think it is reasonable to extend the feature to Xn-U and F1-U for unicast.

Proposal 2: To use modification (required) procedures, rather than release procedures, to indicate the reception of GTP-U Error Indications for Xn-U and F1-U tunnels delivering PDCP PDUs. Both ends of a tunnel may trigger such procedure.

Proposal 3: To use modification (required) procedures, rather than release procedures, to indicate either the failure or the recovery (from failure) of the GTP paths used for Xn-U or F1-U tunnels delivering PDCP PDUs. Only the end hosting the PDCP layer of a tunnel may trigger such procedure.

Proposal 4: For exceptions identified by the PDCP hosting node, an IE similar to Section 9.3.1.271 in TS 38.413 should be introduced into step-by-step AP messages sent from the PDCP hosting node to the corresponding node, and an IE similar to “User Plane Failure Indication Report” excluding the code point to move flows should be introduced in the AP messages of the opposite direction.

Proposal 5: For exceptions identified by the corresponding node, the abovementioned IE should be introduced in messages in the opposite direction, and a note should be added stating that the “UP Path Failure” code point is not used for this case.