3GPP TSG-RAN WG3 #125                                                 R3-244507

Maastricht, NL, 19th – 23th Aug, 2024

Agenda Item:	11.2

Source:	CATT

Title:	Support of AI/ML enabled slicing

Document for:	Discussion

Introduction

The following agreements were achieved in last RAN3 meeting:

In Rel-19, predicted information from CN to NG-RAN is not considered.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

Here we further discuss the AI/ML enabled network slicing use case on the basis of above agreements.

Discussion

  - Slice level UE traffic prediction/measurement

In Rel-17, it was agreed that measured/predicted UE traffic could be the input for load balance use case which is captured in 37.817. Then for per slicing level load balance, it seems natural per slicing level measured/predicted UE traffic could also be local input. But in last RAN3 meeting, there is no consensus on this issue as below.

FFS whether to include measured/predicted UE throughput per slice as input from local node.

The first issue is whether UE traffic could be represented by UE throughput which is defined in TS28.552 as below:

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID.

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for option 3) and for each supported S-NSSAI, and for each PLMN ID.

If UEsThpTimeDl>0, UEsThpVolDlUEsThpTimeDl×1000 [kbit/s]

If UEsThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,  , otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

According to the above definition, we can see that UE throughput is intended for data bursts which depend on radio condition. With good signal quality, big UE throughput can be achieved. On the contrary, with bad radio condition, UE throughput would be small. While average data rate is defined as data volume divided by time duration during which the NG-RAN should allocate resource to deliver these data.

Since UE traffic is used to optimize resource allocation, average data rate other than the burst data rate is more suitable.

For average data rate measurement, it could be defined as data volume/time duration.

Observation 1: Average data rate is more suitable than burst data rate for optimizing resource allocation. So, UE throughput as a burst data rate may be not used as UE traffic.

Proposal 1: It is proposed to include measured/predicted UE data rate per slice from local node as input for slicing use case.

Per UE per slice Measured/Predicted traffic prediction is also useful for neighbour NG-RAN. During handover, target NG-RAN can use this information to decide whether to accept the handed over UE. If the target NG-RAN cannot satisfy the predicated traffic included in handover request, target NG-RAN may reject the UE. Also, after UE handed over to target NG-RAN, target NG-RAN may provide the actual UE traffic information to the source node as a feedback for AI/ML retraining.

Proposal 2: It is proposed to transfer predicted UE data rate per slice from source node to target node and provide measured UE data rate from the target node to source node.

  - SLA related issue

In last RAN3 meeting, there are some discussions on the necessity to transfer measured/predicted SLA between neighbour node, i.e. whether there are scenario where slicing level capacity indicate the resource is  enough while the SLA for this slicing could not be fulfilled.

Some examples are listed below:

F1-U delays are excessive, and they may prevent fulfilment of slice SLA delay requirements. Note that the Slice Available Capacity may not take F1-U delays into account (indeed the definition reported above from TS38.423 does not mandate this and it leaves it up to implementation)

Many UEs at cell edge cannot be served with appropriate resources. As a matter of implementation a RAN node may decides not to use all the resources available towards such cell edge UEs as this may still result il¡n lack of SLA requirement fulfilment. Instead, those resources could be used to served other UEs in better radio conditions.

For F1-U delay, first we think package delay as one of performance measurements should be considered when providing Slice Available Capacity information to neighboring NG-RAN. If package delay cannot satisfy the SLA requirement, slice capacity cannot be judged as available. Second, F1-U delay is not a per slice issue, and should not be considered in slicing use case.

Observation 2: Package delay as one of performance measurements should be considered when reporting slice available capacity. Moreover, F1-U delays is not a per slice issue and should not be captured in slicing use case.

For the cell edge issue, i.e., the SLA fulfilment is good for UEs in the center of the cell while it is bad for UEs located at cell edge. In this case, the average SLA is not fulfilled due to UE in the edge suffered from poor coverage or interference from neighbours. Therefore, Transferring SLA requirement fulfilment due to poor coverage for certain area between NG-RAN is proposed to solve the coverage issue.

However, we have some doubts on whether transferring measured/predicted SLA is useful in this case. Here is an example on coverage issue as the figure 1 below:

Figure 1: example 1

Coverage/interference issue exists between Cell 1 and Cell 2, i.e. the black part area.

Coverage/interference issue does not exist between Cell 1 and Cell 3, i.e. the white part area.

In the scenario depicted in figure1, even SLA could not be satisfied because of the coverage/interference in black part area, it seems not necessary for cell 1 to inform cell 3 that SLA is not fulfilled since UEs handover from cell 3 to cell 1 in the white area could still be served very well. Cell 1 informs Cell 2 the coverage issue may be useful because the UE in black part area may be handed over to Cell 2, or Cell 1 may ask Cell 2 to provide service for this area by CCO.

Observation 3: SLA requirement fulfilment due to poor coverage/interference only impact parts of neighbour NG-RANs.

As for the SLA requirement fulfilment due to poor coverage in cell 1, it is actually a courage issue, not per slice issue, and this issue should fall within the scope of CCO which is to optimize cell coverage between cell 1 and cell2, not only slice related issue.

Observation 4: The SLA requirement fulfilment due to poor coverage, it is actually a coverage issue which may be solved by CCO solution.

Proposal 3: Transferring SLA fulfilment between neighbour nodes could be discussed only when reasonable justification is provided.

  - The granularity of UE performance feedback

In last RAN3 meeting, finer granularity of UE performance feedback has been agreed, but the exact level is FFS. In [1] we make analysis on the granularity of UE performance feedback for general case and proposed to support QoS flow list level UE performance feedback.

Then for slicing use case, since slicing level resource allocation and slicing based load balancing/mobility are considered, it could also be a feasible way to provide slicing level UE performance feedback to assist the peer node to do corresponding optimization.

Proposal 4: Per QoS flow list level together with slicing cell UE performance feedback could be provided from the target to the source node.

Conclusions

Observation 1: Average data rate is more suitable than burst data rate for optimizing resource allocation. So, UE throughput as a burst data rate may be not used as UE traffic.

Proposal 1: It is proposed to include measured/predicted UE data rate per slice from local node as input for slicing use case.

Proposal 2: It is proposed to transfer predicted UE data rate per slice from source node to target node and provide measured UE data rate from the target node to source node.

Observation 2: Package delay as one of performance measurements should be considered when reporting slice available capacity. Moreover, F1-U delays is not a per slice issue and should not be captured in slicing use case.

Observation 3: SLA requirement fulfilment due to poor coverage/interference only impact parts of neighbour NG-RANs.

Observation 4: The SLA requirement fulfilment due to poor coverage, it is actually a coverage issue which may be solved by CCO solution.

Proposal 3: Transferring SLA fulfilment between neighbour nodes could be discussed only when reasonable justification is provided.

Proposal 4: Per QoS flow list level and together with slicing cell UE performance feedback could be provided from the target to the source node.

Reference

[1] R3-244511 (TP for TR 38.743)Enhancement of UE performance metrics CATT