3GPP RAN WG3 Meeting #125	R3-244509

# Maastrict, Netherland, 19th – 23rd August 2024

Source:	CATT

Title:	(TP for TR 38.743) Open issues on the CCO use case

Agenda Item:	11.3

Document for:	Discussion

- Introduction

This document analyses a few open issues on the CCO use case, as well as method to deliver NG-RAN performance measurement result.

- Discussion

Output data of AI/ML based CCO

In last RAN3 meeting, one TP on CCO [1] was noted because there was no consensus on whether future CCO status could be the output of AI/ML based CCO or not. According to the agreement we reached in RAN3#124 meeting, i.e. For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation., it is clear that there are two options for AI/ML based CCO:

Option 1: Both predicted CCO issue and future CCO state are derived via AI/ML messages.

Option 2: Only predicted CCO is derived via AI/ML while future CCO state is derived via legacy mechanism.

In the TR, we should capture the output for both possible options since it depends on implementation which option is adopted. Otherwise, only option 2 is supported and the TR does not reflect the agreements we have reached.

Proposal 1: Include predicted CCO status as the output of AI/ML based CCO.

Future/predicted CCO status transmission over Xn

As to how to support the future CCO status and/or predicted CCO issue transmission over Xn, there are two possibilities on the table as below:

Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.

Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time information to NG-RAN node 2.

We do not consider there is much essential difference between the two alternatives—they are even not mutually distinguishable. One node can surely alter its planned coverage adjustment upon receiving information from neighbour nodes which makes the original plan not preferable. The probability of such alteration can be any value between 0 and 1 and should be fully up to the vendor’s implementation.

Proposal 2: It is fully up to implementation whether one node changes its planned/predicted coverage adjustment upon receiving some information (e.g. feedback) from its neighbours.

Reuse existing IEs or introduce new IEs

Another issue needs to be discussed is whether to reuse the current defined CCO issue and CCO status information for predicted CCO issue and CCO status or introduce a new one.

We do not observe any necessity to define anything new. We acknowledge that the current CCO issues IE may be too coarse, but the detail of any CCO issue is so related with radio circumstance that it is impractical to introduce a universal description format. For the CCO status IE, on the other side, its use is thoroughly checked in RAN3 and we do not consider it righteous to replace it with something else.

And for predicted CCO status, we do not see any benefit to distinguish it from the “future CCO status” in XnAP (or even F1AP) messages. Therefore we consider reusing the existing CCO status IE for predicted CCO status is sufficient enough.

Figure 1: Typical work flow of AI/ML-based CCO.

Proposal 3: Reuse the current defined CCO issue and coverage status information for predicted CCO issue and coverage status.

Time information.

We consider it is natural to carry time information along with the future coverage status, otherwise,the peer node does know when this coverage status would take effect and could not take proper actions accordingly.

Proposal 4: Introduce the time information to be carried along with the future coverage status.

NG-RAN performance measurement

One last thing is on delivering NG-RAN performance measurement results, which reflects how an NG-RAN node performs for all the served UEs (not only the handed over ones) and defined in existing specifications such as TS 28.552. We do not have strong opinion that NG-RAN measurement results should be used for the CCO use case, but if its use is acknowledged by RAN3, we consider it more suitable to deliver them in the Data Collection Update message over Xn interface directly, rather than retrieving from the OAM.

The reason is that the NG-RAN performance measurement results collected by the OAM is usually periodically and with a quite long periodicity, but what is needed in AI/ML use cases such as CCO is generally the difference of NG-RAN performance shortly before an action and shortly after the action, which usually contributes to one result reported to the OAM. Therefore the one collected by the OAM is literally not sufficient for AI/ML.

Proposal 5: If the use of NG-RAN performance measurement results (i.e. something defined in TS 28.552) for the CCO use case is acknowledged, such NG-RAN performance measurement results should be delivered in the Data Collection Update message.

Proposal 6: If Proposal 5 is agreed, we propose that RAN3 discusses what NG-RAN performance metrics are benefit to be delivered in the Data Collection Update message.

- Conclusion

Proposal 1: Include future CCO status as the output of AI/ML based CCO.

Proposal 2: It is fully up to implementation whether one node changes its planned/predicted coverage adjustment upon receiving some information (e.g. feedback) from its neighbours.

Proposal 3: Reuse the current defined CCO issue and coverage status information for predicted CCO issue and coverage status.

Proposal 4: Introduce the time information to be carried along with the future coverage status.

Proposal 5: If the use of NG-RAN performance measurement results (i.e. something defined in TS 28.552) for the CCO use case is acknowledged, such NG-RAN performance measurement results should be delivered in the Data Collection Update message.

Proposal 6: If Proposal 5 is agreed, we propose that RAN3 discusses what NG-RAN performance metrics are benefit to be delivered in the Data Collection Update message.

One TP is provided in the Annex.

- Reference

[1] R3-243939; (TP for TR 38.743) AI/ML enabled CCO; Qualcomm Incorporated, Ericsson, Huawei, ZTE, Nokia, CATT

- Annex: Text proposal to TR 38.743

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2 Standard impact

Xn interface may be enhanced to support:

- Enhanced (future) coverage configuration IE to include more information, e.g. the time to change.

F1 interface needs be enhanced to support:

- Delivering predicted coverage or capacity problem from gNB-CUs to gNB-DUs.

- Enhanced (future) coverage configuration IE to include more information, e.g. the time to change.

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////