3GPP RAN WG3 Meeting #125	R3-244510

# Maastrict, Netherland, 19th – 23rd August 2024

Source:	CATT

Title:	(TP for TR 38.743) Discussion on left over issues on DC and split architecture

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document focuses on how the AI enhancements introduced in Rel-18 may apply in DC and split gNBs.

This document is mainly a revised one based on R3-243291. Some main differences include:

- Removing the part proposing deploying AI/ML model at the gNB-CU-CP (already captured in the TR).

- Removing the part of delivering energy cost (already captured in the TR).

- Further analysis on why the delay result in F1-U is not suitable for AI/ML purpose.

- Analysis on the case that a new node (typically a gNB-DU or an SN) is configured to serve the UE.

- Discussion

  - Exchanging UE performance

The IE “UE Performance” is a group of per-UE metric, and may contain three types of measurement results: Average UE throughput, Average Packet Delay, and Average Packet Loss DL. In DC scenario the data for a UE may be split into more than one node, so the MN has to collect both the UE performance measured in the MN itself and the one measured in the SN in order to generate the final “UE Performance” provided toward e.g. the source node.

The question is which node collects these three types of UE performance result in split scenarios—once this is decided the case for multiple bearer types in DC is decided as well. Our preference on this issue is following the legacy mechanism introduced for SON/MDT.

It was ever proposed that gNB-DUs can send the delay of its part toward the gNB-CU-UPs through the F1-U. It is true, but it cannot satisfy the requirement in AI/ML:

- In AI/ML the requesting node usually indicates the periodicity or time window of measurement, so as to provide proper training input (e.g. as an input of “reward”). But the method to report delay over F1-U does not include a configurable periodicity or time window.

- In AI/ML it is assumed that every node may reject data collection requests due to various reasons, and upon such reject further action may be taken, e.g. issuing a reject further back (gNB-DU rejects gNB-CU and then gNB-CU rejects the originating gNB), altering the parameter of request or cancelling requests to other nodes which are deemed to be useless. But the method to report delay over F1-U does not include an outcome IE.

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

The cases on throughput and packet loss are simple:

- Both downlink throughput and uplink throughput are measured by the gNB-DU as defined in Section 6.3.1.4.1 and Section 6.3.1.4.2 of TS 28.558 respectively.

- Downlink packet loss is measured by the gNB-DU as defined in Section 4.2.1.5.1 of TS 38.314, whereas uplink packet loss is measured by the gNB-CU-UP as defined in Section 6.3.1.3.1 of TS 28.558.

The case is quite complex on packet delay as shown in Section 4.2.1.2.1 of TS 38.314. In the downlink direction it is defined as the sum of 4 items:

- Average delay DL air-interface in Section 6.3.1.1.1 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.1.1 of TS 28.552);

- Average delay in RLC sublayer, measured by the gNB-DU as defined in Section 6.3.1.1.2 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.3.3 of TS 28.552);

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average delay DL in CU-UP, measured by the gNB-CU-UP as defined in Section 6.3.1.1.4 of TS 28.558.

In the uplink direction it is defined as the sum of 5 items:

- UL PDCP packet average delay, measured by the UE as defined in Section 4.3.1.1 of TS 38.314 (it is wrongly defined as measured by the gNB-CU-UP in TS 28.558);

- Average over-the-air interface packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.2 of TS 38.314;

- Average RLC packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.3 of TS 38.314;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average PDCP re-ordering delay, measured by the gNB-CU-UP as defined in Section 4.2.1.2.4 of TS 38.314.

Nevertheless, it is not necessary to report every metric separately. The delay measured in gNB-DU can be combined into one single metric, and so can the delay measured in gNB-CU-UP.

As the summary, there are 5 metrics collected in the gNB-DU whereas 3 metrics collected in the gNB-CU-UP. The remaining one is collected by the UE, which is reported toward the gNB-CU-CP directly and thus has no impact on either E1AP or F1AP.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 3 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT):
-	UL packet loss;
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

  - Measured UE Trajectory

This IE now contains only the list of visited PCell, therefore does not need any information from the SN. In split gNB it is fully known in the gNB-CU-CP and thus does not need any enhancement on E1AP or F1AP either.

Similar to handover optimisation in Rel-18 AI/ML for NG-RAN, it is beneficial to include PSCell change optimisation in the Rel-19 AI/ML WI. And if it includes, it will be beneficial to enhance the “Measured UE trajectory” IE to include the list of visited PSCell, i.e. the target MN collects not only the actual list of PCell but also the actual list of PSCell and provides these lists toward the source MN. There is no need to enhance any XnAP message sent from the target SN toward the target MN, as the legacy mechanism described in Section 13.3 of TS 37.340 can be reused.

Proposal 3: Enhance the existing Measured UE Trajectory IE to include the actual list of PSCells, so that the source MN can train/fine-tune/retrain models predicting future PSCells, or monitor their performance.

  - Cell-Based UE Trajectory Prediction

Similar to the case on Measured UE Trajectory, it is beneficial to enhance this IE to include the list of predicted PSCells, and to push such information from the MN toward the SN.

Proposal 4: Enhance the existing predicted UE trajectory IE to include the predicted list of PSCells.

Proposal 5: Enhance DC-related request messages so that the predicted list of PSCells can be forwarded toward the (target) SN.

  - Introducing new metrics

We observe one type of information which is not introduced in the Rel-18 WI but essential in split scenario: the UE traffic volume. It was ever mentioned in the discussion for the Energy Saving use case, as the “Energy Efficiency” is literally the data volume divided by the energy consumption.

Companies found it hard to describe traffic between neighbouring gNBs, but this is not the case within one gNB: the source gNB-CU-CP knows everything from the QoS requirement toward the UE radio status (i.e. RSRP etc), with the only exception is the data rate for each flow.

Therefore we propose enhancing E1AP so that the gNB-CU-UP can provide the data rate for each flow toward the gNB-CU-CP. (The model for energy saving should surely reside in the gNB-CU-CP.)

Proposal 6: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

  - Message to use

As analysed above, there are many metrics needs to be delivered over XnAP for DC, E1AP and F1AP, many of which the same as the metrics included in XnAP Data Collection Update messages. For XnAP for DC, the Rel-18 XnAP Data Collection Update message can be reused. For E1AP and F1AP, we propose introducing similar procedures for alignment. For measurement objects related to one single UE, a measurement ID pair for reference is added into UE-associated messages, such as XnAP SN Addition Request messages, F1AP UE Context Setup Request messages, E1AP Bearer Context Setup Request messages.

Proposal 7: To introduce new data collection procedures into E1AP and F1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

  - New node is configured sometime during the feedback collection window

After a handover take place, the target node may need to collect the feedback upon the prior request from the source node. It is possible that a new node is added to serve the UE while it is still right within the feedback collection window:

- The UE further experiences an intra-CU inter-DU handover (or PSCell change), thus a new gNB-DU (or even gNB-CU-UP) starts to serve the UE, and some performance needs to be collected by the new gNB-DU (or gNB-CU-UP).

- The target node further configures an SCG for the UE, and some performance needs to be collected by the SN or secondary gNB-DU.

- The UE further experience an SN change, and some performance needs to be collected by the new SN.

Generally speaking, the existing data collection mechanism can be reused for such case, except the length of collection time duration: the duration is now a reduced value (the original duration minus the time length between the two RRC reconfigurations), rather than the original one.

The source node expects the feedback one “collection time duration” after the handover toward the target node take places (after which the source node may delete the UE context for data collection), so the target node must collect all information and send it back to the source node in time. Therefore, the new serving node must collect information within the reduced duration and send it back instantly.

Considering that the new serving node has no idea of the time length between the two RRC reconfigurations, the new serving node needs to be provided with a “remaining” collection time duration. Considering that the time length between the two RRC reconfigurations varies from one UE to another, we prefer indicating the “remaining” collection time duration within a UE-associated message, i.e. the message containing the Data Collection ID.

Proposal 8: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

- Conclusion

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 3 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT):
-	UL packet loss;
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

Proposal 3: Enhance the existing Measured UE Trajectory IE to include the actual list of PSCells, so that the source MN can train/fine-tune/retrain models predicting future PSCells, or monitor their performance.

Proposal 4: Enhance the existing predicted UE trajectory IE to include the predicted list of PSCells.

Proposal 5: Enhance DC-related request messages so that the predicted list of PSCells can be forwarded toward the (target) SN.

Proposal 6: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

Proposal 7: To introduce new data collection procedures into E1AP and F1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

Proposal 8: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

Based on the proposals above, we propose one TP for TR 38.743 in the Annex.

- Annex: TP for TR 38.743

## 5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

### 5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

### 5.1.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Adding List of PSCells into the existing Measured UE Trajectory IE and the existing Predicted UE Trajectory IE.

- Predicted list of PSCells from the MN to the (target) SN.

- Measurement ID pair from the MN to the SN.

## 5.2	Split architecture support for Rel-18 use cases

Editor Note: Capture the description and its potential standard impacts.

### 5.2.1 Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

### 5.2.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

- Data collection procedures.

- Measurement ID pair from the gNB-CU-CP to the gNB-CU-UP.

- Measured UE performance from gNB-CU-UP to gNB-CU-CP.

- Measured data rate from the gNB-CU-UP to the gNB-CU-CP.

F1 interface:

-	Data collection procedures.

-	Measurement ID pair from the gNB-CU to the gNB-DU.

-	Measured UE performance from the gNB-DU to the gNB-CU.

-	Measured Energy Cost from gNB-DU to gNB-CU.