3GPP RAN WG3 Meeting #125	R3-244512

# Maastrict, Netherland, 19th – 23rd August 2024

Source:	CATT

Title:	(TP for TR 38.743) Finer granularity of Energy Cost

Agenda Item:	11.4

Document for:	Discussion

- Introduction

This document discusses the following issue captured last meeting:

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

- Discussion

At first we would like to recall the work flow of the Energy Saving use case (Figure 1 shows one typical work flow, details of other implementation may vary):

Figure 1: Typical work flow for “source node to predict” method (using raw actual EC).

What matters for evaluating an ES action, generally speaking, is the change of EC due to the action. This can be reflected by the actual change of EC of the measured area. However, the measured delta may be polluted by many other factors, from handovers into or out of the measured area, to changes of data rate of UEs within the area, as pointed out by some companies during Rel-18 discussion. The larger the area is, the more significant the pollution is.

An ideal solution might be determining the area as the set of cells which are expected to be affected by the ES action, however this is rarely practical since any of such cells may share a lot of hardware resources with cells that are not expected to be affected—that is to say, “EC per cell” is rarely measurable. It is also unreliable to split the measured EC into multiple “EC per cell” by using DV, because the amount of consumed energy to deliver each bit may be drastically different due to radio condition. Splitting by using PRB usage may be slightly better, but is also affected by differences in transmitting power.

So we turned to “EC per gNB” which is at least measurable according to specifications within 3GPP. In Rel-18 only aggregated gNBs are supported, and one aggregated gNB usually serves few cells, so the problem is not much.

Observation: The pollution on estimating the change of EC caused by an ES action is not much in Rel-18, because only aggregated gNBs are supported to provide EC.

But in Rel-19 now, split gNBs—each may serve many cells—are taken into consideration.

Fortunately, most energy is consumed in gNB-DUs for this scenario, and one gNB-DU usually serves few cells.

So the most straightforward solution—and the most aligned with Rel-18—is to deliver essentially the EC per gNB-DU. Nevertheless, one gNB does not know the internal structure of its neighbour, so in the signalling it can be represented by EC per group of cells.

Such approach may also support finer split within one gNB-DU. For example, if some cells in a gNB-DU use a set of hardware whereas other cells in the gNB-DU use another set of hardware, mutually excluded, the gNB-DU can surely provide two EC per group of cells. We prefer leaving such split fully up to implementation.

Proposal 1: Introducing “EC per group of cells”, identified by NR CGIs. A gNB-DU may also split its served cells into multiple groups and provides one EC per group if it is capable to do so, but how to split should be fully up to the gNB-DU’s implementation.

One last issue: It is obviously a waste of signalling to deliver EC of the groups that is irrelevant for an ES action, but the source node (Node 1 in Figure 1) may not know how the target node split its served cells into groups. Nevertheless, this is not a problem: the source node can anyway include every cell which it considers to be affected by the ES action, and the target node simply includes measured EC of every group which covers at least one cell in the request message.

For example, the target node hosts 9 cells: Cell #1–3 share one set of hardware, Cell #4–6 share another, and Cell #7–9 share the third. At some point of time the source node decides to take an ES action and considers Cell #1, #2 and #4 to be affected. It requests EC for Cell #1, #2 and #4 in the request message, and then the target provides the EC for Cell #1–3 and the EC for Cell #4–6 in the update message, before and after the ES action.

Proposal 2: The node requesting EC may indicate only the EC for some cells are needed, and the requested node provides the measured EC of every group of cells which includes at least one cell in the request messages, and omits other group of cells.

- Conclusion

Observation: The pollution on estimating the change of EC caused by an ES action is not much in Rel-18, because only aggregated gNBs are supported to provide EC.

Proposal 1: Introducing “EC per group of cells”, identified by NR CGIs. A gNB-DU may also split its served cells into multiple groups and provides one EC per group if it is capable to do so, but how to split should be fully up to the gNB-DU’s implementation.

Proposal 2: The node requesting EC may indicate only the EC for some cells are needed, and the requested node provides the measured EC of every group of cells which includes at least one cell in the request messages, and omits other group of cells.

One TP is provided in the Annex according to the proposals above.

- Annex: TP for TR 38.743

## 5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

### 5.3.1	Use case description

Energy Cost (EC) is introduced as a per-node measurement result in Rel-18 mainly used for evaluating the benefit of ES actions, but the measured EC itself may be affected by other factors such as handovers due to mobility, change of data rate due to service. The more cells a node hosts, the larger the affect. It is thus problematic in Rel-19 where split gNBs are taken into consideration.

### 5.3.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn and F1 interface:

-	Measured Energy Cost per group of cells, with each group including at least one cell in the requested message.