3GPP RAN WG3 Meeting #125	R3-244513

# Maastricht, Netherlands, 19th – 23rd August 2024

Source:	CATT

Title:	Discussion on potential RAN3 impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and decision

- Introduction

The LP-WUS WI (NR_LPWUS-Core) starts this meeting. This document analyses some potential RAN3 impacts as quite many topics depends on the output of other WGs.

- Discussion

  - UE paging subgroup ID

One of the most obvious impacts on RAN3 is on UE paging subgroup IDs. Any bit of a subgroup ID for LP-WUS will probably (although still in discussion) either be CN-configured or UE-ID-based (i.e. based on 5G-S-TMSI), similar to the case on PEI.

A UE may be configured with both PEI and LP-WUS, and the UE may—depending on its own implementation—use the two simultaneously: the UE monitors LP-WUS first, and upon being wakened by LP-WUS, monitors PEI, and upon being wakened by PEI, monitors PO. It is therefore beneficial to make subgroup ID for PEI and the one for LP-WUS as statistically independent as possible, in order to minimise false alarm due to multiple UEs using both the same LP-WUS subgroup ID and the same PEI subgroup ID.

For CN-configured (bit of) LP-WUS subgroup ID, one method is reusing the same ID as PEI, but this method will clearly lead to maximise the case for false alarm if both features are configured. And in the current RAN3 specification, this IE is placed within a structure named “PEIPS Assistance Information”, literally dedicated for PEI:

#### 9.3.1.232 PEIPS Assistance Information This IE provides the information related to CN paging subgrouping for a particular UE, as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CN Subgroup ID; M; INTEGER (0..7, …).

Observation 1: Reusing the current CN subgroup ID (which was introduced for PEI) for LP-WUS is neither optimal for performance nor suitable due to its PEI-dedicated IE name. Introducing a new one is more preferable.

Therefore, introducing a new CN-configured subgroup ID seems more reasonable. Like the case with PEI subgroup ID, it should be introduced into NGAP, XnAP and F1AP paging messages, as well as the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP.

Observation 2: If a new CN-configured subgroup ID for LP-WUS is introduced, it should be added into NGAP, XnAP and F1AP paging messages, as well as the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP, similar to the case of CN-configured subgroup ID for PEI.

Proposal 1: Capture in TS 38.300 that a new CN-configured subgroup ID for LP-WUS is included in the “Core Network Assistance Information for RRC INACTIVE”, and its use is similar to the case of CN-configured subgroup ID for PEI.

For UE-ID-based (bit of) subgroup ID, the case is a little more complex.

The part of UE ID which determines the PO is the up to 12 LSBs (least significant bits), and part of UE ID used for PEI is the up to 3 LSBs next to the ones used to determine the PO. That is to say, up to 15 LSBs is used if a UE is configured with PEI only. For CN-originated paging the RAN is provided with the entire UE ID, and for RAN-originated paging RAN node is provided either with the legacy 10 LSBs or with an extended IE of 16 LSBs:

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

Now 3GPP needs to find a method to determine the UE-ID-based subgroup ID. One popular opinion in RAN1 is that the upper bound of number of subgroups ID is 2n and n may be up to 4, which leads to quite a lot of possible designs:

- It may be specified that, the UE-ID-based subgroup ID can be up to 4 bits and utilises the LSBs in the part of UE ID yet not used to determine PEI or PO, but RAN implementation guarantees that the number of bits in a UE ID used to determine LP-WUS, plus the one for PEI, plus the one for PO, shall not exceed 16. As the result, the existing 16-bits IE in RAN3 specs are sufficient enough.

- It may be specified that, the UE-ID-based subgroup ID can be up to 4 bits, utilising the LSBs in the part of UE ID yet not used to determine PEI or PO, and there is no extra limits in configuration. As the result, the total length of the bits in a UE ID used for paging can be up to 19, and a “Further Extended UE Identity Index Value” shall be introduced. Nevertheless, whether delivering so much part of UE ID incurs privacy issue may need evaluation.

- It may be specified that, the UE-ID-based subgroup ID is not truncated as the LSBs of something, but truncated as the MSBs of the aforementioned 16-bits “Extended UE Identity Index Value”.

- It may be specified that, if the number if required bits exceeds 16, the LSBs (used to determine the PO rather than the PEI subgroup) of the 16-bit UE ID is reused to fill the blank.

- It may be specified that, in order to cope with the privacy requirement, the UE-ID-based subgroup ID for LP-WUS is truncated not from the plaintext UE ID but from the existing hashed UE ID (originally introduced for eDRX) instead:

#### 9.3.3.62 Hashed UE Identity Index Value This IE is the 13 Most Significant Bits (MSBs) of the Hashed ID defined in TS 38.304 [12] or TS 36.304 [29]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Hashed UE Identity Index Value; M; BIT STRING (SIZE(13, …)).

Observation 3: There are many possible ways to specify how to determine the UE-ID-based subgroup ID for LP-WUS, and few require introducing new IE.

Proposal 2: For UE-ID-based subgrouping, wait until RAN1/2 makes further progress.

  - Other miscellaneous

System information

Configuration of LP-WUS (except for CN-based subgroup ID) is provided to the UE by the system information. It will not cost many bits and is likely included in SIB1, as for PEI. So there is no impact on RAN3 specs.

UE’s capability to support LP-WUS

Although not discussed yet, it will obviously be encoded as part of the “UE Radio Capability for Paging of NR” container in RAN3 specs, identical to “UERadioPagingInformation” in TS 38.331. So there is no impact on RAN3 specs.

- Conclusion

Observation 1: Reusing the current CN subgroup ID (which was introduced for PEI) for LP-WUS is neither optimal for performance nor suitable due to its PEI-dedicated IE name. Introducing a new one is more preferable.

Observation 2: If a new CN-configured subgroup ID for LP-WUS is introduced, it should be added into NGAP, XnAP and F1AP paging messages, as well as the “Core Network Assistance Information for RRC INACTIVE structure in NGAP”, similar to the case of CN-configured subgroup ID for PEI.

Proposal 1: Capture in TS 38.300 that a new CN-configured subgroup ID for LP-WUS is included in the “Core Network Assistance Information for RRC INACTIVE”, and its use is similar to the case of CN-configured subgroup ID for PEI.

Observation 3: There are many possible ways to specify how to determine the UE-ID-based subgroup ID for LP-WUS, and few require introducing new IE.

Proposal 2: For UE-ID-based subgrouping, wait until RAN1/2 makes further progress.

One TP is provided in [2] accordingly.

- Reference

[1] R3-223961; Summary of Offline Discussion on CB: #UEPowSav_Corrections; ZTE (moderator).

[2] R3-244514; (TP on BLCR for TS 38.300) Support of LP-WUS; CATT.