3GPP TSG-RAN WG3 Meeting #125	R3-244516

Maastricht, NL, August 19-23, 2024

Agenda Item:	15.2

Source:	THALES, Novamint, Sateliot

Title:	Discussion on support of Store and Forward in NTN

Document for:	Discussion

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes the benefit to support Store and Forward operation for NB-IoT NTN.

Benefits specifically associated with the support of Store and Forward (S&F) Satellite operation with regenerative payload: - S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas) - Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure. - The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]. - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In the same time, SA2 concluded on the following normative impacts of the store and forward [2] the two architecture considered, Split MME architecture and full CN onboard:

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following normative impacts: - When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information). - NOTE 1: The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal. - When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. […] The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following normative impacts: - Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

RAN2#126 assumed the following steps for store and forward satellite operation:

For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

In this contribution we discuss the store and forward operation and provide a path to support it, especially in the stage 2 description and S1 operations.

# 2	Discussion

Store-and-Forward Satellite operation

Figure 1 - Store and forward Satellite operation

Store and Forward Satellite operation is defined in [4] as the ability of a NTN to “provides a way to enable autonomously network service to UEs without the satellite always being connected to the ground station, which can extend the service availability for the areas without the connectivity to ground station via feeder link or ISL (e.g. at sea, very remote areas lack of ground-station infrastructures), improve the ground segment affordability with fewer ground stations and allow more robust UE services with the satellite under intermittently/temporarily unavailable feeder link.”.

The study performed in [3] describes 5 use cases for Store and Forward Satellite operation that are:

- Mobile-Originated message Store and Forward

- Mobile-Terminated message Store and Forward

- Inter-Satellite Store and Forward

- Data transfer for IoT devices in remote area

- Emergency report

Also, [4] defines also a list of requirements associated to Store and Forward Satellite operation including:

- Store data on the satellite when the feeder link is unavailable; and forward the data once the feeder link between the satellite and the ground segment becomes available as illustrated on the figure above.

- A 5G system with satellite access shall be able to inform a UE whether S&F Satellite operation is applied

- Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

Finally, [5] studied different solutions to support regenerative IoT NTN payload and store and forward operations. Especially, the regenerative payload architecture pose the problem of the moving interfaces and network registration that are in the scope of the RAN3.

Store and forward Satellite operation needs support of the regenerative payload architecture for IoT NTN

Support regenerative payload for LTE

The regenerative payload is introduced in Rel-18 for NR NTN. The stage-2 endorsed CR for TS 38.300 describe a baseline architecture to support regenerative payload for NR NTN [6] as presented on the Figure below which shows an overall illustration of an NTN with regenerative payload hosting a gNB.

Figure 2 - Overall illustration of an NTN with regenerative payload hosting a gNB

The Figure 4.12-1 in TS 36.300 should be updated with a Figure 4.12-1x that present a regenerative architecture where the eNB and at least a part of the CN is embarked in the NTN payload. Also, a NOTE should precise that the feeder link might be intermittent.

Support the regenerative payload architecture in TS 36.300. The endorsed CR R3-243949 to support regenerative payload in NR NTN could be used as baseline

These RAN modifications are mostly transparent for the UEs. The UEs are informed about the store-and-forward data transmission method by an indication in the SIB.

The UEs are informed via System Information of whether S&F satellite operation mode is activated or not in an NTN cell. The procedure is out of the RAN3 scope

Discussion on multi-satellite Store-and-Forward Satellite operation

Contrary the Figure 2 presenting inter-satellite link (ISL) between satellites, SA2 only studied solutions without ISL between satellites. Therefore, as a baseline, ISL are not considered in the stage-2.

However, it is important to note that due to the nature of a NGSO constellation, the serving satellite of a UE is always changing as allowing a UE to be served by different satellites within the constellation is key to reducing the revisit time for the UE.

Note that the revisit time with a single satellite in typical LEO orbits may be around 12h or longer [9]. Therefore, restricting the solution to single-satellite would mean that a UE wil only be able to send/receive data e.g. twice a day on average.

This case is even reinforced for a moving UE.

Therefore, the support of a Store and Forward Satellite operation solution able to serve a UE via multiple satellites (in other words, not restricting the interaction of a UE and the network to a single satellite of the constellation) should be considered in the architecture from the beginning. This is referred in the following as multi-satellite S&F.

Figure 3 - Discontinuous coverage and Store-and-forward examples (extracted and modified from R2-2403337)

The support of multi-satellite S&F is also interesting for some advanced scenarios of deployment where the number of gateways are limited and ISL does not draw a direct path between a cluster of satellite (inside a same orbital plane for instance) with a gateway. With an appropriate exchange of information (MME-MME S10 interface), the satellite are able to distribute data in the constellation and transmit them to the ground as soon as a feeder link to a gateway is available. The data are not considered stored in a specific satellite’s MME but in the network of MME. It brings more resiliency to the system, since the data can be distributed between satellites in the constellation by routing and can then support longer feeder link interruption for non-delay tolerant data. Also, a satellite when its buffer is full can store data on another neighbour satellite. The need for a communication infrastructure resilient to feeder-link momentarily unavailable is also stated in [7], “the connection between the satellite and the rest of the NB-IoT network functional elements is not guaranteed over time. […] As a consequence, the whole architecture shall be designed in order to allow NTN terminals to exchange data with satellites, even in the temporary absence of the feeder-link”.

The multi-satellite support for Store and Forward Satellite operation (multi-satellite S&F) should be considered as the baseline and general case, while restricting the number of satellites that can serve to a given UE to one (single-satellite S&F) is a particular deployment case.

Impact on S1 interface

The full CN onboard architecture (eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC, etc…) is depicted on the Figure below:

NTN payload

proxy

Figure 4 - EPC architecture with full CN onboard

The intermittent feeder link supports the SGi interface between the P-GTW and the IP network. This interface is outside of the RAN3 scope. SA2 recommend to handle the mobility on this interface using a proxy, that is outside of the 3GPP scope.

For the Store and Forward architecture with full CN onboard there is no impacts on S1

The split MME architecture for supporting S&F satellite operation for SMS and CP CIoT services is presented in Figure 5a.

Figure 5a: EPC architecture with MME split for supporting S&F satellite operation for SMS and CP CIoT services

An extension of the split MME architecture in Figure 5a to consider also S1-U / UP CIoT services is presented in Fig. 5a. Note that the architecture Figure 5b is a proposal in this paper and not an architecture taken from the conclusion of the SA2 study.

NTN payload

Figure 5b – EPC architecture with MME split for supporting S&F satellite operation for SMS, CP CIoT services, S1-U / UP CIoT services.

In both architectures in Figure 5a and 5b, the NTN payload includes the E-UTRAN (i.e. eNB) and a part of the MME, the MME-onboard as defined in conclusion of KI#2 in [2]. The MME split is realized with an interface out of scope of 3GPP through the intermittent Feeder link. The S1-MME interface is local to the NTN payload (so no impact on the specifications) and not modifications are foreseen here to support S&F satellite operation.

On the other hand, if S1-U and UP CIoT services are also to be supported, further discussion is necessary with respect to the need to split also the S-GW and have a S1-U interface local to the NTN payload or to study the impacts on the S1-U interface if it has to be deployed over the feeder link as represented in Figure 5b, where the S1-U between E-UTRAN and S-GTW is transported through the intermittent Feeder link.

In such a case, the optimizations, if needed, to support S1-U mobility are similar to the one for NG mobility for NR NTN for instance a suspend/resume mechanism to avoid disconnect/connect procedures. When the feeder link disconnect, either the satellite get it later and then, it suspends the connection without releasing it or the satellite will connect to another gateway, or the same after one or few revolution around the Earth. In this case, the suspend mechanism is less interesting since it needs to keep UE contexts. The second scenario is more realistic than the first one, especially without ISL where the interruption could be long. The assumption is that current procedure works to handle S1-U mobility.

If S1-U / UP CIoT services are to be supported in S&F mode with S1-U deployed over the feeder link, current S1(-U) procedure could be reused to support the MME split architecture for Store and Forward (This approach would also require the eNB being able to store the data from/to UEs and deliver over the S1-U interface when activated).

The other aspect of the intermittent feeder link impacts lies in the different timers since in the 3GPP specifications, it is assumed a continuous connectivity between the core network components and the RAN. As explained in [8], relevant timers for the completion of attach, detach, tracking area updating, service request and paging procedure have been extended up to 255 seconds to support NB-IoT.

| Procedure | Timer | WB-S1 | NB-S1 | Note |
| --- | --- | --- | --- | --- |
| Attach | T3410 T3460 T3411 T3402 | 15s 6s 10s 10mn | 255s 246s | UE-side UE-side |
| Detach | T3421 T3422 | 15s 6s | 255s 246s |  |
| Tracking Area Updating | T3430 | 15s | 255s |  |
| Service Request | T3417 T3417 | 5s 10s | 245s 250s | Normal Extended |
| Paging | T3413 T3413 |  |  | Network dependent Network dependent |

The bullet 2) to the conclusion of KI#2 in [2] gives the following procedure at least for Attach and TAU:

When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

These methods to reject attachment or TAU re-use existing procedures with probable modifications of NAS messages to support wait timer. Further procedures will be discussed by SA2 during the normative phase and the security-related issues in MME split architecture will be handled by SA3.

The timers-related issues for Attach and TAU reject are handled by SA2

The security-related issues with MME split architecture are handled by SA3

Note that in a multi-satellite architecture, S10 is carried through ISL. Further message exchange will be needed to support for instance, load balancing and data forwarding.

Further message exchange between satellite (eNB + MME) are to be discussed if multi-satellite architecture is considered

# 3	Conclusion

In the previous sections we made the following observations and proposals:

- Store and forward Satellite operation needs support of the regenerative payload architecture for IoT NTN

- Support the regenerative payload architecture in TS 36.300. The endorsed CR R3-243949 to support regenerative payload in NR NTN could be used as baseline

The UEs are informed via System Information of whether S&F satellite operation mode is activated or not in an NTN cell. The procedure is out of the RAN3 scope

The multi-satellite support for Store and Forward Satellite operation (multi-satellite S&F) should be considered as the baseline and general case, while restricting the number of satellites that can serve to a given UE to one (single-satellite S&F) is a particular deployment case.

For the Store and Forward architecture with full CN onboard there is no impacts on S1

If S1-U / UP CIoT services are to be supported in S&F mode with S1-U deployed over the feeder link, current S1(-U) procedure could be reused to support the MME split architecture for Store and Forward (This approach would also require the eNB being able to store the data from/to UEs and deliver over the S1-U interface when activated).

- The timers-related issues for Attach and TAU reject are handled by SA2

- The security-related issues with MME split architecture are handled by SA3

- Further message exchange between satellite (eNB + MME) are to be discussed if multi-satellite architecture is considered

# References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#104, 2024-06, Shanghai, CN.

TR 23.700-29, “Study on integration of satellite components in the 5G architecture; Phase 3”, SA#104, 2024-06, Shanghai, CN.

TR 22.865, Study on satellite access Phase 3, v19.2.0, 2023-12.

TR 22.261, Service requirements for the 5G system, v19.5.0, 2023-12.

TR 23.700-29, Study on integration of satellite components in the 5G architecture, v0.4.0, 2024-03.

R3-243949, “Support for Regenerative Payload in NR NTN”, RAN3#124, Ericsson et al., 2024-05, Fukuoka, JP.

Sciddurlo, Giancarlo, et al. "Looking at NB-IoT over LEO satellite systems: Design and evaluation of a service-oriented solution." IEEE Internet of Things Journal 9.16 (2021): 14952-14964.

Kellermann, Timo, et al. "Novel architecture for cellular IoT in future non-terrestrial networks: store and forward adaptations for enabling discontinuous feeder link operation." IEEE access 10 (2022): 68922-68936.

R2-2107453, “On LEO satellite flyover timing and discontinuous coverage”; Eutelsat, August 9th - 27th, 2021, RAN2#115-e