3GPP TSG-RAN WG3 Meeting #125	R3-244518

Maastricht, NL, 19 - 23 August, 2024

Agenda item:	9.2

Source: 	Huawei, Ericsson, Lenovo, CMCC, Qualcomm, Xiaomi

Title: 	Discussion on DL PSI based discard

Document for:	Discussion and Decision

1. Introduction

DL PSI based discard has been supported in RAN3#123 meeting. In addition to the agreed E1AP enhancement, there are still some other issues need to be resolved. In this contribution, we will analyse the remaining issues and give our proposals.

2. Discussion

According to latest agreement in RAN3#123 meeting [1], the gNB-CU-CP can configure the PSI based discard timer to the gNB-CU-UP to enable DL PSI based discard. As indicated in TS 23.501 [3], PSI based discard should only be active in presence of congestion, with the intention to mitigate the network congestion. If the gNB-CU-UP activates DL PSI based discard when the network is not congested, it may result in unexpected packet loss and effect the user experience.

----------Excerpted from TS 23.501---------- The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

Observation 1: The gNB-CU-UP shall not activate DL PSI based discard when the network is not congested.

However, it is difficult for the gNB-CU-UP to monitor the network congestion level directly since it is not aware of the load over the Uu interface. Instead, the gNB-DU is much more aware of the congestion. To resolve the issue, different options were proposed to enable the gNB-CU-UP be aware of the congestion status. We analyse the options and give our views as below.

- Option 1: Introducing specific notification and suggestion mechanism for DL PSI based discard.

This option has the design similar to PDCP duplication. PDCP duplication is used to improve the reliability when the air interface link quality deteriorates. For DL, the gNB-DU can measure the link quality and suggest the gNB-CU-UP to perform PDCP duplication when needed. Specifically, the gNB-CU-CP indicates the gNB-DU to setup the RLC entities used for PDCP duplication during the UE Context Setup procedure and the UE Context Modification procedure. Then the gNB-DU can send PDCP duplication activation suggestion to the gNB-CU-UP via the ASSISTANCE INFORMATION DATA frame so that the gNB-CU-UP can perform DL PDCP duplication accordingly, the design for such suggestion was captured in specification TS 38.425 and the related parts are excerpted as below.

----------Excerpted from TS 38.425---------- 5.5.2.3 ASSISTANCE INFORMATION DATA (PDU Type 2) This frame format is defined to allow the node hosting the PDCP entity to receive assistance information. The following shows the respective ASSISTANCE INFORMATION DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=2); PDCP Dupl. Ind.; Assistance Info. Ind.; UL Delay Ind.; DL Delay Ind.; 1. Spare; UL Congestion Information Ind.; DL Congestion Information Ind.; PDCP Duplication Activation Suggestion; 1. ……; …….

As shown in Figure 1, an analogue mechanism can be designed for DL PSI based discard. First, to inform the gNB-DU to monitor the network congestion and judge whether PSI based discard is needed, the gNB-CU-CP shall notify the gNB-DU about per DRB PSI based discard configuration. This can be done during the UE context setup and modification procedures. Similar to the IE designed for UL PSI based discard, the notification IE for DL can also include two codepoints to indicate whether DL PSI based discard is (re)configured for the considered DRB. Corresponding CR for TS 38.473 can be found in our correlated contribution in [6].

After getting the notification, the gNB-DU can determine whether PSI based discard should be activated or deactivated for the DRB based on the congestion level and it can further send the suggestion to the gNB-CU-UP. The suggestion can be contained in the ASSITANCE INFORMATION DATA frame. Corresponding CR for TS 38.425 can be found in our correlated contribution in [7].

gNB-CU-CP

gNB-CU-UP

gNB-DU

UE

1.configure DL PSI discard timer

2.Inform DU the DL PSI discard timer is configured

3.gNB-DU monitors the congestion status

4.gNB-DU sends suggestion to (de)activate the PSI based discard

Figure 1. option 1 for DL PSI based discard

- Option 2: Using the Desired buffer size for the data radio bearer and/or the Desired Data Rate contained in the DL DATA DELEVERY STATUS frame.

The two parameters in DDDS are helpful for flow control. For example, using the desired buffer size, the gNB-CU-UP can calculate the amount of data to be sent to the gNB-DU.

But the two parameters are not used for packets discard, and not for deducing the congestion suffered by the gNB-DU. Because the transmission of the buffered data can be postponed not only due to the congestion, but also related to the scheduling strategy. For example, if there are some data with higher priority or less remaining time from another DRB, the gNB-DU may prioritize the transmission of such data and delay the transmission for the considered DRB, resulting in a smaller desired buffer size/data rate for the next period of time. This does not mean the gNB-DU is congested. If the gNB-CU-UP activate DL PSI based discard based on this, unexpected packet loss will occur and the user experience will be affected. From another angle, the desired buffer size is highly related to the buffer allocation strategy among different DRBs and the buffer capability in the gNB-DU. If a gNB-DU’s buffer is small but the air interface status is good, the small value of the “desired buffer size” will cause the CU-UP’s wrong operation on activate the PSI based discard timer even if all DL packets can be scheduled immediately to the UE.

Furthermore, there is no clear guidance that the gNB-DU must send DDDS to the gNB-CU-UP in case of air interface congestion occurring. To make DDDS usable for DL PSI based discard, F1AP has to be enhanced to notify the gNB-DU that DL PSI based discard is configured, and correspondingly, the gNB-DU’s behaviour should be specified to make sure that DDDS would be sent in case of congestion. Besides, the description about how the receiving node use the information in DDDS should be added into the F1-U specification (TS 38.425). The required changes for this option are provided in Annex-1.

- Option 3: Using the DL Congestion Information contained in the ASSISTANCE INFORMATION DATA frame.

To support L4S or congestion information exposure for DL QoS flows in R18 XR, the gNB-DU can be configured to provide the DL congestion information to the gNB-CU-UP, so that the gNB-CU-UP can perform ECN marking for L4S or congestion information report to UPF accordingly.

However, the DL congestion information is only provided when ECN marking or congestion information report is configured for DL for the considered DRB, which actually relies on the mechanisms of L4S or congestion information exposure. In the mechanisms, it is up to the SMF to request the NG-RAN to perform ECN marking or congestion report. Upon receiving the request, the gNB-CU-CP can request the gNB-DU to provide the congestion information. It is not proper for DL PSI based discard to rely on such mechanism since L4S and congestion exposure are optional features.

The option 3 tries to reuse the congestion information defined in R18 L4S to support DL PSI based discard. Since the UL Congestion Information and/or DL Congestion information contained in F1-U may have different meaning depends on the requested type, it is unclear that whether a new meaning needs to be introduced if such congestion information is also used for the DL PSI based discard. To achieve such reusing, the F1AP specification still needs to be enhanced, so that the gNB-DU can be indicated to provide the information even without L4S or congestion information exposure required. In addition, the usage of the DL Congestion Information should be extended. And the description in TS 38.425 should be modified. The required changes for this option are provided in Annex-2.

Based on the above analysis, we think option 1 is more reliable and useful for DL PSI based discard. For option 2, the information provided by the gNB-DU may not reflect the congestion status correctly. For option 3, the DL congestion information is provided as a percentage. It may confuse the gNB-CU-UP on when to activate/deactivate the DL PSI based discard.

Moreover, from the specification perspective, all the three options have impacts to specifications. Even for option 2 and option 3, both the F1AP specification and F1-U specification need some change. But option 1 is the best way to address the DL PSI based discard issue, and the information from gNB-DU to CU-UP is the clearest.

Observation 2: The existing DDDS and DL Congestion Information in F1-U are not suitable to assist the gNB-CU-UP to perform DL PSI based discard.

Observation 3: Reusing the existing information to assist the gNB-CU-UP to perform DL PSI based discard also have impacts to both the F1AP and F1-U specifications.

With the above observations, we propose RAN3 to adopt option 1 for DL PSI based discard since it can provide efficient support while not introducing too much additional specification impacts.

Proposal 1: The gNB-CU-CP can notify the gNB-DU about per DRB DL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 2: The gNB-DU can send the suggestion on PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

3. Conclusion

In this contribution, the potential specification impacts of different ways to support DL PSI based discard are analyzed. The following proposals and observations are provided:

Observation 1: The gNB-CU-UP shall not activate DL PSI based discard when the network is not congested.

Observation 2: The existing DDDS and DL Congestion Information in F1-U are not suitable to assist the gNB-CU-UP to perform DL PSI based discard.

Observation 3: Reusing the existing information to assist the gNB-CU-UP to perform DL PSI based discard will also introduce impacts to both the F1AP and F1-U specifications.

Proposal 1: The gNB-CU-CP can notify the gNB-DU about per DRB DL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 2: The gNB-DU can send the suggestion on PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

4. Reference

[1]. Report of 3GPP TSG RAN3 meeting#123.

[2]. 3GPP TS 37.483: " NG-RAN; E1 application protocol (E1AP)".

[3]. 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]. 3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[5]. 3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[6]. R3-244519, (38.473 CR) Correction on DL PSI based discard.

[7]. R3-244520, (38.425 CR) Correction on DL PSI based discard.

Annex-1. TPs for option 2 (for information)

TP for TS 38.473

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE and the value is set as "configured", the gNB-DU shall, if supported, take it into account to send the DL DATA DELIVERY STATUS frame in presence of congestion.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "configured", the gNB-DU shall, if supported, take it into account to send the DL DATA DELIVERY STATUS frame in presence of congestion.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-LTMCells-ToBeReleased-Item,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBandwidthAggregationRequestIndication,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PSIbasedSDUdiscardDL,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

------ Unchanged part skipped ------

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardDL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardDL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

------ Unchanged part skipped ------

PDUSetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }	OPTIONAL

}

PDUSetQoSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

PSIbasedSDUdiscardDL ::= ENUMERATED {configured, not-configured, ...}

------ Unchanged part skipped ------

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------ Unchanged part skipped ------

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-PSIbasedSDUdiscardDL								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

CHANGES END

TP for TS 38.425

CHANGE STARTS

5.4.2	Downlink Data Delivery Status

5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer. The corresponding node may also transfer uplink user data for the concerned data radio bearer to the node hosting the NR PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the successful delivery of DL control data to the corresponding node.

The Downlink Data Delivery Status procedure is also used to provide the congestion status from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to perform DL PSI based discard if supported.

When the corresponding node decides to trigger the feedback for Downlink Data Delivery procedure it shall report as specified in section 5.2:

a)	in case of RLC AM, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

NOTE 1:	If the NR user plane protocol instance is associated a single RLC-AM entity for an MRB, specification text in bullet a) is applicable.

For all other cases, if the NR user plane protocol instance is associated with an MRB configured with at least one RLC AM entity and RLC-UM, the highest successfully delivered NR PDCP sequence number indicates the combined feedback of the highest NR PDCP sequence number successfully delivered in sequence to all the involved UEs for which the RLC AM entites have been configured, no retransmissions are performed, and the highest NR PDCP sequence number transmitted to the lower layers for PTM.

b)	the desired buffer size in bytes for the concerned data radio bearer or for the MRB;

c)	optionally, the desired data rate in bytes associated with a specific data radio bearer configured for the UE or for the MRB;

d)	the NR-U packets that were declared as being "lost" by the corresponding node and have not yet been reported to the node hosting the NR PDCP entity within the DL DATA DELIVERY STATUS frame;

e)	if retransmission NR PDCP PDUs have been delivered, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs successfully delivered to the UE in sequence of NR-U sequence number;

f)	if retransmission NR PDCP PDUs have been transmitted to the lower layers, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs transmitted to the lower layers in sequence of NR-U sequence number;

g)	the highest NR PDCP PDU sequence number transmitted to the lower layers among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

NOTE 2:	If the NR user plane protocol instance is associated with an MRB configured with RLC-UM entities only, the highest NR PDCP PDU sequence number transmitted successfully to all lower layer instances  is reported.

NOTE 3:	If a deployment has decided not to use the Transfer of Downlink User Data procedure, d), e) and f)  above are not applicable.

h)	in case of RLC AM, the NR PDCP PDU sequence number successfully delivered out of sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs.

As soon as the corresponding node detects the successful RACH access by the UE for the corresponding data radio bearer(s), the corresponding node shall send initial DL DATA DELIVERY STATUS frame to the node(s) hosting the NR PDCP entity(ies). The node hosting NR PDCP entity may start sending DL data before receiving the initial DL DATA DELIVERY STATUS frame. In case the DL DATA DELIVERY STATUS frame is sent before any NR PDCP PDU is transferred to lower layers, the information on the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE and the highest NR PDCP PDU sequence number transmitted to the lower layers may not be provided.

The DL DATA DELIVERY STATUS frame shall also include a final frame indication when this frame is the last DL status report. When receiving such indication, the node hosting the NR PDCP entity considers that no more UL or DL data is expected to be transmitted between the corresponding node and the UE.

The DL DATA DELIVERY STATUS frame may also include an indication of detected radio link outage or radio link resume for the concerned data radio bearer. When receiving an indication of radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node both in UL and DL. When receiving an indication of radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node both in UL and in DL. When receiving an indication of UL or DL radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node for UL or DL, depending on the indicated outage. When receiving an indication of UL or DL radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node in UL or in DL, depending on the indicated resume. These indications are not applicable to E-UTRA PDCP.

For report polling triggered reporting, the DL DATA DELIVERY STATUS frame may include the feedback delay result and the NR-U sequence number of the frame where Report Polling Flag is included and that triggered the signalling of the DL DATA DELIVERY STATUS.

The node hosting the NR PDCP entity, when receiving the DL DATA DELIVERY STATUS frame:

-	regards the desired buffer size under b) and the data rate under c) above as the amount of data to be sent from the hosting node:

-	If the value of the desired buffer size is 0, the hosting node shall stop sending any data per bearer.

-	If the value of the desired buffer size in b) above is greater than 0, the hosting node may send up to this amount of data per bearer:

-	first including the data to be retransmitted;

-	then the new data, starting from the last "Highest successfully delivered NR PDCP Sequence Number" for RLC AM if received, or starting from the last "Highest transmitted NR PDCP Sequence Number" for RLC UM if received.

-	The value of the desired data rate in c) above is the amount of data desired to be received in a specific amount of time. The amount of time is 1 sec.

-	The information of the buffer size in b) above and of the data rate in c) above is valid until the next DL DATA DELIVERY STATUS frame is received.

-	is allowed to remove the buffered NR PDCP PDUs of a RLC AM bearer, according to the feedback of successfully delivered NR PDCP PDUs;

-	decides upon the actions necessary to take for NR PDCP PDUs reported other than transmitted and/or successfully delivered.

-	decides to activate or deactivate DL PSI based discard for the concerned data radio bearer, if supported, by taking the desired buffer size in b) and the data rate in c) into account.

In case of RLC AM, after the highest NR PDCP PDU sequence number successfully delivered in sequence is reported to the node hosting the NR PDCP entity, the corresponding node removes the respective NR PDCP PDUs. For RLC UM, the corresponding node may remove the respective NR PDCP PDUs after transmitting to lower layers.

Figure 5.4.2.1-1: Successful Downlink Data Delivery Status

CHANGE ENDS

Annex-2. TPs for option 3 (for information)

TP for TS 38.473

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE and the value is set as "configured", the gNB-DU shall, if supported, take it into account to provide the DL Congestion Information for the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "configured", the gNB-DU shall, if supported, take it into account to provide the DL Congestion Information for the indicated DRB.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-LTMCells-ToBeReleased-Item,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBandwidthAggregationRequestIndication,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PSIbasedSDUdiscardDL,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

------ Unchanged part skipped ------

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardDL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardDL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

------ Unchanged part skipped ------

PDUSetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }	OPTIONAL

}

PDUSetQoSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

PSIbasedSDUdiscardDL ::= ENUMERATED {configured, not-configured, ...}

------ Unchanged part skipped ------

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------ Unchanged part skipped ------

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-PSIbasedSDUdiscardDL								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

CHANGES END

TP for TS 38.425

CHANGE STARTS

5.4.3	Transfer of Assistance Information

5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or to further send it to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

The ASSISTANCE INFORMATION DATA frame may include DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity may, if supported, take this information into account to activate or deactivate DL PSI based discard.

Figure 5.4.3.1-1: Successful Transfer of Assistance Information Data

CHANGE ENDS