3GPP TSG-RAN WG3 Meeting #125	R3-244526

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	Discussion on the Architecture, Access control and QoS support of WAB

Document for:	Discussion

1	Introduction

In previous meetings, the following agreements were achieved for the architecture design of Rel-19 WAB [1]:

RAN3 123bis:

The study focuses on the use of WAB-MT´s PDU session via NR Uu as backhaul of WAB gNB. Other options for the backhaul (including non-3GPP radio technology) are not precluded but not a part of the study.

It is agreed that the study is focused on NR-Uu backhaul

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

A WAB-gNB cannot serve WAB MTs.

The WAB-MT supports at least a subset of UE functionalities.

The NR-Uu backhaul link does not include the IP layer. The IP layer is included in the NG-U and NC-C. Amend Figure X.Y.3 by correcting the dashed vertical lines.

The scope of the study is limited to PDU Session Backhauling. Other options are not precluded but not treated in this study.

For Figure X.Y removing the white box and making the IP bold line to terminate at the WAB box.

RAN3 124:

Adopt the following terminology:

BH-RAN-node: This is an NG-RAN node serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-5GC: The 5GC serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

UE´s 5GC: The 5GC connected to the WAB-gNB and serving the UEs.

UE´s AMF: The AMF connected to the WAB-gNB and serving the UEs.

UE´s UPF: The UPF connected to the WAB-gNB and serving the UEs.

In this paper, we further analyse the scenario and architecture, multi-hop avoidance, and QoS support in the WAB scenario.

2	Discussion

2.1	Scnario and architecture

In previous meeting, RAN3 agreed to study both in-band deployment and out-band deployment for WAB.  For in-band scenario, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated. For example, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the “Hard” resource configuration of the WAB-gNB. Such resource coordination needs to be performed very frequently between the WAB-node and the current BH-RAN node, considering the WAB-node is moving and the BH-RAN node is changing.  In addition, if the resource configuration of WAB-node changes, the resource coordination needs to be performed again.

For out-band scenario, the access link and backhaul link will use different frequency band, and there is no interference between the two links, so no coordination is needed between access and backhaul link, the operation will become very simple compared to the in-band mode.

Therefore, for the wide deployement of WAB node, out-band should be prioritized. And if the in-band mode occurs at a time, e.g., due to WAB mobility the WAB-MT switch to a serving cell with the same frequency as the collocated WAB-gNB, the BH-RAN node should be an upgraded gNB to be aware of the WAB and can support the WAB resource multiplexing. It is proposed:

Proposal 1: For WAB deployment, out-band should be prioritized.

Proposal 2: For in-band WAB deployment, the BH-RAN must be upgraded to be aware of WAB and support the resource multiplexing.

2.2	Access control: Multi-hop avoidance

2.2.1	Initial access

It was agreed that “A WAB-gNB cannot serve WAB MTs.” The WAB-MT should be able to acquire whether a cell is allowed for the WAB-MT to access. We can consider the following three options:

- Option 1: Broadcasting “WAB-barred”. The WAB-node consider the cell is barred if the SIB1 in this cell contains “WAB-barred” indication, otherwise, the WAB-node can consider this cell is accessable. With such option, the WAB-gNB should broadcast the “WAB-barred” indication to prevent other WAB-MT from accessing it. Option 1 allows the WAB-MT to access the legacy gNBs.

- Option 2: Broadcasting “WAB-support”. Similar as the access control for IAB-MT, the gNB which is capable for the WAB-MT accessing indicates a “WAB-support” IE in SIB1, and the WAB-gNB does not broadcast such indicator since a WAB-gNB cannot serve any WAB-MTs. The WAB-MT only accesses the cell broadcasting the “WAB-support” indicator. In option 2 the WAB-MT cannot access a legacy gNB but only access the upgraded BH-gNBs.

- Option 3: CN-based solution. WAB-gNB provides WAB-indication to AMF (proposed by SA2’s TR 23.700-06, section 6.2.3.6 [2]), and WAB-MT’s AMF reject a WAB-MT’s registration request if the WAB-MT accesses a cell served by WAB-gNB. Option 3 allows WAB-MT access legacy cells by default, but if it accesses a WAB-gNB cell, it takes long time for the situation awareness and reject. Moreover, after the registration failure, the WAB-MT may still choose another WAB-gNB to access because such solution can not avoid wrong choice from the WAB-MT.

Observation 1: Option 1 and option 2 can ensure the WAB-MT select a proper BH-gNB which supports WAB-MT, and can prevent the WAB-MT from accessing a WAB-gNB at early stage.

Observation 2: Option 1 allows the WAB-node to access a legacy gNB by default, while option 2 requires the BH-gNB to be upgraded to support WAB.

Observation 3: Using option 3, the WAB-MT will access another WAB-node successfully but then the registration procedure will be rejected. Such solution cannot prevent the WAB-MT to access another WAB-gNB again.

Based on the introduction and comparison, we can see that option 1 and option 2 are better than option 3, it is proposed:

Proposal 3: For multi-hop avoidance, RAN3 to discuss the following options:

- Option 1: Broadcasting “WAB-barred” solution. WAB-node is barred to a cell if the SIB1 in this cell contains “WAB-barred” indication. A WAB-gNB should broadcast “WAB-barred” in SIB1.

- Option 2: Broadcasting “WAB-support” solution. WAB-node only access the cell which broadcasts “WAB-support” in SIB1. A WAB-gNB will not broadcast “WAB-support”.

2.2.2	Handover

During WAB-MT handover, the WAB-MT should not switch to a target WAB cell. The following two options can be considered:

- Option 1: via explicit indication. The source BH-RAN includes the WAB-node indication in the HO request, then the target BH-RAN node can perform access control for this WAB-node. The WAB-node indication should be introduced in both Xn HO REQUEST and NG HO REQUEST. If the WAB can has some specific slice ID, it is also possible to use the S-NSSAI as an indication of WAB-node during handover.

- Option 2: via UE capability. If the target gNB is a WAB-gNB, the target WAB-gNB reject the WAB-node when receiving the UE capability information in the HO REQUEST for this WAB-MT, which contains information to indicate a UE is a WAB-MT. This solution is not feasible because the UE capability cannot be used for admission control during HO procedure. For example, a R18 UE can access R17 gNB even if some R18 UE capability cannot be supported by the legacy gNB.  In addition, this solution requires RAN2 to define WAB-specific UE capability, but it is unclear whether the WAB-MT will have some WAB-specific UE capability. Moreover, if a WAB-MT can work as a normal UE but not a WAB-node, it should be allowed to access another WAB-gNB, but such solution will result in that the “normal UE” being rejected by the target WAB-gNB.

Based on the above analysis, option 2 may bring some implementation requirements for reducing its advantage. It is proposed:

Proposal 4: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. RAN3 to discuss whether to introduce a new explicit indicator or use a special S-NSSAI value as the indicator.

2.3	QoS support

2.3.1	QoS parameter determination

1) PDB determination for WAB-MT’s QoS flow

For a WAB-node, the WAB-MT establishes one or more PDU sessions to provide the backhaul transmission for UE’s traffic, and one or more DRBs are established between the WAB-MT and the serving gNB. Traditionally, the gNB establishes DRB for the served UE according to the UE’s required QoS parametes sent from the core network. In the WAB scenario, it is important for the WAB-MT’s core network to indicate a proper QoS parameter of the WAB-MT’s QoS flow to the BH-gNB, for satisfying the E2E QoS parameters of the UE.

Taking the PDB as an example, as described in Fig. 1, the BH-gNB should be indicated a proper PDB of MT’s QoS flow, i.e., PDB 1 + PDB 2, for satisfying the UE’s E2E PDB, i.e., PDB 0 + PDB 1 + PDB 2 + PDB 3. To make that possible, the MT’s CN should know the CN PDB requirement between the WAB-gNB and the UE’s UPF, i.e., PDB 1 + PDB 2 + PDB 3.

MT’s CN may derive PDB3 based on measurement of N6 latency, then, it is easy to get the proper PDB for MT’s QoS flow, with the knowledge of CN PDB between the WAB-gNB and the UE’s UPF.

Fig 1. PDB split model in WAB

It is proposed:

Proposal 5-1:	To determine a proper PDB of WAB-MT’s QoS flow satisfying the UE’s E2E PDB, the WAB-MT’s CN should know the CN PDB requirement between the WAB-gNB and the UE’s UPF.

2) QoS parameter setting for UE’s traffic

In general, the UE’s QoS flow/DRB can be set up according to legacy procedures. Specially, NTN backhaul is considered between the WAB-MT and the BH-gNB, as expected in the WID. According to TS 23.501 [3] the UE’s 5QI is set to a fixed value (5QI=10) as long as the UE is served by NTN, with the PDB directly set to a really large value (1100ms).

Fig 2. Special QoS parameters for NTN UE in TS 23.501

And, according to the TS 38.413, the gNB will provide the RAT Information to AMF during the NG Setup procedure or the RAN Configuration Update procedure.

9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) |  |

Therefore, if the WAB-MT adopts NTN backhaul, the UE’s CN should know that and may also configure the NTN related fixed QoS parameters for UE’s QoS flow to the WAB-gNB. Otherwise, the UE’s CN may still determine other 5QI for the UE’s QoS which requires less PDB but cannot supported by the network indvolving WAB-gNB using NTN backhaul.

Proposal 5-2:	If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN for seting proper QoS parameter of UE’s traffic.

2.3.2	QoS mapping

For the DL, there is no special issue for the BH-gNB and WAB-gNB finding the proper DRB, since the gNBs directly see the QoS flows for the WAB-MT and the UE, respectively. The WAB-MT’s UPF can also map the UE’s traffic to the MT’s proper QoS flow based on the current PDR (Packet Detection Rule) mechanism, since the DL UE traffic received by the MT’s UPF is just seen as the MT’s normal data.

Proposal 5-3:	In DL, the WAB-MT’s UPF maps the traffic to WAB-MT’s QoS Flows based on the configured DL PDRs, same as legacy.

Fig 3. QoS mapping model for WAB-node

For the UL, it is worth discussing how the WAB-MT can select a proper DRB in its wireless backhaul link for the uplink UE traffic. The candidate solutions for the WAB-MT to map the uplink UE traffic to the established DRB are listed as follows:

- Sol#1: Based on the QoS parameters mapping

Accoring to TS 23.502 [4], the AMF can send the PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node when the PDU session requested by the UE is established successfully. The PDU SESSION RESOURCE SETUP REQUEST message includes the QoS parameters (e.g. 5QI) and a NAS-PDU containing the N1 SM message, i.e. PDU Session Establishment Accept massage. This N1 SM message sent to the UE transparently may comprise the QoS parameters as well. It is obvious that the UE and the NG-RAN node can both obtain the QoS parameters.

From above, the WAB-gNB acting as a NG-RAN node can know the QoS parameters for each QoS flow of the access link while the WAB-MT acting as a UE can know the QoS parameters for the QoS flow at the backhaul link. As a result, the WAB-node can map the uplink UE traffic to the WAB-MT’s QoS flow satisfying the service quality based on these two QoS parameters, and then the WAB-MT can map its QoS flow to the UL DRB at the backhaul link using existing functionality provided by the SDAP layer.

- Sol#2: Based on 5QI to DSCP mapping configured via OAM

Similar to the LTE relay, the WAB-gNB can be configured with the 5QI to DSCP mapping table. For the uplink UE traffic, the WAB-gNB can mark a related DSCP for the IP header of the NG-U protocol based on this mapping table. Then the WAB-MT can use its UL TFT configured by the serving gNB to filter the marked DSCP and map the IP packets to the UL QoS flow associated with the PDU session backhaul, and then the WAB-MT can map its QoS flow to the UL DRB at the backhaul link using existing functionality provided by the SDAP layer.

- Sol#3: Based on the network slice mapping

In the 5G system, a DRB shall associate with a network slice. Thus, the WAB-node can map UE’s DRB to the WAB-MT’s DRB considering the associated slice of the two DRBs. For example, if the WAB-node receives UE’s UL traffic from UE’s DRB1, and this DRB1 is associated with slice#1 in the access link, the WAB-node can map the UE’s DRB1 to the WAB-MT’s DRB 2 which is associated with slice#2 between the WAB-MT and the serving gNB as long as the slice#2 can meet the service requirement of slice#1.

According to SA2’s conclusion in clause 8.1 of [2], the following sentence is related to the QoS.  “If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is then used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.” Our understanding on such conclusion is exactly match the Sol #2, we would like to suggest RAN3 to align with SA2’s conclusion on the QoS mapping.

Hence, it is proposed that,

- Proposal 5-4: RAN3 to adopt the following solution on UL QoS mapping for the WAB-node, which also align with SA2’s conclusion:

- - Sol#2: Based on 5QI to DSCP mapping configured via OAM

3	Conclusion

In this contribution we discuss the architecture issues, multi-hop avoidance, and QoS support in the WAB scenario.

Scenario and architecture

Proposal 1: For WAB deployment, out-band should be prioritized.

Proposal 2: For in-band WAB deployment, the BH-RAN must be upgraded to be aware of WAB and support the resource multiplexing.

Multi-hop avoidance

Observation 1: Option 1 and option 2 can ensure the WAB-MT select a proper BH-gNB which supports WAB-MT, and can prevent the WAB-MT from accessing a WAB-gNB at early stage.

Observation 2: Option 1 allows the WAB-node to access a legacy gNB by default, while option 2 requires the BH-gNB to be upgraded to support WAB.

Observation 3: Using option 3, the WAB-MT will access another WAB-node successfully but then the registration procedure will be rejected. Such solution cannot prevent the WAB-MT to access another WAB-gNB again.

Proposal 3: For multi-hop avoidance, RAN3 discuss the following options:

- Option 1: Broadcasting “WAB-barred” solution. WAB-node is barred to a cell if the SIB1 in this cell contains “WAB-barred” indication. A WAB-gNB should broadcast “WAB-barred” in SIB1.

- Option 2: Broadcasting “WAB-support” solution. WAB-node only access the cell which broadcasts “WAB-support” in SIB1. A WAB-gNB will not broadcast “WAB-support”.

Proposal 4: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. RAN3 to discuss whether to introduce a new explicit indicator or use a special S-NSSAI value as the indicator.

QoS support issue

Proposal 5-1:	To determine a proper PDB of WAB-MT’s QoS flow satisfying the UE’s E2E PDB, the WAB-MT’s CN should know the CN PDB requirement between the WAB-gNB and the UE’s UPF.

Proposal 5-2:	If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN for seting proper QoS parameter of UE’s traffic.

Proposal 5-3:	In DL, the WAB-MT’s UPF maps the traffic to WAB-MT’s QoS Flows based on the configured DL PDRs, same as legacy.

- Proposal 5-4: RAN3 to adopt the following solution on UL QoS mapping for the WAB-node, which also align with SA2’s conclusion:

- - Sol#2: Based on 5QI to DSCP mapping

4	References

- 3GPP RAN3 124 Chairman notes

- TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, V1.0.0

- TS 23.501, System architecture for the 5G System (5GS), Stage 2, V18.5.0

- TS 23.502, Procedures for the 5G System (5GS), Stage 2, V18.5.0

Annex:	TP for TR 38.799

CHANGES START

4.3	Operational aspects

>>>>>>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<<<<<<

4.3.X QoS support

4.3.X.1 QoS parameter determination

In the WAB scenario, it is important for the WAB-MT’s core network indicates a proper QoS parameter of the WAB-MT’s QoS flow to the BH-gNB, for satisfying the E2E QoS parameters of the UE. Taking the PDB as an example, as described in Figure 4.3.X.1-1, the BH-gNB should be indicated a proper PDB of WAB-MT’s QoS flow, i.e., PDB 1 + PDB 2, for satisfying the UE’s E2E PDB, i.e., PDB 0 + PDB 1 + PDB 2 + PDB 3. To make that possible, the MT’s CN should know the CN PDB requirement between the WAB-gNB and the UE’s UPF, i.e., PDB 1 + PDB 2 + PDB 3.

MT’s CN may derive PDB3 based on measurement of N6 latency, then, it is easy to get the proper PDB for MT’s QoS flow, with the knowledge of CN PDB between the WAB-gNB and the UE’s UPF.

Figure 4.3.X.1-1. PDB split model in WAB

If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN for seting proper QoS parameter of UE’s traffic.

4.3.X.2 QoS mapping

For the DL, the WAB-MT’s UPF can map the UE’s traffic to the MT’s proper QoS flow based on the current PDR (Packet Detection Rule) mechanism, since the DL UE traffic received by the MT’s UPF is just seen as the MT’s normal data.

For the UL, the WAB-gNB can be configured with the 5QI to DSCP mapping table by OAM. For the uplink UE traffic, the WAB-gNB can mark a related DSCP for the IP header of the NG-U protocol based on this mapping table. Then the WAB-MT can use its UL TFT configured by the serving gNB to filter the marked DSCP and map the IP packets to the UL QoS flow associated with the PDU session backhaul, and then the WAB-MT can map its QoS flow to the UL DRB at the backhaul link using existing functionality provided by the SDAP layer.

NEXT CHANGE

4.3.Y Multi-hop avoidance

4.3.Y.1 Multi-hop avoidance during intial access

A WAB-MT should not access another WAB-gNB. To avoid the WAB-MT select another WAB-gNB as the BH-gNB, the following options can be considered:

- Option 1: Broadcasting “WAB-barred”. The WAB-node consider the cell is barred if the SIB1 in this cell contains “WAB-barred” indication, otherwise, the WAB-node can consider this cell is accessable. With such option, the WAB-gNB should broadcast the “WAB-barred” indication to prevent other WAB-MT from accessing it. Option 1 allows the WAB-MT to access the legacy gNBs.

- Option 2: Broadcasting “WAB-support”. Similar as the access control for IAB-MT, the gNB which is capable for the WAB-MT accessing indicates a “WAB-support” IE in SIB1, and the WAB-gNB does not broadcast such indicator since a WAB-gNB cannot serve any WAB-MTs. The WAB-MT only accesses the cell broadcasting the “WAB-support” indicator. In option 2 the WAB-MT cannot access a legacy gNB but only access the upgraded BH-gNBs.

- Option 3: CN-based solution. WAB-gNB provides WAB-indication to AMF, and WAB-MT’s AMF reject a WAB-MT’s registration request if the WAB-MT accesses a cell served by WAB-gNB. Option 3 allows WAB-MT access legacy cells by default, but if it accesses a WAB-gNB cell, it takes long time for the situation awareness and reject. Moreover, after the registration failure, the WAB-MT may still choose another WAB-gNB to access because such solution can not avoid wrong choice from the WAB-MT.

4.3.Y.2 Multi-hop avoidance during WAB mobility

In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. If the target BH-RAN node is WAB-gNB, it should reject the handover request for the WAB-node.

NEXT CHANGE

4.5 Conclusions for WAB

As an outcome of the study, it is recommended to prioritize the out-band deployment during the normative phase.

To avoid multi-hop in WAB, the RAN based broadcasting solution to be adopted, then the WAB-MT can select a proper BH-RAN node based on the broadcasted information. In mobility case, the target BH-RAN node can perform access control for the WAB-node based on explicit indication.

For the QoS support, R3 recommends the following:

- WAB-MT’s CN should know the CN PDB requirement between the WAB-gNB and the UE’s UPF.

- If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN for seting proper QoS parameter of UE’s traffic.

- UL QoS mapping for the WAB-node should based on the 5QI to DSCP mapping configured via OAM.

CHANGES END