3GPP TSG-RAN WG3 Meeting #125	R3-244527

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	Discussion on WAB mobility and some remaining issues

Document for:	Discussion

1	Introduction

The previous RAN3 meeting achieved the following agreements for the mobility and authorization issues of WAB [1]:

WAB Authorization

RAN3 understands that authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

WAB-MT authorization provides the WAB MT with the right to support BH PDU sessions.

In case MT authorization is based on slice, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed. SA2 to decide whether MT authorization based on slicing is supported.

Authorization of WAB-gNB provides service authorization, i.e., the right to serve UEs.

It is RAN3 understanding that WAB-gNB’s authorization status can change.

The UEs served by a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released.

The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. Any Stage 3 decisions are to be taken during normative work.

WAB mobility

Support the following scenarios for intra-PLMN WAB-node mobility:

- UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN.

- UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN.

- BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN.

- BH UPF/AMF changes as WAB-MT moves inside a PLMN.

Legacy mobility procedures can be supported for WAB-MT.

RAN3 assumes that roaming scenarios for WAB-MT are supported.

Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.
Addition of new information in the legacy procedures is not precluded

Upon WAB-gNB mobility:

If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB.

This contribution further dicusses on the mobility, authorization, and resource multiplexing issues for WAB.

2	Discussion

2.2	WAB-node mobility support

2.2.1 How to support UE’s CN change during the mobility of WAB-node

The physical location of UEs served by a WAB-node changes as the WAB-node moves, and if the UEs’ location changes at a wide range, the UEs’ serving AMF needs to be changed based on the current location.

---------Excerpted from TR 23.700-06 v 19.0.0--------------

8.4	KI#4: Efficient mobility and service continuity when served by MWAB Conclusion

The following principles are agreed to be used as the basis for normative work:

-	There is no requirement to change the UE mobility behaviour for UEs served by a MWAB-gNB.

-	MWAB-gNB's configurations (e.g. Cell ID/TAC, AMF address, etc.) are managed by OAM of the PLMN it serves, based on the location information of the MWAB.

>>>>>>>>>>some parts are skipped<<<<<<<<<<

-	It is assumed that when MWAB-gNB changes the AMF it connects due to mobility, the TAC(s) and Cell ID(s) it announces also change.

>>>>>>>>>>some parts are skipped<<<<<<<<<<

-	The MWAB instantiates a new logical MWAB gNB when it is configured with a new TAC (and possibly Cell ID ) and with a different gNB ID. This may be used to support migrating / handing over of UEs served by the MWAB to the new logical gNB. Once all UEs have been migrated, the MWAB removes the old logical MWAB-gNB instance. This can happen e.g. when AMF, serving the UEs of the MWAB, change is required based on the operator policy.

NOTE 4:	This aspect will be coordinated with RAN WG3. Whether the gNB ID will be changed will be reviewed in normative phase based on RAN WG3 input taking into account potential AMF impacts.

NOTE 5:	The solution based on instantiating multiple virtual cells by the same MWAB-gNB needs to wait for RAN WG3 confirmation.

SA2 discussed how to support this AMF change during their SI phase for MWAB, according to the conclusion part in 23.700-06, they will rely on RAN3’s input on how to achieve the AMF change.

In the last RAN3 meeting, companies discussed the solutions for how to support the UE’s AMF change during the WAB-node mobility. The following 3 options are on the table:

- Option 1: inter-gNB handover.

In this solution, two logical gNBs are activated simultaneously in the WAB-node. The source logical gNB and target logical gNB respectively connects to the old AMF and new AMF. Since the two logical gNBs connect to different AMFs, the UE handover between the cells served by the two logical gNBs should be NG-based handover, and the serving AMF is re-located during the NG-based handover.

- Option 2: intra-gNB handover.

Two sub-options exist in the intra-gNB handover based solution. The first is that the intra-gNB inter-cell handover could be enhanced to allow triggerring the NG-based handover, as illustrated in paper [4]. The second is the WAB-gNB activates two cells broadcasting different TACs, and after the legacy intra-gNB handover, UE’s mobility registration update procedure will be triggered to change the AMF for UE. Specifically, the target cell’s broadcasting TAC should be only maintained by the target AMF but not maintained by the source AMF, and should be out of the UE’s RA scope.

- Option 3: intra-cell information update without handover.

In this solution, there is no need to activate new cells on the WAB-gNB. The WAB-gNB connects to two AMFs simutaneously, and the serving cell directly change the broadcasted system information (e.g., TAC, NR cell identity, etc.) from the old value corresponding to the old AMF to the new value corresponding to the new AMF. Then the WAB-gNB sends short message in PDCCH to UE for the system information change notification, and UE will read the new SIB1 and detects the new TAC value. After that, the UE will trigger the mobility registration update when detecting new TAC out of its RA scope, and the AMF change will be achieved during the mobility registration update procedure.

In our view, the handover-based solutions require that both the source cell and target cell should be visible to the UE at the same time, including option 1 and option 2. Then, it means that the two cells should share the resources allocated to this WAB-node, including the frequency resources, RF resources, baseband resources, etc. This will result in low resource utilization because only half of the available resources can be utilized by UE’s serving cell since the WAB-node needs to activate at least two cells at same time.

And considering during WAB-node moving the UE’s connected gNB does not actually change, handover is not actually needed, as long as the AMF re-location can be triggered.

Therefore, option 3 seems to be the best solution, which does not require resource partition between two cells, and can be supported by current messages and procedures.

----------------Excerpted from R3-243844------------

Q1. If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. Is this needed for AMF relocation?

>>>>>>>>>>detailed response from companies are skipped<<<<<<<<<<<<

Summary: In case TAC change, it seems MRU can be used. In case OAM can simply configure the same TAC for both AMFs, it is necessary to further check whether the scenario is valid. RAN3 to further discuss this issue in the next meeting.

According to the offline discussion in RAN3-124 meeting, the summary regarding to whether to handover UE between two cells is needed for AMF relocation is copied below. According to the summary, the only issue for option 3 is it may not work for the scenario that the two AMFs share same TAC value, and the FFS part is whether such scenario is valid. From the conclusion part in [2], it is clearly stated that “It is assumed that when MWAB-gNB changes the AMF it connects due to mobility, the TAC(s) and Cell ID(s) it announces also change”. So, it seems the scenario is not valid for AMF change case.

Observation 1-1：If UE perfroms handover between two cells served by the WAB-node to support AMF relocation, the available resouces needs to be shared between the two cells and will leads to low resource utilization.

Observation 1-2：It was concluded that the MRU procedure is enough to support AMF change in case the TAC of WAB-gNB’s cell change, while SA2 has concluded that the TAC broadcasted by the WAB-gNB will change if the AMF changes.

The workflow of option 3 is given by Fig 1, and it is proposed:

- If UE’s AMF changes as the WAB-gNB moves, the UE served by WAB-node should perform mobility registration update to enable the AMF re-location, based on detecting new TAC broadcasted by the WAB-gNB. No need to activate new cells on the WAB-gNB.

Fig 1. UE’s AMF change based on the MRU without handover

In the Annex, we propose the TP to TR 38.799 for capturing the option 3.

2.2.2 Neighboring relationship update for WAB-gNB

The WAB-node mobiliy will result in the dynamic change of the neighboring relationship for the WAB-gNB. Thus, it is essential for the WAB-gNB to update the neighboring relations timely in order to support the UE mobility via the Xn connection.

Legacy ANR mechanism requires UE to be present in the overlap area of neighbour cells, and may be not be a time-effictive way in the WAB scenario where the cell neighboring relationship changes fast.

An alternative solution is that the BH-gNB can help for the WAB-gNB neighboring relationships determination based on its own neighboring relationships. For example, the BH-gNB can provide its own neighbour cells’ information to the WAB-node via RRC signalling or Xn signalling.

- During the mobility of WAB node, the neighboring relationships for the WAB-gNB can be provided to the WAB-node by the BH-gNB.

2.3	WAB-node authorization

Since the authorization for the WAB-MT and WAB-gNB should be performed separately, we also need to consider two cases for the non-authorized handling, i.e., one case is WAB-MT becomes not authorized and another case is WAB-gNB is not authorized.

On one hand, the WAB-MT cannot continue to provide backhaul transmission for the WAB-gNB when the WAB-MT becomes non-authorized, so that the UP resources for the established PDU session(s) should be released. On the other hand, if the WAB-gNB is non-authorized, the WAB-gNB should not work as a gNB, e.g., the WAB-gNB needs to handover the UEs to other gNB, or release the NG/Xn connection. Under this circumstance, it is meaningless for the WAB-MT to maintain the UP resources because there is no CP/UP traffic from the WAB-gNB. Hence, the WAB-MT is required to inform the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time, then the UP resources for the established BH PDU session(s) of WAB-MT can be released.

- The UP resources for the established BH PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

2.4	Resource multiplexing and energy saving for WAB-node

For out-band mode the WAB-gNB and WAB-MT use different frequency, so that there is no interference between the WAB-gNB and the WAB-MT. For in-band mode, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated. For example, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the “Hard” resource configuration of the WAB-gNB.

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- Similar as IAB system, the BH-gNB should know the multiplexing capabilities of WAB-node, and the resource configuration of WAB-gNB. RAN3 to study Xn signalling for WAB-node resource coordination in normative phase.

Furthermore, as the Rel-18 introduced the cell DTX/DRX energy saving mechanism, the serving cell of the WAB-MT may be configured cell level DTX/DRX. It is meaningful the WAB-gNB set its own cells’ DTX/DRX referring to the serving cell’s DTX/DRX, e.g., it would be beneficial for the energy saving and reducing the storage cost that the WAB-gNB’s cells also turn to inactive when the seving cell becomes inactive. Since the cell DTX/DRX pattern is activated by L1 signalling, reporting and configuring via Xn looks not time efficient for such dynamics, and it is reasonable the WAB-gNB directly set its cells’ DTX/DRX pattern and/or activation status according to that of the serving cell. It is proposed,

- If WAB-node supports energy saving, the WAB-gNB determines the cell DTX/DRX based on the WAB-MT serving cell DTX/DRX.

3	Conclusion

This paper provides mobility and other issues for the WAB-node. The proposals are as follows:

WAB-node mobility support

Observation 1-1：If UE perfroms handover between two cells served by the WAB-node to support AMF relocation, the available resouces needs to be shared between the two cells and will leads to low resource utilization.

Observation 1-2：It was concluded that the MRU procedure is enough to support AMF change in case the TAC of WAB-gNB’s cell change, while SA2 has concluded that the TAC broadcasted by the WAB-gNB will change if the AMF changes.

- If UE’s AMF changes as the WAB-gNB moves, the UE served by WAB-node should perform mobility registration update to enable the AMF re-location, based on detecting new TAC broadcasted by the WAB-gNB. No need to activate new cells on the WAB-gNB.

- During the mobility of WAB node, the neighboring relationships for the WAB-gNB can be provided to the WAB-node by the BH-gNB.

WAB-node authorization

- The UP resources for the established PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

Resource multiplexing and energy saving for WAB-node

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- Similar as IAB system, the BH-gNB should know the multiplexing capabilities of WAB-node, and the resource configuration of WAB-gNB. RAN3 to study Xn signalling for WAB-node resource coordination in normative phase.

- If WAB-node supports energy saving, the WAB-gNB determines the cell DTX/DRX based on the WAB-MT serving cell DTX/DRX.

4. Reference

- RAN3#124 chairman notes.

- 3GPP TR 23.700-06 v 19.0.0, Study on architecture enhancements for vehicle-mounted relays - Phase 2.

- R3-243844, Summary of Offline Discussion on additional topological enhancement, NTT DOCOMO (moderator).

- R3-243327, (TP to TR 38.799) Discussion on impact of WAB, CATT, RAN3 #124

5. Annex——TP for TR 38.799 V1.0.0

Start of Change

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed. In addition, the WAB-MT will inform the BH-5GC that the WAB-gNB is “not authorized”, and the UP resources for the established BH PDU session(s) should be released by the BH 5GC.

Next Change

4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

Figure 4.3.4.2.2-1 Flow chart for WAB-gNB mobility with change of UE’s AMF(s)

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s).

- The WAB-gNB mayapply new cell configuration parameters related to the WAB-gNB’s current location, and broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. in its serving cell. In addition, the WAB-gNB will inform UE about the system information update.

- The UEs are handled as follows:

- A UE detects the new TAC which is out of it’s Registration Area scope.

- TheMobility Registration Update procedure is performed as defined in TS 23.502.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

Next Change

4.3.X Neighboring relationship update for WAB-gNB

The WAB-node mobiliy will result in the dynamic change of the neighboring relationship for the WAB-gNB. Thus, it is essential for the WAB-gNB to update the neighboring relations timely in order to support the UE mobility via the Xn connection.

The BH-gNB can help for the WAB-gNB neighboring relationships determination based on its own neighboring relationships. For example, the BH-gNB can provide its own neighbour cells’ information to the WAB-node via RRC signalling or Xn signalling.

Next Change

4.3.Y Energy saving for WAB-node

If WAB-node supports energy saving, the WAB-gNB determines the cell DTX/DRX based on the WAB-MT serving cell DTX/DRX.

Next Change

4.5	Conclusions for WAB

To support the WAB-gNB mobility with UE’s AMF change, RAN3 recommends the following: the UE served by WAB-node should perform mobility registration update to enable the AMF re-location, based on detecting new TAC broadcasted by the WAB-gNB. No need to activate new cells on the WAB-gNB.

For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link. The BH-gNB should know the multiplexing capabilities of WAB-node, and the resource configuration of WAB-gNB.

End of Change