3GPP TSG-RAN WG3 Meeting #125	R3-244528

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	Discussion on SA2's LS (S2-2407345) on questions regarding FS_VMR_Ph2

Document for:	Discussion

1	Introduction

RAN3 received an LS from SA2 for the Rel-19 FS_VMR_Ph2 study [1]. In the LS, SA2 asked RAN3 the following questions.

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell)  for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

In this paper, we provide our views for the two issues. The draft reply LS is given in the Annex.

2	Discussion

2.1	Answers for the Issue 1

- 1) Prevent multi-hop access

- The first case of accessing restriction is “MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode”, this is same as the “how to prevent multi-hop for WAB” in our RAN SI. According to the candidate solution in clause 6.2.3.6 of TS 23.700-06[2], each WAB-gNB will add “WAB indicaiton” in the NG SETUP REQUEST message to AMF, and AMF will use such information to reject another WAB-MT during registration procedure if this WAB-MT try to access a WAB-gNB.

- This candidate solution in SA2’s TR is feasible to prevent multi-hop for WAB-node, but the drawback is obvious, i.e., if the a WAB-MT access another WAB-gNB, only the AMF can reject the WAB-MT at late stage (during the registration procedure). After that, the WAB-MT’s registration procedure will fail, and the WAB-MT can try to access other cells. However, next time, the WAB-MT may still choose another WAB-gNB to access because such solution can not avoid wrong choice from the WAB-MT.

- Observation 1: It is possible that AMF can reject the WAB-MT when detecting it connects to the network via an WAB-gNB, if WAB-gNB sends “WAB-node indication” to the AMF during NG setup, but the rejection will occur at very late stage, and such solution cannot avoid wrong choice about the BH-gNB in the next try of accessing.

- From RAN persperctive, the multi-hop avoidance can be achieved at earlier stage, i.e., during the cell selection stage. This can be achieved by the two simple solutions as follows.

- Option 1: WAB-gNB and other gNBs which do not allow WAB-node access broadcast “WAB-barred” in SIB 1.

- Option 2: WAB-MT only access the cell which broadcasts “WAB-support” in SIB 1, and WAB-gNB should not broadcast “WAB-support”.

- Option 1 allows WAB-MT connects legacy gNB, while option 2 will requires a gNB be upgraded to work as a BH-gNB. RAN3 can continue to discuss which option is better. However, either options can ensure that an WAB-MT will not select another WAB-gNB at very early stage, even before sending message 1. Compared to the candidate solution in SA2’s TR, the solution from RAN side is more promising since RAN based solution can prevent an IAB-MT from selecting a wrong cell which not allow the WAB-MT access in the first place.

- Observation 2: Broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

- 2) Prevent self-access

- The second case of accessing restriction listed in the LS is to avoid WAB-MT access the co-located WAB-gNB.

- First, we do not think this issue is valid, because the WAB-gNB and WAB-MT should be coordinated for the transmission and receiving operation (e.g., TDM, FDM, SDM, etc.).  And it is not possible for the WAB-MT to consider the cell served by its co-located WAB-gNB as the “best cell” when performing cell selection even if full-duplex is supported since some isolation/interference cancellation needs to be conducted. Otherwise, the transmitting of WAB-gNB and WAB-MT will cause severe interference to each other.

- Second, even if the WAB-MT incidently detects the signal of its co-located WAB-gNB, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation, e.g., the implementation can set restriction that a WAB-MT never access the cell configured to this WAB-node by OAM. On the other hand, the RAN-based broadcasting “WAB-support/WAB-barred” can also avoid such issue.

- Observation 3: With proper multiplexing coordination/isolation, the IAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

- It is proposed:

- Proposal 1: RAN3 provides the following response to Issue 1:

- RAN3 think broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

- RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

2.2	Answers for the Issue 2

- Regarding issue 2, “SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.”

9.3.1.260	Mobile IAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located mobile IAB-MT of the mobile IAB-DU which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU that serves the UE. |
| TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU which serves the UE. |

The SA2’s conclusion is clear that the additional ULI to be provided to the UE’s AMF is the TAC/Cell ID of the cell serving the MWAB-UE, if the WAB-MT and WAB-gNB belongs to same PLMN, then it is same as the additional ULI for mobile IAB, which is excerpted from TS 38.413 as below.

--------------------Excerpted from TR 23700-60-j00------------------------

8.5	KI#5: Support of location services for UEs when MWAB(s) is involved Conclusion

The following aspects are taken as way forward during normative work:

-	Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11.

The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

-	TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

-	If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase.

-	option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	option 2: geo-location of the MWAB.

-	option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

-	The AMF may provide the received Additional ULI to LMF in LCS request.

NOTE:	If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

For the inter-PLMN case, i.e., the WAB-MT and WAB-gNB belongs to different PLMN, there are three options provided by SA2’s conclusion. The contents from SA2’s TR are copied as below.

- Among the three options, the signalling impact to NGAP looks same for option 1 and option 3. However, to support option 1, OAM should provide the mapping relationship between geo-location and the TAC/cell ID to the WAB-gNB, whether and how can OAM support such functionality needs to check with SA5. Moreover, in some location, there maybe no coverage for the UE’s PLMN, then it is unclear how can the “mapped TAC/cell ID” being provided.

- In option 2, the geo-location of the WAB will be transmitted in NG interface. This is technical feasible, but the stage-3 signaling design will be different from legacy location information used in NG interface (i.e., TAI and cell ID are normally used to indicate UE’s location information in NG interface), then the design for intra-PLMN and inter-PLMN scenario will be different.

- For option 3, the signalling design is same as the “Mobile IAB-MT User Location Information”, and how can the AMF use the TAC/cell ID of the WAB-MT’s serving cell should be up to CN’s implementation. Using such option, the impact to NG interface will be same as the same PLMN scenario.

- Observation 4: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

- Based on the above analysis, we think the option 3 is the best solution and can be recommended for the normative phase. It is proposed:

- Proposal 2: RAN3 provides the following response to Issue-2:

- RAN3 will provide signaling design for the additional ULI which contains TAC/cell ID of the WAB-MT’s serving cell during normative phase.

3	Conclusion

In this contribution we provide our views for the questions asked by SA2, the following observations and proposals are provided

Answers for the first issue in LS

- Observation 1: It is possible that AMF can reject the WAB-MT when detecting it connects to the network via an WAB-gNB, if WAB-gNB sends “WAB-node indication” to the AMF during NG setup, but the rejection will occur at very late stage, and such solution cannot avoid wrong choice about the BH-gNB in the next try of accessing.

- Observation 2: Broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

- Observation 3: With proper multiplexing coordination/isolation, the IAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

- Proposal 1: RAN3 provides the following response to Issue 1:

- RAN3 think broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

- RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

Answers for the second issue listed in LS

- Observation 4: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

- Proposal 2: RAN3 provides the following response to Issue-2:

- RAN3 will provide signaling design for the additional ULI which contains TAC/cell ID of the WAB-MT’s serving cell during normative phase.

4	References

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2

- R3-243339, Discussion on the Architecture, Access control and QoS support of WAB, Huawei

# Annex – Reply LS to SA2

3GPP TSG-RAN3 Meeting #125	R3-xxxxxx

Maastricht, NL, 19-23 August, 2024

Title:	[DRAFT] Reply LS on access restriction and additional ULI of WAB

Response to:	S2-2407345/ R3-244019

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 would like to thank SA2 for their LS on the RAN impact considerations for WAB, and provide the following replies to SA2’s question:

- Issue 1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3’s Answer:

RAN3 think broadcasting “WAB-support/WAB-barred” is beneficial for WAB-MT to select proper BH-gNB, and can prevent WAB-MT from accessing a WAB-gNB at early stage.

RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

- Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3’s Answer:

RAN3 will provide signaling design for the additional ULI which contains TAC/cell ID of the WAB-MT’s serving cell during normative phase.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	Hefei, CN

RAN3#126	2024-11-18– 2024-11-22 	Orlando, US