3GPP TSG-RAN WG3 Meeting #125	R3-244529

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	Discussion on the architecture for NR Femto

Document for:	Discussion

1	Introduction

During RAN3#124 meeting, the TP for evaluating the NG architecture options was agreed and captured in the TR 38.799[1]. In addition, two options for the Xn architecture were discussed and captured in the TR also.

This contribution mainly focusses on the recommendation for the architecture of NR Femto.

2	Discussion

2.1	Architecture

To study the overall RAN architecture for supporting the NR Femto, we need to take the LTE HeNB into account since that the study involves a gap analysis of existing 5G functionality with HomeNB functionality.

As defined in TS 36.300 [2], the HeNB may connect to the MME directly, or connect to the MME via a HeNB GW, which appears to the MME as an eNB and appears to the HeNB as an MME. The functions supported by the HeNB shall be the same as those supported by an eNB. In TS 36.300, it is specified that one HeNB serves only one cell.

As for the NR Femto, the most common scenario to deploy the NR Femto is same to the HeNB, e.g., for the coverage enhancement at home or at enterprise, which implies the number of serving users can not be very large. Therefore, the basic principle for the HeNB is also applicable for the NR Femto, i.e., one NR Femto serves only one NR cell.

Same as HeNB, one NR Femto serves only one NR cell.

2.1.1	Recommendation for NG Architecture

According to the progress made in the previous RAN3 meeting, there are 4 candidate options for the NR Femto architecture:

- Option 1: NR Femto node directly connects to the 5GC

- Option 2: NR Femto node connects the 5GC via the NR Femto GW

- Option 3: Using the SCTP concentrator between NR Femto node and the 5GC

- Option 4: NR femto node is deployed as a gNB-DU, and the gNB-CU terminates NG interface towards 5GC

Following table concludes the comparison of the above 4 options according to the evaluation part of the NG architecture options [1].

| Option | Pros | Cons |
| --- | --- | --- |
| 1 | - Already supported by current architecture. - Less CP latency and no processing delay due to absence of a concentration stage. - Suitable for certain deployments depending on number of NR Femtos to connect and/or virtualization support of the 5GC. - Local breakout can be supported. | - Not suitable for certain deployments with large number of NR Femtos and/or 5GC not virtualized. - Not suitable for residential deployments with frequent switch on/off of NR Femtos. |
| 2 | - Only one SCTP association from 5GC to NR Femto GW, so it can support large number of femtos and/or no virtualization of 5GC. - 5GC is shielded from frequent switch on/off of the NR Femtos. - Enables operators who have already deployed 4G Femtos using HeNB GW to capitalize on operating model and integration process of 5G Femtos. - Foreseen specification impacts are already well known from 4G. - Allows to decouple concentration of CP and concentration of UP: concentration of UP is optional i.e. the NR Femto GW can concentrate CP only while the NR Femto connects directly to the UPF. - Local breakout can be supported. | - Some stage3 specification impact. - Some processing delay for CP message. |
| 3 | - Only one SCTP association from 5GC to SCTP concentrator due to using multi-streaming. - Local breakout can be supported. - Only stage2 specification impact. | - The solution requires consistent configuration and handling of SCTP stream identifiers. - The solution requires consistent SCTP implementation of AMF, SCTP concentrators and NR femtos. - Some processing delay for CP message. - Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN. |
| 4 | - Reuse existing split gNB architecture. | - F1-C was not designed to face frequent switch on/off. Usually F1-C is operated by the network operator and statically configured. - Foreseen additional interoperability issue of F1 compared to NG. - F1-C carried over internet backhaul can lead to latency and reliability issue not meeting the stringent requirement for F1-C interface. - This option forces the concentration of User Plane and not only control plane i.e. concentration of CP only while NR femto UP connects directly to UPF is not possible. - Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN. - Specification impact for F1 needs to be further assessed. - Local breakout requires collocation of NR Femto with CU UP. |

- According to the above comparison table, we can see that the option 1 is the most promising solution when compared to other options, since it can be achieved directly by reusing the existing NG architecture and provides the least CP latency and processing delay. Regarding the scalability issue raised as the only cons of option 1, we do not see the issue since there is no clear gap analysis between the deployment requirement and the capability of the SCTP associations supported by the AMF which is much powerful than 4G core network. And in addition, considering that each network function in 5GC can be implemented as a virtualised function instantiated on a cloud infrastructure, the listed cons are not convincing at all.

For NG interface architectures, RAN3 selects option 1.

2.1.2	Recommendation for Xn Architecture

In RAN3#124 meeting, the following two options for Xn architecture were captured in the TR:

- Option A: NR Femto node directly connects to the neighboring gNB/NR Femto node

- Option B: NR Femto node connects the neighboring gNB/NR Femto node via the Xn GW

Apparently, Option A requires no change on the architecture, and it has advantage on the less processing delay when compared to option B. Option B has advantage from the scalability point of view. However, the scalability issue even for the NG interface has not been justified, considering that the Xn connection for NR Femto node will be very limited, the concentrator for Xn connection management is not needed. Note that even for 4G, the X2 GW was regarded as lower priority and therefore introduced in latter releases.

For Xn interface architectures, RAN3 selects option A.

3	Conclusion

This paper focus on the architecture recommendation for the NR femto, and propose the follows:

- Same as HeNB, one NR Femto serves only one NR cell.

- For NG interface architectures, RAN3 selects option 1.

- For Xn interface architectures, RAN3 selects option A.

4. Reference

- 3GPP TR 38.799 v1.0.0, Study on additional topological enhancements for NR.

- TS 36.300 V18.1.0, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2.

Annex——TP for TR 38.799 v1.0.0

Start of Change

5	5G Femto

5.1	General

The study of 5G Femto is based on the following requirements:

- one NR Femto serves only one NR cell.

Next Change

5.2	Architecture

Next Change

5.2.x	Evaluation of Architecture options for the Xn interface

Following table concludes the comparison of the two Xn interface options [1].

| Option | Pros | Cons |
| --- | --- | --- |
| A | - Already supported by current architecture. - Less CP latency and no processing delay due to absence of a concentration stage. |  |
| B | - Can support large number of Xn connections for one NR Femto. | - Some stage3 specification impact. - Some processing delay for CP message. |

Next Change

5.4	Local services access

Editor Note: Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any

5.5	Conclusions

- As an outcome of the study, it is recommended to adopt option 1 for NG interface architecture selection, and adopt option A for Xn interface architecture selection.