3GPP TSG-RAN WG3 Meeting #125	R3-244530

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	Discussion on the Access control for NR Femto

Document for:	Discussion

1	Introduction

During RAN3#124 meeting, the TP for evaluating the NG architecture options was agreed and captured in the TR 38.799[1]. In addition, two options for the Xn architecture were discussed and captured in the TR also. “Access control with CAG” was discussed but mainly related to the incoming LS.

In this contribution, we mainly focus on the Access control of NR Femto.

2	Discussion

The UE access can be controlled by using the CSG mechamism under the LTE HeNB deployment. There are three access modes for the UE to perform an access [2]:

- Open access mode: The network configures and activates a PLMN cell, which can be accessed by legacy UE and the UE supporting CSG without access control.

- Hybrid access mode: The network configures and activates a hybrid cell. For the UE with CSG subscription, the HeNB provides the selected CSG ID and CSG access mode to the MME for CSG membership verification. This cell is accessible as a CSG cell by UEs which are members of the CSG. And for the other UEs, this cell is viewed as a normal PLMN cell.

- Closed access mode: The network configures and activates a CSG cell, which can only be accessed by the UEs which has subscribed the related CSG ID. The HeNB provides the selected CSG ID to the MME for CSG access control.

----------excerpt from TS 38.331----------

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

----------excerpt from TS 38.331----------

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

Similar to the CSG, NR uses the CAG mechanism when designing the support for NPN. We provide some introduction for the CAG according to the existing NR specifications:

----------excerpt from TS 38.300----------

16.7	Public Network Integrated NPN

16.7.1	General

A PNI-NPN is a network deployed for non-public use which relies on network functions provided by a PLMN (see clause 4.8). In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier broadcast in SIB1.

A CAG-capable UE can be configured with the following per PLMN (see clause 5.30.3.3 of TS 23.501 [3]):

-	an Allowed CAG list containing the CAG identifiers which the UE is allowed to access; and

-	a CAG-only indication if the UE is only allowed to access 5GS via CAG cells.

Dual Connectivity is supported and may involve both PNI-NPN and PLMN cells, according to the mobility restrictions in the UE context as described in TS 37.340 [21].

----------excerpt from TS 38.300----------

## 16.7	Public Network Integrated NPN

### 16.7.1	General

A PNI-NPN is a network deployed for non-public use which relies on network functions provided by a PLMN (see clause 4.8). In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier broadcast in SIB1.

A CAG-capable UE can be configured with the following per PLMN (see clause 5.30.3.3 of TS 23.501 [3]):

-	an Allowed CAG list containing the CAG identifiers which the UE is allowed to access; and

-	a CAG-only indication if the UE is only allowed to access 5GS via CAG cells.

Dual Connectivity is supported and may involve both PNI-NPN and PLMN cells, according to the mobility restrictions in the UE context as described in TS 37.340 [21].

The existing CAG mechanism already supports the above 3 access modes of HeNB as the following:

- To support the open access mode: The NR Femto activates a PLMN cell, which can be accessed by legacy UE without access control.

- To support the hybrid access mode: The NR Femto cell can be shared by both PLMN and CAG, through broadcast both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.  Then, this cell is accessible as a CAG cell by UEs which has the allowed CAG list including this cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

- To support the closed access mode: The NR Femto activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value be set as “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the NR Femto cell.

The existing CAG mechanism already supports all the access modes involved in CSG, i.e. open, hybrid and closed, no gap is found.

- For Access control, NR Femto reuse the existing CAG functionalities, no enhancement is needed.

3	Conclusion

This paper provides analysis for the access control of NR femto, the following observation and proposal are obtained:

- Access control

- The existing CAG mechanism already supports all the access modes involved in CSG, i.e. open, hybrid and closed, no gap is found.

- Proposal 1:	For Access control, NR Femto reuses the existing CAG functionalities, no enhancement is needed.

4. Reference

- RAN3#123bis Chair notes.

- TS 36.300 V18.1.0, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2.

- 3GPP TS 38.401 v18.1.0, NG-RAN; Architecture description.

Annex——TP for TR 38.799 v0.0.0 (R3-242242)

Start of Change

5.3	Access control

To support the access control during the initial access and mobility, the NR Femto can resue the existing CAG mechanism which is defined for the PNI-NPN.

- With the existing CAG mechanism, the access modes involved in CSG, i.e. open, hybrid and closed, can be supported as the following:

- To support the open access mode: The NR Femto activates a PLMN cell, which can be accessed by legacy UE without access control.

- To support the hybrid access mode: The NR Femto cell can be shared by both PLMN and CAG, through broadcast both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.  Then, this cell is accessible as a CAG cell by UEs which has the allowed CAG list including this cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

- To support the closed access mode: The NR Femto activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value be set as “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the NR Femto cell.

Next Change

5.4	Local services access

Editor Note: Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any

5.5	Conclusions

- As an outcome of the study, it is concluded that for Access control, NR Femto reuses the existing CAG functionalities, and therefore no enhancement is needed.

End of Change