3GPP TSG RAN3 125	R3-244549

# Maastricht, NL, 19 - 23 August, 2024

Agenda item:	9.2

Source: 	Samsung, ZTE, CATT, China Unicom, Lenovo

Title: 	Discussion on QMC support in NR-DC for RRC segmentation

Document for:	Discussion and decision

Introduction

Last RAN3 meeting further discussed the necessity to introduce the RRC segmentation over XnAP, but no consensus achieved.

In this contribution, we further discuss this open issue.

Discussion

The open issue to discuss is whether to introduce the coordination about RRC segmentation related information over XnAP.

2.1 Motivation

Recall that in terms of the QoE configuration, RAN2 has introduced three RRC parameters in QoE configuration, namely,

-       reportingSRB: This parameter indicates which SRB (SRB4 or SRB5) UE should use for QoE reporting.

-       rrc-SegAllowedSRB4: This parameter indicates whether the RRC segmentation is enabled on SRB4.

-       rrc-SegAllowedSRB5: This parameter indicates whether the RRC segmentation is enabled on SRB5.

AppLayerMeasConfig information element

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}                                                       OPTIONAL  -- Need R

]]

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                         OPTIONAL, -- Cond ServiceType

...,

[[

reportingSRB-r18                     ENUMERATED {srb4, srb5, spare2, spare1}                                    OPTIONAL, -- Need M

appLayerMeasPriority-r18             INTEGER (1..16)                                                            OPTIONAL, -- Need M

appLayerIdleInactiveConfig-r18       AppLayerIdleInactiveConfig-r18                                             OPTIONAL  -- Need M

]]

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17           ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17       INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                                                                   OPTIONAL, -- Need M

...,

[[

ran-VisibleReportingSRB-r18          ENUMERATED {srb4, srb5, spare2, spare1}                                    OPTIONAL  -- Need M

]]

}

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

Then the following cases could happen,

Case 1: For those MN configured QoE configuration, MN can configure a QoE configuration to UE with reportingSRB=SRB5. Then, SN can forward QoE report to MCE directly.

Case 2: For those SN configured QoE configuration, SN can configure a QoE configuration to UE with reportingSRB=SRB4. Then, MN can forward QoE report to MCE directly.

By looking at the RRC spec, as a consequence, the following questions should be answered,

- If MN sends QoE configuration to the UE, how MN knows/sets rrc-SegAllowedSRB5?

- If SN sends QoE configuration to the UE, how SN knows/sets rrc-SegAllowedSRB4?

To answer the above questions, whether the coordination is needed between MN and SN for RRC segmentation should be further discussed.

Observation 1: Further discussion is needed on how MN knows/sets rrc-SegAllowedSRB5 if MN sends QoE configuration to the UE, and on how SN knows/sets rrc-SegAllowedSRB4 if SN sends QoE configuration to the UE.

2.2 Previous discussion

Whether the RRC segmentation is allowed over SRB4 or SRB5 is node capability?

According to the discussion last several RAN3 meetings, some company argued that whether the RRC segmentation is allowed over SRB4 or SRB5 is node capability, and it should be avoided to exchange the node capability by signalling. However, as we look into more details on what RAN2 has defined in RRC,

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

It can be observed from RRC spec that, firstly, such configuration is configured on a per UE basis, which means that there’s possibility that the RRC segmentation is enabled on e.g. SRB5 for one UE but not enabled for another UE under the same gNB. In addition, it is also possible that whether to enable the RRC segmentation over SRB4 or SRB5 can be reconfigured by the network side by using RRC signalling.

If we consider whether to enable the RRC segmentation over SRB4 or SRB5 is purely node capability, it will be redundant to signal such information through dedicated RRC signalling, and instead such information should have been introduced in the System Information over Uu. However, the current RRC spec adopts the dedicated RRC signalling approach which reveals the fact that perceiving whether to enable the RRC segmentation over SRB4/5 as the node capability deviates from the intention to introduce such information in the RRC spec.

Observation 2: According to RRC spec, the configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is on a per UE basis.

Observation 3: According to RRC spec, it is possible that the RRC segmentation is enabled on e.g. SRB5 for one UE but not enabled for another UE under the same gNB acting as the SN for both UEs.

Observation 4: According to RRC spec, it is possible that whether to enable the RRC segmentation over SRB4/5 can be reconfigured to a UE by UE dedicated signalling.

Observation 5: Perceiving the configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 as node capability deviates from the intention to introduce such IEs and restricts the flexibility for good gNB implementation.

Whether MN and SN can send configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 independently to the UE without coordination?

According to the discussion last meeting, some company raised a point that although the current spec indicates that the QoE configuring node sends QoE configuration to the UE, it is also allowed that MN and SN can send configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 independently without coordination. Such understanding implies that the configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is not contained in QoE configuration.

However, as we check more details on the RRC spec,

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

We notice that the rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 IEs are optional with Need R field. Recall that the Need R field indicates that if the IE is not present then release the current configuration. As a consequence, if it is allowed that MN sends the configuration on rrc-SegAllowedSRB4 and SN sends the configuration on rrc-SegAllowedSRB5 independently regardless of which node is the QoE configuring node, the misalignment will happen. To be more specific, take the following case as an example,

Step 1, MN sends the configuration to enable rrc-SegAllowedSRB4 (and rrc-SegAllowedSRB5 is not present), then current status at the UE side for RRC segmentation is rrc-SegAllowedSRB4 (enabled), rrc-SegAllowedSRB5 (not enabled).

Step 2, SN sends the configuration to enable rrc-SegAllowedSRB5 (and rrc-SegAllowedSRB4 is not present so the configuration on rrc-SegAllowedSRB4 is released), then current status at the UE side for RRC segmentation is rrc-SegAllowedSRB5 (enabled), rrc-SegAllowedSRB4 (not enabled).

After Step 2, UE will consider the RRC segmentation is not allowed on SRB4; however, MN still enables the RRC segmentation over SRB4. So the misalignment between UE and MN happens.

Therefore, in order to avoid such misalignment issue, the only way is to let the QoE configuring node sets both IEs (i.e. rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5) via coordination with non-QoE configuring node.

Observation 6: If it is allowed that MN sends the configuration on rrc-SegAllowedSRB4 and SN sends the configuration on rrc-SegAllowedSRB5 independently regardless of which node is the QoE configuring node, the misalignment between the network and the UE may happen.

Observation 7: To avoid such misalignment issue, the only way is to let the QoE configuring node sets and sends the configuration on both IEs (i.e. rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5) to the UE. Therefore, the coordination between QoE configuring node and non QoE configuring node is needed.

How to coordinate for initial coordination?

According to the discussion last meeting, there are two coordination options on the table,

Option 1: The QoE configuring node sends the request explicitly on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [1]).

Option 2: The QoE configuring node sends the QMC coordination request which implicitly requests on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [2]).

It can be observed that the main difference between Option 1&2 is that whether an explicit request on whether the non QoE configuring node enables the RRC segmentation is needed.

For Option 2, it is always up to the non QoE configuring node to decide on whether to enable the RRC segmentation for its own SRB.

While for Option 1, it additionally requires the QoE configuring node to firstly decide whether to initiate the RRC segmentation coordination, and if the QoE configuring node decides not to initiate the RRC segmentation coordination (i.e. the RRC segmentation request is not contained in the QMC coordination request), then the non QoE configuring node will not enable the RRC segmentation for its own SRB by default.

Compared to Option2, Option 1 may also require the QoE configuring node to initiate the RRC segmentation coordination based on some QMC related information the non QoE configuration node does not have, including, e.g., the Service Type, the Assistance Information for QoE configuration (i.e. the priority).

In our understanding, both options can work, so we prepare two XnAP CRs for two options separately, and leave it to RAN3 discussion on which option to follow.

How to coordinate the modification of rrc-SegAllowedSRBx, after the initial coordination?

This open issue has not been discussed in previous meetings; however, according to the previous observation, it is possible that whether to enable the RRC segmentation over SRB4/5 can be reconfigured to a UE by UE dedicated signalling. As a result, it is worthy to investigate how to coordinate the modification of rrc-SegAllowedSRBx between MN and SN.

Basically there are two approaches on how to initiate the coordination of the modification on rrc-SegAllowedSRBx,

Method 1: QoE configuring node can initiate the RRC segmentation coordination when it intends to modify the QoE configuration.

Method 2: Non QoE configuring node can modify whether it enables the RRC segmentation over its own SRB autonomously.

It should be noted that for Method 1, non QoE configuring node cannot modify whether it enables the RRC segmentation over its own SRB, until a coordination request is received from the QoE configuring node.

We observe that Method 2 may introduce more XnAP spec impact, so basically we prefer to follow the similar way as initial coordination, for the modification coordination on RRC segmentation.

Observation 8: In order to avoid further spec impact, it is preferable to follow the similar way as initial coordination, for the modification coordination on RRC segmentation.

At which level such coordination is provided?

As indicated by RRC spec, whether the RRC segmentation is enabled over SRB4/5 is configured on the UE basis, i.e. if the rrc-SegAllowedSRBx is enabled, all QoE measurement reported over SRBx will enable RRC segmentation.

However, the contributions submitted to previous meetings ([1] [2]) provided a method that whether the RRC segmentation is enabled over SRBx is coordinated per QoE Reference, which is not aligned with the RRC spec. So we suggest to put the RRC segmentation coordination at the same level as QMC Coordination Request/Response List, and further details can be found in our companion contributions.

Based on the above analyses and observations, we propose,

Proposal 1: The QoE configuring node explicitly/implicitly asks the non QoE configuring node whether the non QoE configuring node enables RRC segmentation in QMC Coordination Request over XnAP.

Proposal 2: The non-QoE configuring node explicitly responses whether it enables RRC segmentation or not in QMC Coordination Response over XnAP.

Proposal 3: Agree either the CR in R3-24xxxx or the CR in R3-24yyyy.

Conclusion

In this paper, we provides further considerations for QoE on support of NR-DC. The following proposals are provided,

Observation 1: Further discussion is needed on how MN knows/sets rrc-SegAllowedSRB5 if MN sends QoE configuration to the UE, and on how SN knows/sets rrc-SegAllowedSRB4 if SN sends QoE configuration to the UE.

Observation 2: According to RRC spec, the configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 is on a per UE basis.

Observation 3: According to RRC spec, it is possible that the RRC segmentation is enabled on e.g. SRB5 for one UE but not enabled for another UE under the same gNB acting as the SN for both UEs.

Observation 4: According to RRC spec, it is possible that whether to enable the RRC segmentation over SRB4/5 can be reconfigured to a UE by UE dedicated signalling.

Observation 5: Perceiving the configuration on rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 as node capability deviates from the intention to introduce such IEs and restricts the flexibility for good gNB implementation.

Observation 6: If it is allowed that MN sends the configuration on rrc-SegAllowedSRB4 and SN sends the configuration on rrc-SegAllowedSRB5 independently regardless of which node is the QoE configuring node, the misalignment between the network and the UE may happen.

Observation 7: To avoid such misalignment issue, the only way is to let the QoE configuring node sets and sends the configuration on both IEs (i.e. rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5) to the UE. Therefore, the coordination between QoE configuring node and non QoE configuring node is needed.

Observation 8: In order to avoid further spec impact, it is preferable to follow the similar way as initial coordination, for the modification coordination on RRC segmentation.

Proposal 1: The QoE configuring node explicitly/implicitly asks the non QoE configuring node whether the non QoE configuring node enables RRC segmentation in QMC Coordination Request over XnAP.

Proposal 2: The non-QoE configuring node explicitly responses whether it enables RRC segmentation or not in QMC Coordination Response over XnAP.

Proposal 3: Agree either the CR in R3-24xxxx or the CR in R3-24yyyy.

References

[1] R3-242035

[2] R3-243613