3GPP TSG RAN3 125	R3-244552

# Maastricht, NL, 19 - 23 August, 2024

Agenda item:	14.2

Source: 	Samsung

Title: 	Discussion on support MBS broadcast service for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of MBS broadcast service for NR NTN, and the agreement and open issues are captured as follows,

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Other options can be further checked.

MBS Broadcast service delivery area over NG:

- Mapped cell ID/list in MBS service area

- New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Based on GNSS description

- To be continued...

In this contribution, we further discuss these open issues.

Discussion

According to the R19 NR NTN WID, one of the objectives is to support MBS broadcast service. And the following details are provided in the WID,

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

Regarding the first bullet, the motivation to work on enhancement is that for some cases the intended service area is expected to be smaller than the coverage of a Uu cell. So the SIB signalling needs to provide the intended service area more granular than Uu cell provided by NTN. According to the WID, it is up to RAN2 to determine the detailed content related to the intended service area in the SIB to be used. If a new SIB is determined to be introduced, RAN3 can focus on potential enhancement on new SIB coordination in case of split architecture.

More specifically, the open issues could be, e.g.

- Which node provides the intended service area related information contained in the new SIB?

- Which node encodes the new SIB, CU or DU?

- Whether the existing mechanism on SIB coordination over F1AP is enough for the new SIB?

According to RAN2 progress, the new SIB has not been introduced, and it is also possible to reuse the existing SIBs or RRC messages. Therefore, such open issues could be further checked once RAN2 has progress on the detailed contents of the SIB.

Observation 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Regarding the second bullet, last RAN3 meeting has further discussed this issue, and the following WA has achieved,

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

According to the last RAN3 meeting, it is the majority view that the signalling of intended service area should provide a finer granularity compared to the Uu NTN cell. And the mapped cell ID approach is at least one way to provide the MBS service area, over NG signalling, especially for cases when the geographical areas of the mapped cells match perfectly with the MBS service area. So in our understanding, there’s no showstopper to provide a list of the mapped cell IDs to represent the MBS service area.

Proposal 1: Turn WA into agreement that the mapped cell ID can be used to define MBS broadcast service in NTN.

While for other options on the table, the motivation to put forward such options is based on an assumption that the geographical areas of the mapped cells cannot match the MBS broadcast service area. Recall that other potential options are as follows,

MBS Broadcast service delivery area over NG:

- New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Based on GNSS description

For the above options to provide the intended service area over NGAP, such options could be further discussed and enhanced if deemed necessary.

We notice that there’s one agreement achieved in RAN2 last meeting,

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

In addition, RAN2 has achieved the following agreements in April meeting,

Agreements:

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

It can be observed from RAN2 agreements that RAN2 is also considering to discuss other alternatives to describe the intended service area over Uu. So the first thing we need to identify is that whether the mapped cell is good enough to approximate the intended shape of service area; if not, which option is the best to approximate the intended shape of service area.

Observation 2: Further discuss other alternatives to describe the intended service area if the mapped cell is not enough to approximate the intended shape of service area.

Conclusion

In this paper, we provide discussion on support MBS broadcast service for NR NTN. The following proposals are provided,

Observation 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Proposal 1: Turn WA into agreement that the mapped cell ID can be used to define MBS broadcast service in NTN.

Observation 2: Further discuss other alternatives to describe the intended service area if the mapped cell is not enough to approximate the intended shape of service area.

References