3GPP TSG RAN3 125	R3-244554

# Maastricht, NL, 19 - 23 August, 2024

Agenda item:	15.2

Source: 	Samsung

Title: 	Discussion on support of regenerative payload for IoT NTN

Document for:	Discussion and decision

Introduction

This meeting will primarily discuss the support of regenerative payload for IoT NTN, and the following objective is captured,

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

- Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

-At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

In this contribution, we primarily discuss the support of regenerative payload for IoT NTN.

Discussion

According to the WID, the R19 IoT NTN will focus on the support of S&F operation with full gNB as regenerative payload in RAN3.

From architectural point of view, R19 IoT NTN clearly indicates to work on the scenario with full eNB as regenerative payload, which in our understanding indicates that only the full eNB is on board the satellite. On the other hand, we also notice that the ongoing SA2 SI has also considered the case when part of the Core Network (such as MME-NT and/or part of S-GW) is also on board the satellite, which are also captured in TR 23.700-29.

Since the discussion in SA2 is still ongoing, no conclusion can be made for now on which part of the Core Network can also be on board the satellite. As a result, it is suggested that RAN3 firstly focuses on the network architecture that only the full eNB is on board the satellite, and we can further consider whether the partial Core Network is on satellite after SA2 conclusion.

The figure below illustrates the network architecture which RAN3 primarily focuses on,

Proposal 1: RAN3 focuses on the network architecture that only the full eNB is on board the satellite at the initial stage. It is suggested to further consider other network architecture (e.g. whether consider partial CN on satellite) after the conclusion of SA2 SI.

As indicated by the WID in RP-234077, the support of S&F satellite operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure. And a typical scenario can be illustrated as follows,

Assuming the full eNB is on board the satellite, at time instance T1, the satellite can provide the service link with the UE without the feeder link. After obtaining UL data from UE which is to be transmitted to the CN, the Satellite needs to store such UL data during T1->T2->T3. At time instance T3, when the satellite establishes the feeder link with the NTN GW, such UL data from UE can be further transmitted to the CN for further operations.

It should be noted that in TR 23.700-29, SA2 has discussed the possibility of the support of S&F satellite operation with both single satellite deployment and multiple satellite deployment. More specifically, for single satellite deployment, only one regenerative payload serves a specific UE; while for multiple satellite deployment, multiple regenerative payloads serve a specific UE. Consequently, RAN3 can also consider both scenarios with single satellite deployment and multiple satellite deployment.

Proposal 2: RAN3 is kindly asked to consider both single satellite deployment and multiple satellite deployment on support of S&F operation serving a UE.

In addition, it should be noted that this is the first time discussing the support of S&F operation with the full eNB as the regenerative payload, compared to normal TN scenario or NTN scenario with transparent payload, the network architecture as well as the node behaviour has been totally changed. Thus, there is a need to firstly pay attention to whether there has to be any update on the current basic procedures, and we believe such basic procedures may include the following ones,

- (Initial) Attach procedure

- Service Request procedure

- Connection Suspend procedure

- Connection Resume procedure

- Control Plane CIoT EPS Optimisation

- User Plane CIoT EPS Optimisation

Moreover, for procedures of CP/UP CIoT EPS Optimisation, more detailed procedures including MO-EDT, MT-EDT and transmission using PUR can be further investigated.

Proposal 3: RAN3 is kindly asked to further investigate potential enhancement on the following basic procedures on support of S&F operation with full eNB as regenerative payload,

- (Initial) Attach procedure

- Service Request procedure

- Connection Suspend procedure

- Connection Resume procedure

- Control Plane CIoT EPS Optimisation

- User Plane CIoT EPS Optimisation

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for IoT NTN. The following proposals are provided,

Proposal 1: RAN3 focuses on the network architecture that only the full eNB is on board the satellite at the initial stage. It is suggested to further consider other network architecture (e.g. whether consider partial CN on satellite) after the conclusion of SA2 SI.

Proposal 2: RAN3 is kindly asked to consider both single satellite deployment and multiple satellite deployment on support of S&F operation serving a UE.

Proposal 3: RAN3 is kindly asked to further investigate potential enhancement on the following basic procedures on support of S&F operation with full eNB as regenerative payload,

- (Initial) Attach procedure

- Service Request procedure

- Connection Suspend procedure

- Connection Resume procedure

- Control Plane CIoT EPS Optimisation

- User Plane CIoT EPS Optimisation

References