3GPP TSG RAN3 125	R3-244555

# Maastricht, NL, 19 - 23 August, 2024

Agenda item:	16.2

Source: 	Samsung

Title: 	Discussion on RAN architecture for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting RAN3 continued to discuss the RAN architecture for Ambient IoT, and the following open issue is captured,

Continue to discuss the architecture for Topology2

In this contribution, we further discuss the RAN architecture aspects for Ambient IoT.

Discussion

The RAN architecture aspect for Ambient IoT was discussed last meeting. And the main achievement of the last meeting is to capture a logical system architecture for AIoT as follows in [1],

According to the discussion last meeting, it is still controversial on the following two aspects,

- As raised by some companies, whether and how to define AIoT RAS is still controversial.

- As given by Editor Note 9, whether the logical system architecture above is applied to topology 2.

Regarding the first open issue, [1] also captures a tentative description on AIoT RAS as follows,

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

However, in our understanding, the terminology above needs to be further discussed. Recall that in TS 23.501, we’ve already had a definition on 5G System as below,

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

We can observe that for the definition of 5G system, the UE is also contained in the 5G system.

While in the proposed definition of AIoT RAS, the AIoT device is not included. Therefore, from this perspective, the definitions are not aligned between 5G System and AIoT RAS.

So what do we do to offset such misalignment?

One possible way is that we still adopt a definition of AIoT RAS, but we also include AIoT device within AIoT RAS. So the definition of AIoT RAS can be reworded as follows,

AIoT RAS: AIoT Radio Access System consisting of AIoT RAN and AIoT device.

And as a complement, we need an additional definition for AIoT RAN, which is proposed as follows,

AIoT RAN: hosts certain functions for AIoT as part of the functional split between RAN and CN.

As a consequence, the logical system architecture can be modified as follows,

And the above figure at least applies to Topology 1, and whether to keep the big box (AIoT RAS) can be FFS.

In addition, after the discussion last meeting, we actually see no strong need to captured a unified architecture for both topologies. Firstly, there’s no requirement in any other WGs on mandating to adopt a unified system architecture (RAN2 only tries to have a unified design on AIoT radio instead of a unified system architecture). Secondly, of course RAN3 could strive to discuss on a unified architecture, but after putting into so much endeavour, it has turned out that such try just brings more controversies than benefits. As a result, we’d like to have separate figures on the system architecture for Topology 1 and Topology 2.

For the system architecture for Topology 2, we can start with the version captured in [2] as follows,

To tell the truth, the above figure is quite fancy and I personally truly respect the effort to generate such a figure for topology 2. But we have to admit that we fail to understand why the Uu interface is out of AIoT RAS, as well as why we need to capture non-AIoT functions here since it is irrelevant to AIoT at all.

Note that we’ve captured the following definition on AIoT RAN node function in [1],

AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

If we have achieved consensus that the AIoT RAN node function contains the control of the AIoT radio resources, then we have no other options but to use Uu interface between the AIoT-enabled UE and AIoT-enabled gNB to perform AIoT resource allocation in topology 2. So in our opinion, the Uu interface should be in the scope of AIoT RAS (if the definition of AIoT RAS is adopted).

Based on the above understandings, we think the sustem architecture for Topology 2 can be modified as follows,

And also whether to keep the big box (AIoT RAS) can be FFS for the system architecture for topology 2.

Proposal 1: RAN3 is kindly asked to agree two separate system architectures for two topologies, and FFS on whether there’s still need to adopt a definition on AIoT RAS.

Conclusion

In this paper, we provide further discussion on RAN architecture for Ambient IoT. The following proposals are provided,

Proposal 1: RAN3 is kindly asked to agree two separate system architectures for two topologies, and FFS on whether there’s still need to adopt a definition on AIoT RAS.

Reference

[1] R3-243962

[2] R3-243807