3GPP TSG RAN3 125	R3-244556

# Maastricht, NL, 19 - 23 August, 2024

Agenda item:	16.3

Source: 	Samsung

Title: 	Discussion on RAN-CN interface impact for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting continued to discuss the RAN-CN interface impact for Ambient IoT, and the agreement and open issues provided as follows,

AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command).

Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices.

Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices.

What’s the device context？

The content in Inventory and Command?

In this contribution, we further discuss these open issues.

Discussion

The RAN-CN interface impact for Ambient IoT was discussed last meeting, and some progress has been achieved.

According to the outcome of the last meeting, there are several open issues captured as follows,

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

In Topology 2, it is FFS which entity (e.g., AIoT CN and/or AIoT enabled gNB) performs the UE reader selection.

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

It can be seen from the above open issues that, one main discussion point is about the content in Inventory and Command, and the other is about the overall procedure.

Regarding the content in Inventory and Command, last meeting has agreed to introduce AIoT Device Identification as well as Scope (related to area) in the Inventory Request message, while no agreement has been achieved on the content in Command message.

For the remaining potential parameters that can be included in the Inventory Request message, we think the Inventory Session ID is also needed. Recall that RFID supports multiple sessions for inventory. In order to provide a comparable capability for Ambient IoT to RFID. We see no obstacle to support multiple inventory sessions. In our understanding, the inventory session is determined by AF/CN, and a session related identifier should be provided in the Inventory Request message.

While for other potential parameters, the Estimated Number of expected responses from devices can be further discussed depending on the use cases and solutions. In our understanding, the purpose of the Inventory is to let the server understand how many and which AIoT devices are in the scope. So whether the Server or AIoT CN could estimate the number of expected responses is actually depending on SA2 progress. On the other hand, the usage of the Estimated number of expected responses may be used for determining the Q value used for Random Access which is depending on RAN2 progress. So RAN3 may not be able to achieve consensus on whether to introduce such parameter in Inventory Request message until more progress can be achieved in other WGs.

In addition, we see no strong need to introduce Periodicity in Inventory Request message since the periodic inventory can be easily achieved by sending another Inventory Request message.

Proposal 1: Consider the Inventory Session ID as the content of Inventory Request message.

From the other aspect, the content of Command sent from CN to RAN has not been discussed last meeting. In our understanding, at least the following information can be included in the Command message,

- The Command container which is provided at NAS level.

- AIoT Device Identification. If the Command message is sent by setting up device context after inventory and before the command, such device related information can be AP UE IDs.

- The information related to the session. If the information related to the targeted Ambient IoT device, e.g. an device identifier used over RAN-CN interface is not globally unique, then the session related information may be needed if the device identifier used over RAN-CN interface can be allocated per session.

- The information on whether a reply is needed in response to the Command. The Command may contain Read command, Write command, etc. at the NAS or application layer level to perform corresponding operations with the storage of AIoT devices. For some command such as Write command, a reply from the AIoT device may not be needed; while for some other command such as Read command, a reply from the AIoT device is needed. In order to leverage the AIoT RAN to perform resource allocation for AIoT radio, the AIoT RAN needs to know whether the UL resources for a transmission should be allocated. Therefore the information on whether a reply is needed in response to the Command should be signalled from AIoT CN to AIoT RAN.

Proposal 2: Consider the following information as the content of Command message,

- The NAS container containing Command sent to UE.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

For the overall procedure of Inventory as well as Command,

As the first step, after receiving the Inventory message over AIoT radio, the Ambient IoT device(s) that matches the inventoried group related information should send the Ambient IoT device related information (e.g. identifier) to the network. And such Ambient IoT device related information needs to be transferred from RAN to CN.

There are multiple potential methods to transfer Ambient IoT device related information from RAN to CN and perform indoor command subsequently,

Method 1: Use the Initial UE Message like procedure to transfer the Ambient IoT device related information for a specific Ambient IoT device. And a RAN UE NGAP like ID is allocated by RAN and included in Initial UE Message to identify the Ambient IoT device over the CN-RAN interface.

Subsequently, after the authorization of the Ambient IoT device within CN, both CN and RAN is able to use device associated signalling (e.g. Downlink/Uplink NAS Transport) over RAN-CN interface to transfer indoor command and reply if any for a specific Ambient IoT device.

Figure - Method 1

Method 2: RAN can collect multiple Ambient IoT device related information from multiple Ambient IoT devices, and sends a list of Ambient IoT device related information to CN using non UE associated signalling. A list of device identifiers, each of which identifies a specific Ambient IoT device is also included in the message sent from RAN to CN. Such device identifier is allocated by RAN (e.g. UE NGAP like ID) or received from Ambient IoT device (e.g. 5G-S-TMSI like ID).

Subsequently, after the authorization of the Ambient IoT devices within CN, every time when the CN transfers indoor command to the RAN, the CN also includes the device identifier associated with the indoor command to let RAN understand the targeted Ambient IoT device. CN can also send multiple indoor commands for multiple Ambient IoT devices in one message. If the Ambient IoT device needs to reply to the indoor command, the reply is contained in a message transmitted from RAN to CN associated with a specific device identifier.

Figure - Method 2

In our understanding, both methods above are workable to accomplish the indoor inventory and the indoor command operations. Both methods require device context management (at least for the management of device identifiers which are used to identify the Ambient IoT devices over RAN-CN interface). We are open to further discuss these potential methods to investigate pros and cons in the future.

Proposal 3: Capture the above call flows in TR.

Conclusion

In this paper, we provide primary discussion on RAN-CN interface impact for Ambient IoT. The following proposals are provided,

Proposal 1: Consider the Inventory Session ID as the content of Inventory Request message.

Proposal 2: Consider the following information as the content of Command message,

- The NAS container containing Command sent to UE.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

Proposal 3: Capture the above call flows in TR.

Reference