3GPP TSG-RAN WG3 #125                                                                        R3-244559

Maastricht, NL, 19th – 23rd Aug 2024

Agenda item:	13.2

Source:	Samsung

Title:	Discussion on Inter-gNB-CU LTM

Document for:	Discussion & Decision

Introduction

RAN3 had some discussions on inter-CU LTM in previous meeting, and below agreements are achieved:

RAN3#123-bis: Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. RAN3#124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

In this paper, we will further discuss the basic procedure for inter-gNB-CU LTM and the corresponding potential network interface impact.

Discussion

First, to achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1, and provide the basic description for each involved message flows, and finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	Source gNB-CU sends a HANDOVER REQUEST message to the candidate gNB-CU(s) for each candidate cell to request LTM configurations, which carries a LTM Information Request IE containing a LTM Indicator, LTM Reference Configuration (if available) or an indication of its request, SSB/CSI Resource Configuration of candidate cells (if available), LTM configuration ID mapping list, and Early Sync Information Request.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations, containing one target candidate cell ID, LTM configuration ID mapping list, and the CSI resource configuration for subsequent LTM. The candidate gNB-CU may request PRACH resources for early TA acquisition from the candidate gNB-DU(s). The candidate gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations and the allocated PRACH resource for early TA acquisition for the accepted target candidate cell.

6.  If the candidate gNB-CU accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE including LTM configuration information of LTM candidate cell to the source gNB-CU, FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs, which may contain LTM reference configuration if requested or Complete Configuration Indicator, SSB/CSI resource configuration allocated by the requested candidate cell, and Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell. The HANDOVER REQUEST ACKNOWLEDGE message is sent for each candidate cell.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB-CU may send a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST with new code-point or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST) to the candidate gNB(s) containing the information for subsequent LTM, which may contain SSB/CSI resource configuration allocated for all the accepted candidate cells, LTM Reference Configuration(if available), LTM configuration ID mapping list, and Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

10.	The candidate gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration (e.g., the updated CSI report configuration).

12.	The candidate gNB-CU responds with a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST ACKNOWLEDGE or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST ACKNOWLEDGE) including the updated lower layer configuration (e.g., the updated CSI report configuration).

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed. UE performs early TA acquisition with the candidate cell(s) as requested by source gNB-DU before receiving the cell switch command. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via a new non-UE associated XnAP message (FFS on message name, e.g., LTM TA INFORMATION TRANSFER).

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE sends the L1 measurement result to the source gNB-DU.

23.	The source gNB-DU decides to execute LTM to a target cell.

24.	The source gNB-DU sends the Cell Switch command to the UE.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to target gNB-CU in the LTM CELL SWITCH NOTIFICATION message.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32.	The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell.

33.	The source gNB-CU sends the SN STATUS TRANSFER message for the late data forwarding.

NOTE 2: Late data forwarding may be initiated as soon as the source gNB-CU receives the DU-CU CELL SWITCH NOTIFICATION message from source gNB-DU in step 25.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC(AMF) to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC(AMF) confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the 5GC(AMF), the target gNB-CU may send a message (e.g., UE CONTEXT RELEASE) to inform the source gNB-CU about the success of the handover if no LTM candidate cell(s) exist in the source gNB. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

37.	The source gNB-CU can send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

38.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

39.	For the subsequent LTM, if early data forwarding is applied, the new source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 3: The steps 18-39 can be performed multiple times for subsequent LTM cell switch executions.

From the above overview of inter-gNB-CU LTM procedure, the impacts on Xn interface mainly reflected in the following aspects:

- Regarding the initiation of LTM, we already reached the agreement Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. However, need FFS on the detail information in HO REQ/ACK. Following similar enhancements for F1AP UE Context Setup Request/Response message in Rel-18 Intra-CU LTM procedure, we propose the following.

Proposal 1: Enhance Handover Request and Handover Request Acknowledge message with below related LTM information included for the LTM preparations. FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the HO REQ/ACK.

- HO REQ message to include:

  - LTM Information Request IE

    - LTM indicator = initiation

    - LTM reference configuration (if available), or its request.

    - SSB / CSI resource configuration of candidate cells (if available)

  - LTM configuration ID mapping list

  - Early Sync Information Request

- HO REQ ACK message to include:

  - LTM Information Acknowledge IE

    - LTM reference configuration if requested

    - Complete configuration indicator

    - SSB / CSI resource configuration allocated by the requested candidate cell

  - Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell.

- For supporting subsequent inter-CU LTM, in RAN2#126 meeting, RAN2 reached the following agreements and some details:

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Based on the agreements from RAN2, we propose the following.

Proposal 2: Introduce a Class-1 UE associated XnAP message to transfer LTM configuration information of accepted candidate cells to other each candidate gNB for subsequent LTM with below related LTM information included (FFS whether to reuse existing message, i.e., HANDOVER REQUEST with new code-point or to define a new message, e.g., HANDOVER/LTM MODIFICATION REQUEST/ACKNOWLEDGE) and FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the XnAP message.

- HO/LTM MOD REQ or HO REQ message to include:

  - LTM Information Modification IE

    - LTM indicator = modification

    - SSB/CSI resource configuration allocated for all the accepted candidate cells

    - LTM reference configuration

    - LTM configuration ID mapping list

  - Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

- HO/LTM MOD REQ ACK or HO ACK message to include:

  - LTM Information Acknowledge IE

    - the updated lower layer configuration (e.g., the updated CSI report configuration)

    - Complete configuration indicator for each admitted candidate cell

- Regarding early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU, in step20, the candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via a new XnAP message. We already reached the agreement Introduce a new procedure on Xn to transfer the TA information. Due to R18 F1AP DU-CU/CU-DU TA INFORMATION TRANSFER message is a non-UE associated message, to inherit the legacy principle, we suggest this XnAP message is also specified a non-UE associated message. Furthermore, following the current detail IEs in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, we have a below proposal.

Proposal 3: Introduce a new non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB with below related LTM information included ( e.g., LTM TA INFORMATION TRANSFER).

- XnAP LTM TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID

- Regarding cell switch notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already reached the agreement A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION, since TA values may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM , we have a below proposal.

Proposal 4: XnAP LTM Cell Switch Notification message can forward the valid TA values of candidate cells for subsequent LTM.

- Regarding the remaining the issue on FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB, our view is that when UE successfully accesses to target gNB via LTM, if no LTM candidate cell(s) exist in the source gNB for subsequent LTM, source gNB need release radio and C-plane related resources associated to the UE context. Following the current inter-gNB handover procedure, propose to reuse UE CONTEXT RELEASE message due to no impact to standard.

Proposal 5: Reuse the existing XnAP UE CONTEXT RELEASE message to inform the source gNB if no LTM candidate cell(s) exist in the source gNB.

Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM in SA and propose some enhancements for Xn interface in below aspects. Therefore we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

LTM Preparation

Proposal 1: Enhance Handover Request and Handover Request Acknowledge message with below related LTM information included for the LTM preparations. FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the HO REQ/ACK.

- HO REQ message to include:

  - LTM Information Request IE

    - LTM indicator = initiation

    - LTM reference configuration (if available), or its request.

    - SSB / CSI resource configuration of candidate cells (if available)

  - LTM configuration ID mapping list

  - Early Sync Information Request

- HO REQ ACK message to include:

  - LTM Information Acknowledge IE

    - LTM reference configuration if requested

    - Complete configuration indicator

    - SSB / CSI resource configuration allocated by the requested candidate cell

  - Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell.

LTM Configuration Transfer for Subsequent LTM

Proposal 2: Introduce a Class-1 UE associated XnAP message to transfer LTM configuration information of accepted candidate cells to other each candidate gNB for subsequent LTM with below related LTM information included (FFS whether to reuse existing message, i.e., HANDOVER REQUEST with new code-point or to define a new message, e.g., HANDOVER/LTM MODIFICATION REQUEST/ACKNOWLEDGE) and FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the XnAP message.

- HO/LTM MOD REQ or HO REQ message to include:

  - LTM Information Modification IE

    - LTM indicator = modification

    - SSB/CSI resource configuration allocated for all the accepted candidate cells

    - LTM reference configuration

    - LTM configuration ID mapping list

  - Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

- HO/LTM MOD REQ ACK or HO ACK message to include:

  - LTM Information Acknowledge IE

    - the updated lower layer configuration (e.g., the updated CSI report configuration)

    - Complete configuration indicator for each admitted candidate cell

Early TA Info Transfer in Xn Interface

Proposal 3: Introduce a new non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB with below related LTM information included ( e.g., LTM TA INFORMATION TRANSFER).

- XnAP LTM TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID

Cell Switch Notification

Proposal 4: XnAP LTM Cell Switch Notification message can forward the valid TA values of candidate cells for subsequent LTM.

Source gNB Release Notification

Proposal 5: Reuse the existing XnAP UE CONTEXT RELEASE message to inform the source gNB if no LTM candidate cell(s) exist in the source gNB.