3GPP TSG-RAN WG3 #125                                                                       R3-244561

Maastricht, NL, 19th – 23rd Aug 2024

Agenda item:	13.2

Source:	Samsung

Title:	(TP to BLCR for TS 38.401/38.300) Procedure for Inter-CU/gNB LTM

Document for:	Approval

Introduction

The discussion paper is in R3-244559. It is the text proposal to introduce the procedure for inter-gNB-CU/gNB LTM to the TS38.401/38.300 baseline CR.

Text Proposal to 38.401

============ Start of the change ==============

### 8.9.x	Inter-gNB-CU LTM

Figure 8.9.x-1 shows the overall procedures used for inter-gNB-CU LTM involving F1.

Figure 8.9.x-1: Inter-gNB-CU LTM procedure

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	Source gNB-CU sends a HANDOVER REQUEST message to the candidate gNB-CU(s) for each candidate cell to request LTM configurations, which carries a LTM Information Request IE containing a LTM Indicator, LTM Reference Configuration (if available) or an indication of its request, SSB/CSI Resource Configuration of candidate cells (if available), LTM configuration ID mapping list, and Early Sync Information Request.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations, containing one target candidate cell ID, LTM configuration ID mapping list, and the CSI resource configuration for subsequent LTM. The candidate gNB-CU may request PRACH resources for early TA acquisition from the candidate gNB-DU(s). The candidate gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations and the allocated PRACH resource for early TA acquisition for the accepted target candidate cell.

6.  If the candidate gNB-CU accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE including LTM configuration information of LTM candidate cell to the source gNB-CU, FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs, which may contain LTM reference configuration if requested or Complete Configuration Indicator, SSB/CSI resource configuration allocated by the requested candidate cell, Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell. The HANDOVER REQUEST ACKNOWLEDGE message is sent for each candidate cell.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB-CU may send a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST with new code-point or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST) to the candidate gNB(s) containing the information for subsequent LTM, which may contain SSB/CSI resource configuration allocated for all the accepted candidate cells, LTM Reference Configuration(if available), LTM configuration ID mapping list, and Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

10.	The candidate gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration (e.g., the updated CSI report configuration).

12.	The candidate gNB-CU responds with a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST ACKNOWLEDGE or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST ACKNOWLEDGE) including the updated lower layer configuration (e.g., the updated CSI report configuration).

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed. UE performs early TA acquisition with the candidate cell(s) as requested by source gNB-DU before receiving the cell switch command. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via a new non-UE associated XnAP message (FFS on message name, e.g., LTM TA INFORMATION TRANSFER).

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE sends the L1 measurement result to the source gNB-DU.

23.	The source gNB-DU decides to execute LTM to a target cell.

24.	The source gNB-DU sends the Cell Switch command to the UE.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to target gNB-CU in the LTM CELL SWITCH NOTIFICATION message.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32.	The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell.

33.	The source gNB-CU sends the SN STATUS TRANSFER message for the late data forwarding.

NOTE 2: Late data forwarding may be initiated as soon as the source gNB-CU receives the DU-CU CELL SWITCH NOTIFICATION message from source gNB-DU in step 25.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC(AMF) to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC(AMF) confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the 5GC(AMF), the target gNB-CU may send a message (e.g., UE CONTEXT RELEASE) to inform the source gNB-CU about the success of the handover if no LTM candidate cell(s) exist in the source gNB. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

37.	The source gNB-CU can send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

38.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

39.	For the subsequent LTM, if early data forwarding is applied, the new source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 3: The steps 18-39 can be performed multiple times for subsequent LTM cell switch executions.

============ End of the change ==============

Text Proposal to 38.300

============ Start of the change ==============

### 9.2.3.5.x	Inter-gNB LTM

Figure 9.2.3.5.x-1 shows the overall procedures used for inter-gNB LTM involving Xn.

Figure 9.2.3.5.x-1: Inter-gNB LTM procedure

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB.

2.	The source gNB determines to initiate inter-gNB LTM configuration.

3.	Source gNB sends a HANDOVER REQUEST message to the candidate gNB(s) for each candidate cell to request LTM configurations, which carries a LTM Information Request IE containing a LTM Indicator, LTM Reference Configuration (if available) or an indication of its request, SSB/CSI Resource Configuration of candidate cells (if available), LTM configuration ID mapping list, and Early Sync Information Request.

4.  Admission Control may be performed by the target gNB.

5.	If the candidate gNB-CU accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE including LTM configuration information of LTM candidate cell to the source gNB-CU, FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs, which may contain LTM reference configuration if requested or Complete Configuration Indicator, SSB/CSI resource configuration allocated by the requested candidate cell, and Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell. The HANDOVER REQUEST ACKNOWLEDGE message is sent for each candidate cell.

6.	The source gNB may send a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST with new code-point or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST) to the candidate gNB(s) containing the information for subsequent LTM, which may contain SSB/CSI resource configuration allocated for all the accepted candidate cells, LTM Reference Configuration(if available), LTM configuration ID mapping list, and Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

7.	The candidate gNB responds with a UE associated XnAP message (FFS whether to reuse existing message, i.e., HANDOVER REQUEST ACKNOWLEDGE or to define a new message, e.g., LTM/HANDOVER MODIFICATION REQUEST ACKNOWLEDGE) including the updated lower layer configuration (e.g., the updated CSI report configuration).

8.	The source gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

9.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

10. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

11.	Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	The candidate gNB forwards the TA value and the associated CFRA resource information to source gNB via a new XnAP signaling (FFS on message name, e.g., LTM TA INFORMATION TRANSFER).

13.	The UE sends the L1 measurement result to the source gNB.

14.	The source gNB decides to execute LTM to a candidate target cell.

15.	The source gNB sends the Cell Switch Command to the UE via MAC CE.

16.	The source gNB forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to target gNB in the LTM CELL SWITCH NOTIFICATION message.

17.	The target gNB detects the UE access as specified in TS 38.300 [2].

18.	The UE sends an RRCReconfigurationComplete message to the target gNB.

19.	The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell.

20.	The source gNB sends the SN STATUS TRANSFER message for the late data forwarding.

NOTE 2: Late data forwarding may be initiated as soon as the source gNB sends the Cell Switch Command to the UE via MAC CE in step 15.

21. The target gNB sends a PATH SWITCH REQUEST message to 5GC(AMF) to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

22.	The 5GC(AMF) confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB.

23.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the 5GC(AMF), the target gNB may send a message (e.g., UE CONTEXT RELEASE) to inform the source gNB about the success of the handover if no LTM candidate cell(s) exist in the source gNB. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

24.	For the subsequent LTM, if early data forwarding is applied, the new source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 3: The steps 11-24 can be performed multiple times for subsequent LTM cell switch executions.

============ End of the change ==============