3GPP TSG-RAN WG3 #125								R3-244562

19th – 23th Aug 2024, Maastricht, NL

Agenda Item:	12.2

Source:	China Telecom

Title: 	On WAB Awareness and Authorization

Document for:	Discussion

Introduction

The additional topological enhancements for NR were agreed in RAN#102 meeting [1]. In RAN3#123bis meeting, the following agreements and open issues on authorization were made:

- Whether a BH RAN node can be unaware of WAB needs to be further studied.

- RAN3 understands that authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

- WAB-MT authorization provides the WAB MT with the right to support BH PDU sessions.

- In case MT authorization is based on slice, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed. SA2 to decide whether MT authorization based on slicing is supported.

- Authorization of WAB-gNB provides service authorization, i.e., the right to serve UEs.

- It is RAN3 understanding that WAB-gNB’s authorization status can change.

- The UEs served by a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released.

- The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. Any Stage 3 decisions are to be taken during normative work.

In SA2#163 meeting, the conclusions of awareness and authorization were captured in TR23.700-06. In addition, we understand the RRC state supported by WAB-MT and corresponding operation still need to be studied. Due to the time limitation for SI phase, we propose our initial views in this paper and further discussion could be considered in WI phase. To sum up, this contribution aims to further discuss the leftover issues on WAB awareness and authorization.

Discussion

RRC States

According to the following RAN3 agreements, the WAB-MT supports a subset of UE functionalities:

The WAB-MT supports at least a subset of UE functionalities.

We understand WAB-MT like NCR-MT and IAB-MT also need to support all the existing RRC states, namely Connected, Idle, and Inactive. In Connected state, the WAB-gNB could broadcast MIB and SIB message in air interface and establish Xn and NG interface with BH gNB/5GC. Regarding the action of WAB-gNB for Idle and Inactive state, there was not discussed in the past RAN3 and RAN2 meetings. Upon the WAB-MT transit from Connected state to Idle state, the WAB-gNB ceases to broadcast MIB and SIB message and remove Xn/NG interface, and redirection all UEs to other NG-RAN nodes or E-UTRAN nodes. For Inactive state, whether WAB-gNB ceases to broadcast MIB and SIB message need to be studied in RAN2 WI phase. Additionally, considering the WAB-MT supports subset of UE functionalities, it also needs to support RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR). The action of WAB-gNB during RLF or Beam Failure need to be further discussed in RAN2 WI phase.

Proposal 1: WAB-MT like NCR-MT and IAB-MT also need to support all the existing RRC states, namely Connected, Idle, and Inactive.

Proposal 2: Upon the WAB-MT transit from Connected state to Idle state, the WAB-gNB ceases to broadcast MIB and SIB message and remove Xn/NG interface, and redirection all UEs to other NG-RAN nodes or E-UTRAN nodes.

Proposal 3: For Inactive state, whether WAB-gNB ceases to broadcast MIB and SIB message need to be studied in RAN2 WI phase.

Proposal 4: WAB-MT needs to support RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR). The action of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

WAB Awareness for BH RAN

In last RAN3 meeting, one open issue needs to be further studied is whether a BH RAN node can be unaware of WAB. The WAB awareness information could provide useful information for BH RAN to configure dedicated resource for WAB-MT or WAB-gNB. However, from the perspective of signalling enhancement, we prefer to not introduce any new indicator over air interface. According to the objective of SI on WAB, WAB is expected to not impact on air interface. Additionally, SA2 had also concluded that there is no need to specify a new indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB. Regarding whether new UE capability is needed for WAB-MT, we understand that BH RAN node does not have additional operation for WAB-MT and it also do not need to really know what slices are used for WAB operation although SA2 agreed to configure dedicated slices for a MWAB-UE to indicate the intension to BH 5GC. Moreover, the authorization information for WAB-MT can be send via NAS signalling. To sum up, BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

Proposal 5: BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

WAB authorization

According to the conclusion of SA2, the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs. Therefore, it is need to introduce a new indicator in NG Setup Request/RAN Configuration Update Message to indicate it is a WAB node. This indicator cannot be changed after NG interface was established. Without this new indicator contained in NG Setup Request/RAN Configuration Update Message, the AMF will take this node as legacy gNB.

Proposal 6: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

According to RAN3 agreements in last meeting, authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. As regards whether SCTP connection for Xn and NG interface is still maintained, we understand it depends on pre-configuration or OAM configuration. If the WAB-MT authorization status changes from “not authorized” to “authorized” , since BH PDU session and its corresponding DRBs resource can be reallocated to WAB-gNB, it could perform the normal WAB-gNB setup procedure.

Proposal 7: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the authorization information can be delivered via NGAP message., NG Setup Response/RAN Configuration Update ACK message. The WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. In this case, the SCTP connection for Xn and NG interface should be also removed. While the authorization information is triggered by OAM, the WAB-gNB could also not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. Since the BH PDU session can be still kept and, the SCTP connection can be maintained.

Proposal 8: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: WAB-MT like NCR-MT and IAB-MT also need to support all the existing RRC states, namely Connected, Idle, and Inactive.

Proposal 2: Upon the WAB-MT transit from Connected state to Idle state, the WAB-gNB ceases to broadcast MIB and SIB message and remove Xn/NG interface, and redirection all UEs to other NG-RAN nodes or E-UTRAN nodes.

Proposal 3: For Inactive state, whether WAB-gNB ceases to broadcast MIB and SIB message need to be studied in RAN2 WI phase.

Proposal 4: WAB-MT needs to support RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR). The action of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

Proposal 5: BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

Proposal 6: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

Proposal 7: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

Proposal 8: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Reference

- RP-234041, New SID: Study on additional topological enhancements for NR，AT&T (moderator, RAN VC)

- R3-243021 LS from SA2