3GPP TSG-RAN WG3 #125								R3-244563

19th – 23th Aug 2024, Maastricht, NL

Agenda Item:	12.2

Source:	China Telecom

Title: 	On Xn Connection Management

Document for:	Discussion

Introduction

The additional topological enhancements for NR were agreed in RAN#102 meeting [1]. In RAN3#123bis meeting, the following agreements and open issues on Xn connection management were made:

- Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

- Addition of new information in the legacy procedures is not precluded

In this contribution aims to further discuss the leftover issues on Xn Connection Management.

Discussion

According to RAN3 agreement in last meeting, WAB-gNB can establish Xn interface with its neighbouring nodes including BH RAN node. In order to trigger a Xn interface establishment procedure, the WAB needs to have the information of surrounding nodes including PLMN ID List, IP address, gNB-ID, and etc. We understand at least one Xn interface could be established between WAB-gNB node and BH gNB node. The WAB-MT can acquire PLMN ID List and gNB-ID by reading the system information from BH gNB cell. The IP address of BH gNB node can be discovered via legacy Xn-C TNL address discovery procedure specified in TS38.300 clause 15.3.4. In practical network, whether the destination IP address can be used to establish Xn interface rely on many restrictive conditions. Therefore, OAM can provide necessary guideline or information to WAB-gNB to indicate whether the Xn interface between WAB and BH node can be established. The leftover issue is how to acquire the configuration information from other surrounding nodes and establish Xn interface. There are three Options as follow:

- Option 1: ANR based solution. The BH node can configure WAB-MT to read the system information of surrounding nodes, and then the WAB-MT provide the acquired system information to WAB-gNB. Since the BH node may not aware of WAB, the legacy ANR function may not be configured to WAB-MT as the neighbouring relation for BH node were already configured.

- Option 2: neighbour cell information exchange via Xn interface. The WAB-gNB can acquire the cell configuration of surrounding NG-RAN node via the established Xn interface between WAB-gNB and the BH-RAN-node. This solution is a simple and straightforward solution. The only restriction is that the Xn interface between WAB-gNB and the BH-RAN-node may not exit in some cases and this option could not work.

- Option 3: OAM based solution. OAM can provide the configuration information for neighbour cells based on location information.

Proposal 1: In order to support Xn setup with surrounding NG-RAN nodes, following options are proposed:

- Option 2: neighbour cell information exchange via Xn interface.

- Option 3: OAM based solution.

Since the WAB node support the mobility procedure which unlike the legacy fixed node, it is need to support automatically remove neighbour relation and Xn Removal procedure. The WAB-gNB can determine whether to remove the Xn connection with one surrounding node based on location information or OAM configuration. Additionally, if the WAB-MT or WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB needs to ceases broadcast and remove Xn/NG connection. In this scenario, Xn Connection still needs to remove. Therefore, in order to differential the cause for Xn removal, it is need to introduce a new cause value in Xn Removal Request message.

Proposal 2: in order to differential the cause for Xn removal, it is need to introduce a new cause value in Xn Removal Request message.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: In order to support Xn setup with surrounding NG-RAN nodes, following options are proposed:

- Option 2: neighbour cell information exchange via Xn interface.

- Option 3: OAM based solution.

Proposal 2: in order to differential the cause for Xn removal, it is need to introduce a new cause value in Xn Removal Request message.

Reference

- RP-234041, New SID: Study on additional topological enhancements for NR，AT&T (moderator, RAN VC)

- R3-243021 LS from SA2