3GPP TSG-RAN WG3 Meeting #125	R3-244565

Maastricht , NL, 19th – 23th Aug 2024

Title: 	TP to TR38.743 on Finer Granularity of Measured Energy Cost

Source: 	CMCC, Samsung

Agenda item: 	11.4

Document Type:	Discussion&Approval

- Introduction

In this paper we want to continue the discussion on predicated Energy Cost for AI/ML for NG-RAN Energy Saving use case, and the related procedures.

During the last RAN3 #119-bise meeting, the following agreements and FFS were captured:

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

And in RAN3 #123-bis meeting, the following agreements were captured:

Agree to have discussions on AI/ML based energy saving at the next meeting, where enhancements will be based on new solutions not discussed in Rel18. It is strongly recommended not to reiterate Rel18 proposals on which consensus could not be achieved. If consensus on a solution approach is not reached at May meeting, the topic is down prioritized.

Further discuss the ES and multi-hop use cases and solutions based on principles as approved above

Which means consensus on the enhanced solution of Energy Saving must be achieved in the RAN3#124 meeting, otherwise it will be ruled out from R19 discussions.

In RAN3 #124 meeting, it was agreed that:

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

2	Discussion

In R18, some companies think it is impossible to get the accurate Energy Cost at cell level. To make the progress and continue the discussion, it was agreed that the Measured Energy Cost should be at NG-RAN node level.

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

However, it is too coarse to measure the Energy Cost in node level. The target for AI/ML Energy Saving use case is to optimize the overall Energy Cost for a given area in real time. To achieve it, decision such as Cell activation/deactivation, Cell level Power control shall be made according to the Measured Energy Cost received from neighbouring node via Xn for the given Cell.

The coverage for a NG-RAN node is wide. It is not reasonable to deactivate the whole NG-RAN node according to the Measured Energy Cost. From network operation point of view, the node-level measured Energy Cost transferred through Xn interface is not useful for making decision/action in order to optimize energy consumption.

Proposal 1: It is beneficial to introduce cell-level Measured Energy Cost transferred via Xn interface in AI/ML for NG-RAN in R19.

In R19, some companies proposed it is reasonable and feasible to provide finer granularity Energy Cost such as for a group of cells which share the same hardware resource.

Observation 1: some companies think it is feasible to measure finer granularity Measured Energy Cost for a group of cells which share the same hardware resource.

Meanwhile, this group of cells whose EC can be measured should be very limited. They need to share the same hardware resource such as power supply,mainboard, RF, transport equipment and so on. If any of these resources are not the same or these resources are shared by some other cells, the EC for this group can’t be measured.

Observation 2: The Measured EC for a group of cells can only be collected in very strictly limited situations. In most of the real environment, it is impossible to collect the Measured EC for a group of cells.

Proposal 2:  It is not suggested to introduce and transfer the Measured EC for a special group of cells in RAN3 specification.

However, for the group of cells whose EC can be measured, we propose the following thoughts:

When NG-RAN node make decision whether to handover UE to the neighbouring cells, and then deactivate its own cell, the Energy Cost for a group of cells is still not a direct input. The risk exists when the Energy Cost for the group of cells is low but UE is handover to the most energy cost cell under the group.

Since the source NG-RAN node has no configuration information of this group of cells, the capacity, spectrum, current data volume and other factors may all impact each cell’s Energy Cost under this group.

Meanwhile for the target node, it can do a better evaluation on the split Energy Cost for this group of cells which share the same hardware resource. It can provide a more precise calculation on the Energy Cost for each cell according to the configuration and traffic status information before transferring the value to source NG-RAN node.

For example, the simplest way is to calculate the Energy Cost for each cell based on the percentage of data volume. If Cell1 carries 30% traffic among this cell group, then its Energy Cost shall be calculated as 30% of the total Energy Cost for this group.

It is not the most accurate way to evaluate the dedicated Energy Cost for a specific cell, but it is more reasonable and precise to calculate it on target node than the source node. When target node transfers this cell-level Measured Energy Cost to source node, it can be effectively used to make further strategies such as UE HO and local cell power control.

The algorithm to calculate the Energy Cost for each cell under the group is out of scope of 3GPP. But the algorithm should be reasonable and feasible according to the operation situation, and it shall be unified among the same neighbourhood. So we suggest to let OAM provision it to the NG-RAN nodes.

Proposal 3: If the Measured Energy Cost for a group of cells which share the same hardware resource is available, use it as baseline to calculate cell-level Measured Energy Cost.

Proposal 4: OAM shall configure the algorithm on target Node to calculate the Energy Cost for each cell under the same group.

Proposal 5: RAN3 to study the exchange of cell-level Measured EC via Xn between gNBs.

Proposal 6: Agree the TP in the annex related to cell level Measured Energy Cost.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN Energy Saving, following observations and proposals are made:

Proposal 1: It is beneficial to introduce cell-level Measured Energy Cost transferred via Xn interface in AI/ML for NG-RAN in R19.

Observation 1: it is reasonable and feasible to measure finer granularity Energy Cost for a group of cells which share the same hardware resource.

Observation 2: The Measured EC for a group of cells can only be collected in very strictly limited situations. In most of the real environment, it is impossible to collect the Measured EC for a group of cells.

Proposal 2:  It is not suggested to introduce and transfer the Measured EC for a special group of cells in RAN3 specification.

Proposal 3: If the Measured Energy Cost for a group of cells which share the same hardware resource is available, use it as baseline to calculate cell-level Measured Energy Cost.

Proposal 4: OAM shall configure the algorithm on target Node to calculate the Energy Cost for each cell under the same group.

Proposal 5: RAN3 to study the exchange of cell-level Measured EC via Xn between gNBs.

Proposal 6: Agree the TP in the annex related to cell level Measured Energy Cost.

Annex: TP to TS 38.743

5.3	Energy saving enhancements

<<<<<< Start of CHANGES >>>>>>

5.3.1 Use case description

In R19, under the assumption that the Measured Energy Cost for a group of cells which share the same hardware resource is available, Cell-level Measured Energy Cost may be introduced to be transferred from target NG-RAN node to source NG-RAN node, to help make decision such as UE Handover, Cell deactivation, to optimize the overall Energy Cost.

5.3.2 Solution and Potential Standard impacts

Solution to transfer the Measured Energy Cost includes:

- Target node shall measure the Energy Cost for a group of cells which share the same hareware resource.

- OAM shall configure the algorithm on how to calculate the Energy Cost for a single cell under this group.

- Transfer the cell-level Measured Energy Cost via Xn interface using the current DATA COLLECTION procedures.

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

- Introduce Cell-Level Energy Cost IE in DATA COLLECTION UPDATE message.

<<<<<< End of CHANGES >>>>>