3GPP TSG-RAN WG3 Meeting #124	R3-244566

Maastricht , NL, 19th – 23th Aug 2024

Title: 	TP to TR38.743 Discussion on AI/ML for split architecture use cases

Source: 	CMCC, ZTE，Lenovo, Qualcomm

Agenda item: 	11.4

Document Type:	Discussion and Decision

- Introduction

In R18, the following consensus are gotten on the split architecture use cases:

RAN3 will focus on non-split architecture use cases and procedures first and discuss split architecture use cases and procedures when completion for the non-split architecture use cases and procedures is achieved.

UE traffic metric takes the data volume for a UE as the starting point.

The cell-level UE trajectory prediction function is located in gNB CU-CP.

The location for resource status prediction in split architecture:

For current resource status input data from gNB DU, the resource status prediction function is located in gNB CU-CP.

For current resource status input data from gNB CU-UP, the resource status prediction function is located in gNB CU-CP.

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

In RAN3 123-bis meeting, the following information is captured in chairman notes:

- Formulate the use case and solution for split architecture support, NR-DC, and continuous MDT use cases

So split architecture support for the 3 R18 use cases shall be discussed in RAN3 #124.

And in RAN3 124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

Measured Energy Cost can be transferred from DU to CU.

.2	Discussion

In R18, RAN3 focused first on non-split architecture use cases and procedures, and split RAN architecture are not pursued. And in R19, we will continue the discussion on split architecture use cases and procedures.

In R18, three use cases: Load Balance, Mobility Enhancement and Energy Saving are considered and discussed:

- For Load balance, it has been agreed to transfer Measured UE performance from CU-UP to CU-CP.

- For Energy Saving use case, it has been agreed that Measured Energy Cost can be transferred from DU to CU.

Except these local data, we also think the following shall be discussed:

2.1 Load Balance

- For load balance, the UE performance feedback may be requested through Xn after HO, which includes Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss. The Throughput DL and Throughput UL are measured in gNB DU, Packet Loss DL is measured in gNB DU, but Packet Loss UL is measured in gNB CU-UP, while Packet Delay involve measurements from both gNB DU and gNB CU-UP.

Observation 1: For load balance use case, UE performance data are collected both from gNB DU and gNB CU-UP. The prediction function and feedback through Xn are agreed to be located in gNB CU-CP.

Proposal 1: Except Transferring Measured UE performance from CU-UP to CU-CP, it is also beneficial to transfer UE performance data from gNB DU to gNB CU through F1 interface.

2.2 Mobility Enhancement

- It has been agreed that the prediction function of UE trajectory is located in gNB CU-CP. The prediction of UE trajectory may need to collect current UE traffic metric locally.

- The gNB needs predicted UE traffic (data volume) as input for AI/ML Training and Inference. To predict UE traffic, gNB CU-CP needs to collect current UE traffic metric(data volume) from gNB CU-UP and gNB DU.

For mobility enhancement, UE traffic metric shall be reported to gNB CU. It has been agreed to use data volume as the start point for UE traffic metric, the current DL/UL PDCP SDU Data Volume are measured in gNB CU-UP. The cell-level UE trajectory prediction function is agreed to be located in gNB CU-CP.

Proposal 2: It is beneficial to transfer the UE traffic (data volume) from gNB CU-UP to gNB CU-CP through E1 interface. gNB CU-CP shall use the UE traffic (data volume) as input for UE traffic predication and UE trajectory predication.

2.3 Procedure Design over E1 and F1 interfaces

In #121-bis, the WA has been achieved as following:

WA: Defining new procedures to transfer the measured EC over F1 follows the design over Xn.

From above discussion, we can see not only Energy Cost, but also the other information needs to be transferred from gNB DU, gNB CU-UP to gNB CU-CP. Currently, only resource status can be transferred in existing Resource Status Request Initiation and Reporting procedures over E1 and F1 interfaces.

The procedure over F1 and E1 for AI/ML data collection is very similar as the procedures over Xn, also the data collected over F1 and E1 is related to the ones over Xn. So it is straightforward to introduce new procedures over F1 and E1 for Data Collection, and follow the current design over Xn.

To follow the design over Xn interface, the new introduced procedures over E1 and F1 interfaces shall include the Request, Response/Failure and Update messages. Different measurement shall be supported using different code points in the report characteristics request, but they can share the same Request and Update message. Both one-time and periodic collections shall be supported.

Proposal 3: It is proposed to turn the WA into agreement, to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU and gNB DU over F1 interface, to transfer UE performance, energy consumption data. Similar as the procedures designed over Xn interfaces.

Proposal 4: It is proposed to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU-UP and gNB CU-CP over E1 interface, to transfer UE performance and UE traffic (data volume). Similar as the procedures designed over Xn interfaces.

Proposal 5: Agree the TP in the annex related to split architecture use cases.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN split architecture use cases, following observations and proposals are made:

Observation 1: For load balance use case, UE performance data are collected both from gNB DU and gNB CU-UP. The prediction function and feedback through Xn are agreed to be located in gNB CU-CP.

Proposal 1: Except Transferring Measured UE performance from CU-UP to CU-CP, it is also beneficial to transfer UE performance data from gNB DU to gNB CU through F1 interface.

Proposal 2: It is beneficial to transfer the UE traffic (data volume) from gNB CU-UP to gNB CU-CP through E1 interface. gNB CU-CP shall use the UE traffic (data volume) as input for UE traffic predication and UE trajectory predication.

Proposal 3: It is proposed to turn the WA into agreement, to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU and gNB DU over F1 interface, to transfer UE performance, energy consumption data. Similar as the procedures designed over Xn interfaces.

Proposal 4: It is proposed to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU-UP and gNB CU-CP over E1 interface, to transfer UE performance and UE traffic (data volume). Similar as the procedures designed over Xn interfaces.

Proposal 5: Agree the TP in the annex related to split architecture use cases.

Annex TP for TR 38.743

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[3]                       3GPP TS 38.473: “F1 application protocol (F1AP)”.

[4]                       3GPP TS 37.483: “E1 Application Protocol (E1AP)”.

# 5	Rel-18 Leftovers and solutions

5.2	Split architecture support for Rel-18 use cases

Editor Note: Capture the description and its potential standard impacts.

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

-	Measured UE performance from gNB-CU-UP to gNB-CU-CP.

- Measured UE traffic(data volume) from gNB-CU-UP to gNB-CU-CP.

- Introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU-UP and gNB CU-CP over E1 interface, to transfer UE performance and UE traffic (data volume). Similar as the procedures designed over Xn interfaces.

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU.

-    Measured UE performance from gNB-DU to gNB-CU.

-    Introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB CU and gNB DU over F1 interface, to transfer UE performance, energy consumption data. Similar as the procedures designed over Xn interfaces.