3GPP TSG-RAN WG3 Meeting #125	R3-244568

Maastricht , NL, 19th – 23th May 2024

Title: 	Discussion on Supporting MBS broadcast service for NR NTN

Source: 	CMCC

Agenda item: 	14.2

Document Type:	Discussion and Decision

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775.

For RAN3, one of the objectives is to specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

During RAN3 #123-bis, agreements were achieved as following:

RAN3 starts work on MBS broadcast service.

MBS Broadcast service delivery area over NG:

- Mapped cell ID/list in MBS service area

- New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Based on GNSS description

To be continued...

And During RAN3 #124, consensus was gotten on the following working assumption:

RAN3#124:

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

2	Discussion

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service.

In this paper, we will discuss the granularity of intended service area.

Currently in TS38.413 for NGAP, the MBS service area information is defined as below：

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS Service Area Information | M |  | 9.3.1.209 |  |
| >location dependent |  |  |  |  |
| >>MBS Service Area Information List |  | 1 |  |  |
| >>>MBS Service Area Information Item |  | 1..<maxnoofMBSServiceArea Information> |  |  |
| >>>>MBS Area Session ID | M |  | 9.3.1.207 |  |
| >>>>MBS Service Area Information | M |  | 9.3.1.209 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSServiceAreaInformation | Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256. |

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |

And the NR CGI IE is used to globally identify an NR cell (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) | The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.3.1.6). |

And in TS38.300, it has been defined in Non-Terrestrial Networks NG-RAN signalling that:

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

Observation 1: It has been clearly defined that for NTN PWS service, cell ID included in MBS service area corresponds to the mapped cell.

Currently NTN MBS broadcast service is similar with PWS, they are aimed to send to a specific geographical location, which can be indicated by a set of NR cells, and presented by Mapped Cells.

Proposal 1: It is proposed to change the WA into agreement. Clarify in TS 38.413 that cell ID included in MBS service area corresponds to the Mapped Cell ID, to indicate the intended service area in RAN for NTN broadcast service, the same way as current PWS.

Observation 2: Whether to introduce smaller granularity of MBS service area should be defined according to the service requirement, not the granularity in UE. So RAN3 don’t need to wait for RAN2 to start the discussion.

RAN3 can begin the discussion to introduce more information in Intended Service Area, but need clear use cases to decide what and how to include the extra information than Mapped Cell ID.

Proposal 2: It is proposed to discuss the use cases where the intended service area may be smaller than NTN Mapped cells.

In some scenarios, the MBS broadcast intended geographical location can’t be exactly mapped to Cell coverage area. Especially when a NTN cell’s coverage is much more larger than the intended MBS broadcast service’s geographical area. For example, a NTN cell covers multiple countries. And the MBS broadcast service is only intended to a specific country’s geographical location in the area. UE may roam to another PLMN which is also covered by the same cell, then AMF may need to carry extra information such as GNSS description to RAN, to indicate the intended service area.

Also in the future if more solution needs to be supported such as Store&Forward for IoT NTN, or NG suspend/resume or Xn Handover enhancements for specific UEs. The small granularity location information in MBS Service Area can help the satellite (gNB on board) make the decisions to choose different solution/enhancement for a given UE. These UE may be under the same cell coverage area but needs different solutions according to their geometry location. So we think introduce GNSS description information can assistant the gNB do UE associated enhancements.

Proposal 3: It is beneficial to introduce GNSS description information IE under MBS Service Area Information Item IE, to provide smaller granularity service area than cell level.

3	Summary

In this contribution, we provide considerations on the NTN broadcast service and PWS service in RAN3, following observations and proposals are made:

Observation 1: It has been clearly defined that for NTN PWS service, cell ID included in MBS service area corresponds to the mapped cell.

Observation 2: Whether to introduce smaller granularity of MBS service area should be defined according to the service requirement, not the granularity in UE. So RAN3 don’t need to wait for RAN2 to start the discussion.

Proposal 1: It is proposed to change the WA into agreement. Clarify in TS 38.413 that cell ID included in MBS service area corresponds to the Mapped Cell ID, to indicate the intended service area in RAN for NTN broadcast service, the same way as current PWS.

Proposal 2: It is proposed to discuss the use cases where the intended service area may be smaller than NTN Mapped cells.

Proposal 3: It is beneficial to introduce GNSS description information IE under MBS Service Area Information Item IE, to provide smaller granularity service area than cell level.

4	Reference

[1] 	TS 38.413, NGAP

[2]       TS 23.247, Architectural enhancements for 5G multicast-broadcast services

[3]       TS 38.300, NG-RAN stage2