3GPP TSG-RAN WG3 Meeting #125	R3-244569

Maastricht , NL, 19th – 23th Aug 2024

Title: 	Discussion on Support of regenerative payload for NR NTN

Source: 	CMCC

Agenda item: 	14.3

Document Type:	Discussion and Decision

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775. One of the objectives is support of regenerative payload [RAN3, RAN2, RAN4]:

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

Based on it, in RAN3 #124 meeting, the following conclusions are captured:

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue.

Continue to discuss the NG interface management solution based on requirement inputs from operators

2	Discussion

So based on the conclusion from RAN3#124, in this contribution, we will focus on the discussion of NG interfaces management solution based on requirement inputs from operators.

2.1 NG Removal/Setup Procedures Enhancement

For the Regenerative Payload mode, the satellite moves and the coverage area for gNB node on board changes. So the gNB node on board shall connect to multiple AMF in different time according to various AMF serving areas. And the gNB node on board will connect to the same AMF periodically. The frequency in which gNB node changes the AMF depends on the trajectory and velocity of satellite. To let the gNB node quickly connect to AMF and set up link for UE, during RAN3#123-bis and #124, there are a lot of discussions on whether to introduce NG Release/Setup and NG Suspend/Resume procedures.

Currently when the gNB moves out the service area of AMF, from gNB and AMF point of view, the NG connection between them is still existing until the feeder link (transport layer) disconnects. Then AMF will tear down SCTP, terminate the NG connection and remove all the related UE context and RAN context. And when the gNB on board returns to the service are of this AMF, gNB will trigger NG setup procedures to AMF again.

This legacy procedure works well for TN, also for most of the NTN scenarios. However for the Regenerative Payload mode, the satellite may move fast and change AMF frequently. And the feeder link is not stable. So it is more possible that the gNB is out of connection with AMF because of the transport layer outage, but gNB and AMF are not aware of it yet until TNL timeout. In this scenario, UE outage can be avoided by the active NG release and setup from gNB. When gNB is going to move out of the service area of AMF, gNB shall initialize the NG release procedure actively and AMF can update the RAN correspondingly.

Proposal 1: It is beneficial to introduce NG Release procedure from gNB to AMF when gNB is going to leave AMF serving area after UEs handover to new gNB. The active NG release from gNB to AMF may reduce the UE outage for NR NTN Regenerative Payload mode.

In TS38.413, NG reset procedure has been defined from gNB to AMF, to release the NG connections. The legacy NG reset procedures to remove the NG connections between AMF and gNB on board can be reused. gNB on board can switchover UEs through Xn or through AMF request. After UE Handover execution succeeds, the gNB shall send NG reset to AMF, and AMF sends NG reset response to the gNB. Then gNB and AMF can remove the UE associated NG connection.

If all the UE have been handovered to new gNB, the gNB can send NG reset to AMF and release all NG connections with the AMF at once.

Another advantage to use NG reset from gNB to AMF procedures to implement NG Release is that partial UE associated NG connections reset is supported. Since the satellite gNB usually covers a wide area, it may take several hours from the satellite moves into an AMF service area to it moves out. In this scenario, some UE on the boarder should first be changed to new gNB while others can still stay on the current gNB. Using the legacy NG reset procedures can manage these NG connections for different UEs more effectively and precisely, which reduces the signal interworking and improve the energy efficiency and satellite resource usage.

When to initiate NG release procedure, and whether for all UEs at the same time are up to the gNB’s implementation.

Proposal 2: It is proposed to re-use the legacy NG reset procedures from gNB to AMF to release the NG connections between AMF and gNB on board.

Proposal 3: In some scenarios where the satellite gNB moves out of AMF for a long time, the NG reset for partially UE associated NG connections procedures from gNB to AMF can be used.

Meanwhile, when the gNB on board re-enters into the AMF serving area, it shall setup NG connection automatically. The legacy NG Setup procedure can be reused.

Proposal 4: It is proposed to re-use the legacy NG Setup procedures from gNB to AMF.

Scenario 1: UE transfer from gNB1 to gNB2 through Xn when it is leaving the AMF serving area, NG Release/Setup

Scenario 2: UE transfer from gNB1 to gNB2 through AMF when it is leaving the AMF serving area, NG Release/Setup

2.2 NG Suspend/Resume Procedures Enhancement

To recover the UE connections quickly, some companies suggest to introduce the NG Suspend and Resume Procedures.  When the satellite is moving out of the service area of AMF, it switches over UEs to the new gNB through Xn or through AMF. After UE Handover execution, the gNB can sends NG Suspend procedures to AMF and restore the UE context such as UE ID, PDU session ID and some others in the satellite (gNB on board), NG connections will be suspended. And when the satellite moves back to the previous AMF, it can quickly set up NG, RRC connections for the UE through NG Resume messages.

Proposal 5: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

The NG-RAN node ID, UE ID, PDU Session ID shall be carried in NG Resume message from NG-RAN node to AMF, to quickly recover the NG connection.

When AMF receive this NG Resume message, it shall check the UE ID and PDU session ID in its own UE context information, and know which PDU sessions are still active and shall be resumed.

Proposal 6: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Scenario 3: UE transfer from gNB1 to gNB2 through Xn when it is leaving the AMF serving area, NG Suspend/Resume

Scenario 4: UE transfer from gNB1 to gNB2 through AMF when it is leaving the AMF serving area, NG Suspend/Resume

However, some satellite companies think the storage and energy resource is very expensive and limited in satellite, store the so many UE context in satellite for such a long time cost too much.

Also some other companies think the UE won’t stay so long under the same coverage area for after the satellite moves back.  It is meaningless to store the UE context in satellite for such a long time.

So it is better to let the satellite decides whether it shall active the NG Suspend/Resume Procedures for quick recovery according to its own resource status and its moving trajectory.

Proposal 7：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

2.3 NG Pre-Set Up Procedures Enhancement

In above procedures, to guarantee the continuity of UE service, the new satellite (gNB on board) needs to set up NG connection with AMF before the previous satellite (gNB on board) is leaving the AMF serving area.

Usually, if the ephemeris is provisioned, AMF know when to initiate the setup of NG connections with satellites in advance. If not, satellite can be triggered to setup NG with AMF in advance according to time and location condition which is provisioned, before the previous satellite leaves the AMF serving area.

The procedures for NG pre-setup we proposed can refer to the NTN time or location triggered CHO procedures via Xn. Time-based or location-based triggerment may be configured independently for NG pre-setup in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible.

Here the satellite (gNB on board) still keeps the NG connections with the previous AMF when it sets up the NG connections with the new AMF. The old NG connections will be released or deleted after UE HO, as procedures proposed in chapter 2.1 and 2.2.

Proposal 8: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

3	Summary

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: It is beneficial to introduce NG Release procedure from gNB to AMF when gNB is going to leave AMF serving area after UEs handover to new gNB. The active NG release from gNB to AMF may reduce the UE outage for NR NTN Regenerative Payload mode.

Proposal 2: It is proposed to re-use the legacy NG reset procedures from gNB to AMF to release the NG connections between AMF and gNB on board.

Proposal 3: In some scenarios where the satellite gNB moves out of AMF for a long time, the NG reset for partially UE associated NG connections procedures from gNB to AMF can be used.

Proposal 4: It is proposed to re-use the legacy NG Setup procedures from gNB to AMF.

Proposal 5: It is beneficial to introduce NG Suspend/Resume procedures in some scenarios, to help the NG connection recover quickly.

Proposal 6: It is proposed to carry the NG-RAN node ID, UE ID, PDU Session ID in NG Resume message from NG-RAN node to AMF. Using these information, AMF can check with its own UE context information and know which PDU sessions shall be resumed.

Proposal 7：It is up to satellite (gNB on board) to make decision whether and when to activate NG Suspend/Resume Procedures.

Proposal 8: It is proposed to introduce the time and location-triggered NG pre-setup before the UE HO to the new satellite (gNB on board), to guarantee the UE service continuity.

4	Reference

[1] 	RP-240775, work plan for NTN

[2]       TS 38.300, NG-RAN stage2

[3]       TS 38.423, Xn application protocol (XnAP)

[4]       TS 38.413, NG application protocol (NGAP)

[5]       TR 38.821, Solutions for NR to support NTN