3GPP TSG-RAN WG3 Meeting #125	R3-244570

Maastricht , NL, 19th – 23th Aug 2024

Title: 	Discussion on Support of IoT NTN

Source: 	CMCC

Agenda item: 	15.2

Document Type:	Discussion and Decision

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 has been agreed in RP-240776:

The aim of this WI is enhancement of IoT-NTN with the following objectives:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

2	Discussion

Benefits specifically associated with the support of Store and Forward (S&F) Satellite operation with regenerative payload:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

Observation 1: Support of Store and Forward Satellite with regenerative payload is beneficial to provide cost-effective 3GPP IoT NTN solutions for delay-tolerant services in some scenarios.

In the solution of S&F satellite operation with full eNB as regenerative payload, when the satellite moves into the UE coverage area, the service link is available, eNB connects with UE, store the message; and when it moves to the area where feeder link is available, it can connect with ground CN, forward the message.

And if both eNB and MME/C-SGN on board, both the S1-AP interface between eNB and MME and the S1-U between eNB and C-SGN are on the satellite, there will be no S1 switchover over feeder link, so the enhancement is out of RAN3 scope.

In TR23.700-27 (Study on integration of satellite components in the 5G architecture), serval solutions which apply in different deployment scenarios are provided. Such as

- Solution #11: MME Split Architecture to support Control plane procedure for S&F

- Solution #12: S&F for multiple satellite deployment with anchor MME-ground

- Solution #13: S&F for single satellite deployments with eNB and MME-onboard

- Solution #14: Initial Registration with prior rejection in satellite network for S&F operation

- Solution #15: S&F solution for multi-satellite IoT NTN

- Solution #16: Enabling S&F operation with C-SGN onboard

- Solution #17 for KI#2: Support of Store and Forward Satellite operation for small data transmission

- Solution #18: Attach Procedure with MME and HSS on board the satellite

- Solution #19: Store and Forward Operation using a UE Proxy and Endpoint Proxy

- Solution #20: Solution on enabling S&F operation and data delivery with only eNB onboard satellite

- ……

In the above solutions, only solution#20 is explicitly requiring S1AP enhancement through feeder link.

Observation 2: Most S&F solution for different deployment scenarios suggest to have both eNB and MME/C-SGN on board, which needs no enhancement on S1AP.

Some other alternative solution for S&F satellite operation is that when the ISL exist, X2 switch over through ISL can forward the message stored, or S1AP over ISL can be considered.

Single Satellite and multiple Satellite deployment are discussed and compared. If only one satellite provides service for the UE, it may take a long time for the satellite complete one round. And since the orbit is an oval, after one round, it is most possible that the satellite can’t return to cover the same UE. It may need to take several rounds for the satellite to return and cover the same UE. So it is obvious that single satellite deployment takes too long to store the message. It is hard to finish the authentication between UE and CN in time and provide the service.

Multiple satellite solution is also discussed in SA2, but with MME/C-SGN on board. Up to now there is no discussion on whether S&F Satellite operation with only eNB on board and multiple satellite (with ISL or without ISL) deployment shall be supported.

In conclusion, S&F Satellite operation with only eNB on board is not a popular solution and deployment scenario.

Proposal 1: It is proposed that RAN3 shall first discuss and clarify the scenario which needs S1 enhancement to support S&F satellite operation with full eNB as regenerative payload.

Meanwhile, if some company really think S&F Satellite operation with regenerative payload shall be supported as described as the aim of this WI, Solution #20 described in TR23.700 may be the baseline for RAN3 discussion: Solution on enabling S&F operation and data delivery with only eNB onboard satellite has been proposed to resolve KI#2 Store and Forward operation with only eNB onboard satellite.

This solution assumes the following:

-	Only eNB is embedded and the eNB can buffers data for Store and Forward operation and no other Core Network function is embedded in the satellite.

-	UE and MME negotiate about the support of Store and Forward operation capability during attach procedure.

-	MME and eNB store in the UE context that the UE is operate in Store and Forward operation mode.

-	It is assumed that there is only one satellite serving the UE for Store and Forward Operation.

-	The satellite broadcasts that it supports Store and Forward operation in the SIB and satellite ID. The UE operates for S&F operation mode accesses the satellite based on this information.

-	During the feeder link is unavailable, it is assumed that the S1 interface is maintained between the satellite eNB and MME. (e.g. S1 suspend/resume)

-	The satellite eNB shall Store and Forward the received message between the UE and MME.

-	It is assumed that the UE communicate with the same serving satellite based on the satellite ID broadcasted from the serving satellite during the attach procedure.

-	The eNB and MME know that they support S&F operation each other based on the pre-configuration or in the S1 SETUP REQUET and S1 SETUP RESPONSE message, the eNB and the MME indicate to each other that they support S&F operation.

Editor's note:	It is FFS that how to manage S1 interface connection regarding S&F during the connection between eNB and MME is interrupted and will be determined by RAN WG3.

Impact to eNB includes:

-	New S1-AP message from MME for User Plane activation and indicates that buffering data in the satellite is needed for Store and Forward operation, and includes the UE location information (e.g. TA) for assistance information when paging.

-	New S1-AP message from MME indicates that the feeder link will be unavailable.

-	Support user plane activation with S-GW when the eNB is not connected with UE.

-	Support RRC Connection resume with UE when the eNB is connected with CN.

-	Paging for stored data/signalling when the eNB covers the area corresponding to the UE location information (e.g. TA) provided by MME.

-	Preconfigure when and which areas (e.g. supported TAI list and valid period) are covered by the satellites.

-	Support Store and Forward the NAS message between the UE and the MME for the UE operates in S&F mode.

-	During the feeder link is unavailable, support to maintain the S1 interface between the eNB and MME. (e.g. suspend/resume)

-	UE Context in eNB includes whether the UE is operating in Store and Forward operation mode

-	New IEs in SIB to broadcast that it supports Store and Forward operation.

-	Indicates to the MME that it supports S&F operation in the S1 SETUP REQUEST message.

So from our understanding, RAN3 should discuss whether these assumption and try to make agreement on it.

Proposal 2: If RAN3 initializes the discussion on S1 enhancement for S&F, take the solution#20 in TS23.700 as baseline to initialize the discussion in RAN3 for S&F. Agree the WA for the discussion:

- Only eNB is embedded and the eNB can buffers data for Store and Forward operation and no other Core Network function is embedded in the satellite.

- It is assumed that there is only one satellite serving the UE for Store and Forward Operation.

- During the feeder link is unavailable, it is assumed that the S1 interface is maintained between the satellite eNB and MME. (e.g. S1 suspend/resume)

- The eNB and MME know that they support S&F operation each other based on the pre-configuration or in the S1 SETUP REQUET and S1 SETUP RESPONSE message, the eNB and the MME indicate to each other that they support S&F operation.

Proposal 3: Based on the WA in proposal 2, RAN3 should prioritize the discussion to address the following issues:

- How to introduce message from MME to indicate that feeder link is unavailable.

- How to introduce Suspend/Resume procedures in S1 interface between the eNB and MME.

- How can let eNB and the MME indicate to each other that they support S&F operation.

3	Summary

In this contribution, we provide considerations on the IoT NTN in RAN3, following observations and proposals are made:

Observation 1: Support of Store and Forward Satellite with regenerative payload is beneficial to provide cost-effective 3GPP IoT NTN solutions for delay-tolerant services in some scenarios.

Observation 2: Most S&F solution for different deployment scenarios suggest to have both eNB and MME/C-SGN on board, which needs no enhancement on S1AP.

Proposal 1: It is proposed that RAN3 shall first discuss and clarify the scenario which needs S1 enhancement to support S&F satellite operation with full eNB as regenerative payload.

Proposal 2: If RAN3 initializes the discussion on S1 enhancement for S&F, take the solution#20 in TS23.700 as baseline to initialize the discussion in RAN3 for S&F. Agree the WA for the discussion:

- Only eNB is embedded and the eNB can buffers data for Store and Forward operation and no other Core Network function is embedded in the satellite.

- It is assumed that there is only one satellite serving the UE for Store and Forward Operation.

- During the feeder link is unavailable, it is assumed that the S1 interface is maintained between the satellite eNB and MME. (e.g. S1 suspend/resume)

- The eNB and MME know that they support S&F operation each other based on the pre-configuration or in the S1 SETUP REQUET and S1 SETUP RESPONSE message, the eNB and the MME indicate to each other that they support S&F operation.

Proposal 3: Based on the WA in proposal 2, RAN3 should prioritize the discussion to address the following issues:

- How to introduce message from MME to indicate that feeder link is unavailable.

- How to introduce Suspend/Resume procedures in S1 interface between the eNB and MME.

- How can let eNB and the MME indicate to each other that they support S&F operation.

4	Reference

[1] 	RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]       TR 23.700 -29, Study on integration of satellite components in the 5G architecture

[3]       TS 36.413, S1 application protocol (S1AP)