3GPP TSG-RAN WG3 Meeting #125														R3-244571

Maastricht, NL, Aug 19th - Aug 23th, 2024

Agenda item:		11.2

Source:				CMCC

Title:	Discussion on AI-based network slice

Document for:		Discussion

Introduction

A follow-up Release 19 new study item “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in RAN#102 [1].

During the last RAN3#124 meeting, the following agreements and FFS were captured:

In Rel-19, predicted information from CN to NG-RAN is not considered.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

FFS on SLA requirements fulfilment information exchange.

In this paper, we provide our understanding of AI/ML based Network Slicing.

Discussion

Finer granularity of UE performance feedback

During RAN3#124, RAN3 has reached consensus on enhancing the granularity of UE performance feedback to support network slicing, but no further conclusion on which finer granularity UE performance feedback needs to be supported. In [2], how to define UE performance feedback in R18 was discussed in RAN3#124, and the following conclusions were reached:

UE level performance is derived as the average value of per DRB level measurements of the UE.

And also, based on the previous discussion and contributions, the following granularities of UE performance feedback have been mentioned:

- QoS flow level

- DRB level

- PDU session level

- S-NSSAI level

- UE level

For network slicing use case, the UE level performance feedback is not enough for evaluating the handover UE services performance. For per slice level UE performance feedback, each slice of the UE may contain multiple PDU sessions, and each PDU session may represent one service. In order to evaluate the service performance after handover, slice level UE performance feedback seems not sufficient enough as the PDU session level UE performance cannot be derived from slice level UE performance. To directly obtain the UE performance feedback for each UE service after handover, PDU session level UE performance propose to be introduced.

Observation 1: The per slice level UE performance feedback cannot reflect UE service performance as each slice may contain multiple PDU sessions, and each PDU session may represent one service.

According to the conclusion of the last meeting, the UE level performance is derived as the average value of DRB level measurements, it seems feasible to introduce UE performance feedback with DRB level. However, after handover, the target node may have a different DRB configuration to map the QoS flows to DRB different from the source node. For example, the source DRB to QoS flow mapping is QoS flow 1/2/3 all mapping to DRB#1, after handover, the target DRB to QoS flow mapping is QoS flow 1/2 mapping to DRB#1 and QoS flow 3 mapping to DRB#2. Besides, if the handover UE service performance needs to be monitored, the PDU session level UE performance feedback can be considered. If finer granularity performance needs to be monitored, the QoS flow level UE performance feedback can be considered. As the mapping between DRB and QoS flows may different between the source node and target node, which is inconvenient to monitor, we propose to introduce PDU session level and/or QoS flow level UE performance feedback for network slicing use case.

Observation 2: The mapping between DRB and QoS flows may different between the source node and target node, which is inconvenient to monitor.

Proposal 1: Enhance finer granularity of UE performance feedback to support reporting per PDU session level or per QoS flow level.

UE traffic prediction/measurement

During RAN3#124, the following FFS for UE traffic were captured

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

First, for whether to include measured/predicted UE traffic per slice as input, according to 38.300, as in RAN slicing, the traffic can be used by the NG-RAN node to select the appropriate configuration for each network slice

To enable differentiated handling of traffic for network slices with different SLA: - NG-RAN is configured with a set of different configurations for different network slices by OAM; - To select the appropriate configuration for the traffic for each network slice, NG-RAN receives relevant information indicating which of the configurations applies for this specific network slice.

Observation 3: The traffic can be used by the NG-RAN node to select the appropriate configuration for each network slice.

When extended to AI/ML enabled network slice, we believe that UE traffic measurement/prediction is beneficial for the AI/ML model to optimize network slice resource allocation for both local node and neighbor node. For local node, dynamic network slice resource allocation can be performed by taking UE traffic measurement/prediction into consideration. For example, if the UE traffic is predicted to increase rapidly in the future, the local node can allocate more slice resources to avoid congestion to guarantee the QoS requirement. For neighbor node, by transmitting the UE traffic measurement/prediction to the neighbor node, the neighbor node can evaluate and determine whether the slice resources can meet the requirements for the handover UE. Therefore, we propose to include the UE traffic measurement/prediction as input information for AI/ML model inference and also exchange between nodes.

Observation 4: The UE traffic measurement/prediction is beneficial for local node to optimize slice resource allocation and for neighbor node to evaluate and determine whether the slice resources can meet the requirements for the handover UE.

Proposal 2: Include the UE traffic measurement/prediction as input information for AI/ML-based network slice.

Proposal 3: UE traffic measurement/prediction can be transmitted to the neighbor node during handover procedure.

Then, from the FFS of RAN3#124, the UE throughput is used to define UE traffic. However, in order to evaluate whether UE traffic rises rapidly after a specific point in time, rather than as a data burst that rises rapidly and then falls back rapidly, we think data volume can be considered as a representation of UE traffic.

Proposal 4: Data volume is preferred to represent UE traffic.

Signalling procedure

In last meeting, the following agreement has been captured.

The deployment below of AI/ML function is applied to AI/ML based network slicing:

- AI/ML Model Training is located in OAM, and AI/ML Model Inference is located in NG-RAN node (gNB-CU).

- AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node (gNB-CU).

For non-split architecture, the solutions and procedure for Model training in OAM and inference in NG-RAN is illustrated in figure 1.

Figure 1: Model training in OAM and inference in NG-RAN

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network slicing.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network slicing.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2 and UE, NG-RAN node 1 generates model inference output(s).

Step 10: NG-RAN node 1 executes AI-based network slicing actions according to the model inference output.

Step 11: NG-RAN node 1/2 provides feedback to OAM.

Model training and inference in NG-RAN node is illustrated in figure 2:

Figure 2: Model training and inference in NG-RAN

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 2 sends input data for Model Training to NG-RAN node 1.

Step 5: An AI/ML Model Training is located at NG-RAN node 1. The required measurements and input data from other NG-RAN nodes are leveraged to train the AI/ML model.

Step 6: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 7: NG-RAN node 2 sends the input information to NG-RAN node 1 for AI-based network slicing inference.

Step 8: NG-RAN node 1 performs model inference and generate network slicing resource allocation predictions or decisions.

Step 9: NG-RAN node 1 executes AI-based network slicing actions according to the model inference output.

Step 10: NG-RAN node 2 sends feedback information to NG-RAN node 1.

Proposal 5: Capture the above procedures in TR 38.743 for AI-based network slicing.

Based on the above discussion, we provide a TP for TR38.743 in R3-244581.

Proposal 6: Agree the TP in R3-244581 for TR 38.743.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: The per slice level UE performance feedback cannot reflect UE service performance as each slice may contain multiple PDU sessions, and each PDU session may represent one service.

Observation 2: The mapping between DRB and QoS flows may different between the source node and target node, which is inconvenient to monitor.

Proposal 1: Enhance finer granularity of UE performance feedback to support reporting per PDU session level or per QoS flow level.

Observation 3: The traffic can be used by the NG-RAN node to select the appropriate configuration for each network slice.

Observation 4: The UE traffic measurement/prediction is beneficial for local node to optimize slice resource allocation and for neighbor node to evaluate and determine whether the slice resources can meet the requirements for the handover UE.

Proposal 2: Include the UE traffic measurement/prediction as input information for AI/ML-based network slice.

Proposal 3: UE traffic measurement/prediction can be transmitted to the neighbor node during handover procedure.

Proposal 4: Data volume is preferred to represent UE traffic.

Proposal 5: Capture the above procedures in TR 38.743 for AI-based network slicing.

Proposal 6: Agree the TP in R3-244581 for TR 38.743.

- References

[1] RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

[2] R3-243578, “Discussion on definition of UE performance metrics”, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom