3GPP TSG-RAN WG3 Meeting #125														R3-244572

Maastricht, NL, Aug 19th - Aug 23th, 2024

Agenda item:		11.3

Source:				CMCC

Title:	Discussion on AI/ML-based CCO

Document for:		Discussion

Introduction

A follow-up Release 19 new study item “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in RAN#103 [1].

During the last RAN3#124 meeting, the following agreements and FFS were captured:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

Time information to be further discussed in next meeting.

In this paper, we provide our understanding of AI/ML based Coverage and Capacity Optimization.

Discussion

Future CCO state

During RAN3#124, RAN3 has agreed that inference in gNB-DU is not considered in R19, and how to derive future CCO state is up to implementation. And also, there is no consensus on whether the future CCO state and predicted CCO issue should be exchanged between nodes.

In the existing CCO mechanism, both CCO issue and CCO state should be exchanged via Xn interface. For the AI-based CCO mechanism, if only the predicted CCO issue is transmitted via Xn, the neighbour node will not know how the coverage configuration will change, and the neighbor node cannot make some preparations in advance. Therefore, we propose both future CCO state and predicted CCO issue should be exchanged between nodes.

Observation 1: Without exchanging future CCO state via Xn, the neighbor node will only aware of the upcoming CCO issue, but will have no idea about what coverage configuration the local node will change to.

Proposal 1: Both the future CCO state and predicted CCO issue shall be exchanged between NG-RAN nodes.

Besides, only the future CCO state exchanged between NG-RAN nodes is not enough, the neighbor nodes still not know when the future CCO state will be applied and cannot do some preparation in advance. Notifying neighboring node(s) the predicted coverage modification time can help them make some preparations in advance at an appropriate time and change their coverage state synchronously at the predicted coverage modification time point provided by the local node, which can also avoid delay impact between the local node first modifies its coverage state and then informs neighboring nodes its modification.

Observation 2: Provide the predicted coverage modification time to neighbor nodes can help neighbor nodes make some preparations in advance at an appropriate time.

Proposal 2: The future CCO state shall be exchanged between neighbor NG-RAN nodes together with the predicated coverage configuration modification time.

Then, considering that exchanging the CCO state over Xn is supported in the current spec, the NG-RAN node can inform its neighbor nodes the cell/beam coverage configuration before and after the cell coverage is modified by using the Coverage Modification List IE. The future CCO state exchanged between NG-RAN node is similar with the NG-RAN node inform its neighbor nodes the cell coverage state planned to be used at the next reconfiguration, the difference is that the future CCO state can be generate with AI/ML and it’s for the future use. Thus, two options for implementation:

Opt 1: Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and predicted CCO issue.

Observation 3: There are two options for delivering predicted CCO issue and future CCO state to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and predicted CCO issue.

We think both options can be discussed. But either way needs to distinguish legacy CCO state from future CCO state, to avoid neighbor node confusing about the receiving information. If opt 2 is selected, the new IE can be used to only transmit AI/ML related predicted/future information, which can be distinguished from the existing IE to avoid neighbor node confusing about the received information is legacy CCO state or future CCO state.

For Opt 1, if reusing existing CCO issue and CCO state IE, an indication needs to be introduced to distinguish whether the CCO issue and CCO state are predicted or for future use. The predicated coverage configuration modification time is an appropriate information for distinguish use. For example, when the CCO issue and CCO state is for legacy use, the predicated coverage configuration modification time is not included, when the CCO issue and CCO state is predicted or for future use, the predicated coverage configuration modification time is included.

Proposal 3: Regardless of which way to transmit predicted CCO issue and future CCO state to neighbor node, it is essential to distinguish the legacy CCO state/issue from predicted CCO issue and future CCO state.

Proposal 4: If opt 1 is agreed, the predicated coverage configuration modification time can be used to distinguish the legacy CCO state/issue from predicted CCO issue and future CCO state.

Feedback information

For the feedback information, as the UE(s) may locate in the gNB1 before coverage configuration changes, and locate in gNB2 after coverage configuration changes, the performance impact on such kind of UE(s) needs to be collected and evaluated when coverage modification performed. If the UE experience severe performance degradation, the serving gNB can perform actions to reduce the impact, e.g. provide more resources for the UE, and also this information can help the local node update the AI/ML model. In R18, the legacy UE performance feedback including average UE throughput UL/DL, average packet dalay and average packet loss. For the coverage edge UE(s), i.e. the UE(s) serving gNB is changed after CCO, the UE performance collected information can reuse the legacy UE performance feedback information.

Proposal 5: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. And for the feedback information, except the UE performance, we also propose to collect the NG-RAN node performance. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node this information and the local node can update the AI/ML model with this information.

Proposal 6: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

Others

Then, as one of the Coverage Modification Cause is the network energy saving, it’s proposed that consider the network energy saving with CCO, when the coverage modification cause is “network energy saving”, neighbor node should provide its measured energy consumption if it’s in the NES mode to avoid the total energy consumption increase after the coverage configuration modification.

Proposal 7: RAN3 discusses to consider the network energy saving with CCO and exchange the measured/predicted energy consumption between nodes to avoid the energy consumption increase after the coverage configuration modification.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: Without exchanging future CCO state via Xn, the neighbor node will only aware of the upcoming CCO issue, but will have no idea about what coverage configuration the local node will change to.

Proposal 1: Both the future CCO state and predicted CCO issue shall be exchanged between NG-RAN nodes.

Observation 2: Provide the predicted coverage modification time to neighbor nodes can help neighbor nodes make some preparations in advance at an appropriate time.

Proposal 2: The future CCO state shall be exchanged between neighbor NG-RAN nodes together with the predicated coverage configuration modification time.

Observation 3: There are two options for delivering predicted CCO issue and future CCO state to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and predicted CCO issue.

Proposal 3: Regardless of which way to transmit predicted CCO issue and future CCO state to neighbor node, it is essential to distinguish the legacy CCO state/issue from predicted CCO issue and future CCO state.

Proposal 4: If opt 1 is agreed, the predicated coverage configuration modification time can be used to distinguish the legacy CCO state/issue from predicted CCO issue and future CCO state.

Proposal 5: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Proposal 6: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

Proposal 7: RAN3 discusses to consider the network energy saving with CCO and exchange the measured/predicted energy consumption between nodes to avoid the energy consumption increase after the coverage configuration modification.

- References

[1] RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.743

- 4.2	AI/ML based Coverage and Capacity Optimization

- 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

- 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

- 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

- 4.2.2.2		Input data of AI/ML based CCO

For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO State

From neighbouring gNBs:

-	Measured/Predicted radio resource statusFrom the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.3	Output data of AI/ML based CCO:

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue

-	Future CCO state

-	Predicated coverage configuration modification time

#### 4.2.2.4	Feedback of AI/ML based CCO:

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs:

- Measured radio resource status

- Legacy UE performance feedback for those UEs handed over from the source gNB

- SON Reports (e.g., RLF, CEF, RA)

- UE performance feedback for coverage edge UE(s)

- NG-RAN node(s) performance of neighbor node(s) (e.g. the number of RRC connections, PRB usage)

#### 4.2.2.5	Potential standard impacts:

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Future CCO State

F1 interface:

- Predicted CCO Issue (including predicted cells/beams)

- 4.2.2.6 Model training in OAM and inference in NG-RAN for Non-split architecture

The signaling flow for Model training in OAM and inference in NG-RAN is shown in figure X-1.

Figure X-1: Model training in OAM and inference in NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information.

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for CCO.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based CCO.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2 and UE, NG-RAN node 1 generates model inference output(s).

Step 10: NG-RAN node 1 executes AI-based CCO actions according to the model inference output.

Step 11: NG-RAN node 1/2 provides feedback to OAM.

- 4.4.2.7 Model training and inference in NG-RAN for Non-split architecture

The signaling flow for Model training and inference in NG-RAN is shown in figure X-2.

Figure X-2: Model training and inference in NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information.

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 2 also sends input data for Model Training to NG-RAN node 1.

Step 5: An AI/ML Model Training is located at NG-RAN node 1. The required measurements and input data from other NG-RAN nodes are leveraged to train the AI/ML model.

Step 6: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 7: NG-RAN node 2 sends the input information to NG-RAN node 1 for AI-based CCO inference.

Step 8: NG-RAN node 1 performs model inference and generate CCO predictions or decisions.

Step 9: NG-RAN node 1 executes AI-based CCO actions according to the model inference output.

Step 10: NG-RAN node 2 sends feedback information to NG-RAN node 1.