3GPP TSG-RAN WG3 Meeting #125     	          	                                           R3-244573

Maastricht, NL, August 19th-23rd, 2024

Agenda item:	10.3.2

Source:	CMCC

Title:	Discussion on SON/MDT for network slicing

Document for:	Discussion

1	Introduction

In previous meeting [1,2], RAN3 discussed on SON/MDT for network slicing and had the following progress.

RAN3#123b Both the logged MDT and immediate MDT enhancements based on scenarios to be worked. Work on the Slice aware MRO scenarios. Work on logged MDT and immediate MDT enhancements based on scenarios RAN3#124 RAN3 to support immediate MDT measurements collection on specific area per slice(s). FFS on which immediate MDT measurements can be collected per slice. FFS on whether to support logged MDT on specific area per slice(s). RAN3 to work on logged MDT enhancement for slice-based cell reselection. For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

This contribution will discuss the potential SON/MDT enhancements for network slicing.

2	Discussion

## Issue 1: Slice aware MRO scenarios

Figure 1. The hospital cells are lack of coverage, while the public cells are over coverage

Figure 1 shows an example of the issue that we are facing. Some slices are deployed at a specific area with strict geographical boundaries, e.g., hospital. Such kind of slices are expected to provide continuous services within the specific area. And the public cells outside don’t support the hospital slice.

However, it is observed that, due to imperfect antenna angle planning, the slice may become unavailable in some spots inside the area, especially near the boundaries. As shown in Figure 1, as UE moving close to the boundary, the connected mode UE may handover from a hospital cell to a public cell. Although the radio condition is still good and no RLF is declared, the ongoing PDU session for the hospital slice is rejected by the public cell during the handover procedure.

If such issue can be collected by MDT and reported to OAM, it will help the operator to figure out the problematic cells and adjust the corresponding antenna angle in order to guarantee the hospital slice available inside the hospital area.

However, such kind of slice unavailability within the hospital area is difficult to be visible by OAM. Based on Rel-16 MDT, one way is to configure immediate MDT whenever PDU session is rejected during handover and to let UE report location immediately. But such kind of configuration is a bit of overkill, since normally, when UE is leaving the hospital, the slice should be released or rejected just as a normal case. Such configuration causes too much unnecessary measurement reporting, and it is difficult to distinguish the error cases from large numbers of normal cases.

Observation 1: It is difficult for the current SON/MDT procedure to identify the slice unavailability issue during HO is an error case due to less of coverage or a normal case due to leaving the specific area.

In legacy logged MDT procedure, the network can receive the logged info (e.g. frequency info, cell info and location info) from idle UEs. One possible argument may be that the network can identify the potential issue on coverage from the logged info received from idle UEs. Firstly, we think the cell measurement results in idle mode and connected mode are different, since there may be no L3 filtering for idle UE to derive the cell quality. On the other hand, although it seems the lack of coverage based on the logged info from idle UEs, there may not actual handover occurring for connected mode UE. In addition, the reporting and analysis of massive data from idle UEs will also enormously increase the UE power consumption and the workload of network, but the issue cannot be identified accurately and addressed.

Observation 2:  It is difficult to identify the slice unavailability issue during HO based on current logged info from idle UEs.

Figure 2. WLAN AP deployment in the hospital

It is observed that WLAN AP is always deployed inside the hospital, as shown in Figure 2. Since the WLAN name list (SSID) configuration is supported in Rel-16 MDT, the WLAN SSID can be utilized for UE to figure out whether the slice rejection happened inside the hospital area or not. For example, UE can figure out whether UE is still inside the geographical area by detecting the WLAN AP which covers the same area as hospital slice.

Therefore, one candidate solution to address the issue is to let UE report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage. This solution means to introduce a new trigger event to report the slice unavailability after HO.

Proposal 1: Support to report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage.

## Issue 2: Slice request rejection due to slice unavailable in current registration area

During the registration procedure, UE reports the requested NSSAI list to AMF and then receives an allowed NSSAI list and a rejected NSSAI list from AMF. The allowed list and rejected list are a sub-set of the NSSAIs in the requested list. In current specification, only the slices within the allowed NSSAI list are allowed to be initiated by the UE in the registration area.

Some slices are triggered by higher layer Application. Even if the specific slice is not allowed in the registration area, i.e., the specific slice is not in the allowed NSSAI list, the user may still try to launch the APP for this slice. Such request of APP layer will be rejected by UE NAS layer after the NAS layer checking the corresponding NSSAI of the APP with the allowed NSSAI list. Such kind of rejection is done inside the UE NAS layer. So, the network side has no idea how often such failure happens.

Figure 3. Example for reporting the failure of slice initiation inside the UE

However, we find such rejection information valuable for network side to assess if there is a strong demand of a specific slice. If the operator observes a great number of UEs trying to launch a slice which is not supported in this area, the operator will deploy this slice in the near future to meet the customers’ requirements.

Observation 3: The failure of a slice initiation inside the UE is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Proposal 2: Support to report the failure of slice initiation inside the UE, when the higher layer triggers a slice but is rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

## Issue 3: Slice-based cell reselection

In last meeting, RAN3 agreed to work on logged MDT enhancement for slice-based cell reselection. For slice-based cell re-selection, the UE will consider the NSAG priority received by NAS and the slice information received in SIB16 or RRCRelease, e.g. nsag-CellReselectionPriority, sliceAllowedCellListNR, sliceExcludedCellListNR.

As specified in TS 38.304 [3], if the cell is either listed in the sliceAllowedCellListNR or is not listed in the sliceExcludedCellListNR, or neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured, the UE will consider the cell to support the NSAG.

The UE considers a cell on an NR frequency to support all slices of an NSAG if - the nsag-ID and TA of the NSAG indicated for the NR frequency in dedicated signalling but not in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; or - the nsag-ID and TA of the NSAG indicated for the NR frequency in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; and - the cell is either listed in the sliceAllowedCellListNR (if provided in the sliceInfoList) or the cell is not listed in the sliceExcludedCellListNR (if provided in the sliceInfoList); or - Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the sliceInfoList.

On the other hand, it is also specified in TS 38.304 [3] that if the highest ranked cell or best cell does not support the NSAG, the UE will re-derive the re-selection priority based the NSAG supported by this cell or legacy cell priority. Then the UE continues to evaluate based on the re-derived re-selection priority, and will select a cell which supports the highest priority NSAG within UE.

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause: - if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency); - Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

If only sliceExcludedCellListNR is configured and the highest ranked cell or best cell is not within the sliceExcludedCellListNR, the UE will consider the cell to support the NSAG. However, after the UE camps on the cell, it may find the cell does not support the NSAG via SIB16, especially when the UE is at the boundary of different TAs. If the UE can report the NSAG supported by the highest ranked cell or best cell, it can be helpful for NW to adjust the sliceAllowedCellListNR, sliceExcludedCellListNR. Then the UE will find a more suitable cell for slice-based cell re-selection.

Proposal 3: The NSAG supported by the highest ranked cell or best cell can be reported to the network, which can be helpful for NW to adjust the sliceAllowedCellListNR and sliceExcludedCellListNR, and can help the UE to select a more suitable cell for slice-based cell re-selection.

3	Conclusion

Here are the observations and proposals for SON/MDT enhancements for network slicing.

Issue 1: Slice aware MRO secnarios

Observation 1: It is difficult for the current SON/MDT procedure to identify the slice unavailability issue during HO is an error case due to less of coverage or a normal case due to leaving the specific area.

Observation 2:  It is difficult to identify the slice unavailability issue during HO based on current logged info from idle UEs.

Proposal 1: Support to report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage.

Issue 2: Slice request rejection due to slice unavailable in current registration area

Observation 3: The failure of a slice initiation inside the UE is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Proposal 2: Support to report the failure of slice initiation inside the UE, when the higher layer triggers a slice but is rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

Issue 3: Slice-based cell re-selection

Proposal 3: The NSAG supported by the highest ranked cell or best cell can be reported to the network, which can be helpful for NW to adjust the sliceAllowedCellListNR and sliceExcludedCellListNR, and can help the UE to select a more suitable cell for slice-based cell re-selection.

4	Reference

RAN3#123-bis_Meeting_Report

RAN3#124_Meeting_Report

TS 38.304 NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state